3GPP TSG-SA5 Meeting #156	S5-244321

Maastricht, Netherlands, 19 - 23 August 2024

# Source:		Nokia

# Title:	Rel-19 pCR TR 28.880 Add use case for Energy consumption of cloud-native NFs

# Document for:	Approval

Agenda Item:	6.19.20

# 1	Decision/action requested

The group is requested to discuss and approve the pCR below.

# 2	References

[1]          3GPP TR 28.880: "Study on energy efficiency and energy saving aspects of 5G networks and services".

[2]          3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[3]          3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[4]          3GPP TR 28.869: "Study on Cloud Aspects of Management and Orchestration".

[5]          SP-231723: "Study on energy efficiency and energy saving aspects of 5G networks and services".

[6]          SP-231781: "Study on Cloud Aspects of Management and Orchestration".

# 3	Rationale

Currently Power consumption measurements are defined for PNF as in clause 5.1.1.19.2 of TS 28.552 [2], and the energy consumption of virtualized NF is defined in clause 6.7.3.1.2 of TS 28.554[3].

Clause 5.1 of TR 28.880 [1] studies the estimation of energy consumption of containerized VNF/VNFC using NFV-MANO.

Rel-19 study TR 28.869 [4], includes use of ETSI VNF generic OAM functions [2], use of industry solutions including ETSI NFV MANO, open source and other solutions for management (including life cycle management and other management aspects) of cloud native network functions (see clause 1 of TR 28.869 [4]).

The energy consumption of containerized NF/cloud-native NFs without using ETSI-NFV-MANO also needs to be considered. This is currently not part of any use case in TR 28.880 [1].

NOTE: The terminology containerized NF/cloud-native NFs is under discussion in Rel-19 SI FS_Cloud_OAM, and needs to be aligned with the outcome that would be documented in TR 28.869 [4].

This contribution proposes to add a new use case study the energy consumption measurement and/or KPIs for cloud-native NFs without using ETSI-NFV-MANO.

This use case is related to the following objectives as described in FS_Energy_OAM_Ph3 (see SP-231723 [5]).

WT-1.1 Study new or enhanced Energy Consumption (EC) and Energy Efficiency (EE) KPIs and measurements, e.g. for new types of network slices, RAN sharing, etc.

WT-1.2 Consideration of containerized VNFs, study enhancements to virtualized NF EC KPIs, including containerized VNF/VNFCs, and possible impacts to metrics collected from NFV MANO so as to be able to estimate their energy consumption based on e.g. their virtual CPU usage, virtual memory usage, etc.

# 4	Detailed proposal

The following changes are proposed for TR 28.880[1].

1st Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[3]	ETSI GR NFV-IFA 029 V3.3.1 (2019-11): "Network Functions Virtualisation (NFV) Release 3; Architecture; Report on the Enhancements of the NFV architecture towards "Cloud-native" and "PaaS"".

[4]	ETSI GS NFV-IFA 040 V4.3.1 (2022-05): "Network Functions Virtualisation (NFV) Release 4; Management and Orchestration; Requirements for service interfaces and object model for OS container management and orchestration specification".

[5]	ETSI GS NFV-IFA 027 V4.3.1 (2022-06): "Network Functions Virtualisation (NFV) Release 4; Management and Orchestration; Performance Measurements Specification".

[6]	ETSI GS NFV-IFA 027 V5.1.1 (2024-4): "Network Functions Virtualisation (NFV) Release 5; Management and Orchestration; Performance Measurements Specification".

[7]		3GPP TS 22.261: Service requirements for the 5G system; Stage 1

[8]	ETSI GS OEU 020: "Operational energy Efficiency for Users (OEU); Carbon equivalent Intensity measurement; Operational infrastructures; Global KPIs; Global KPIs for ICT Sites".

[9]	ETSI EN 303 472: "Environmental Engineering (EE); Energy Efficiency measurement methodology and metrics for RAN equipment".

[10]	ISO/IEC 30134-3:2016: Information technology - Data centres - Key performance indicators - Part 3: Renewable energy factor (REF)

[11]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[12]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G"

[13]	3GPP TS 28.313: " Management and orchestration; Self-Organizing Networks (SON) for 5G networks ".

[14]                      3GPP TS 28.622: "Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[X]                      3GPP TR 28.869: "Study on Cloud Aspects of Management and Orchestration".

Editor’s note: reference [6] is not published yet, will be published soon, the latest draft of DGS/NFV-IFA027 is available in the following location: https://docbox.etsi.org/ISG/NFV/Open/Drafts/IFA027ed451

Next Change

## 5.A	Use case #A:  Energy consumption of cloud-native NFs

### 5.A.1	Description

The existing definition of the Energy Consumption (EC) of VNF/VNFCs (see TS 28.554 [2] – clauses 6.7.3.1.2 and 6.7.3.1.3) is valid for VM-based VNFs, i.e. when VNF/VNFC(s) are implemented on Virtual Machine(s) (VM). Use case #1 (described in clause 5.1) studies the estimation of energy consumption of containerized VNF/VNFC using NFV-MANO.

This use case addresses the energy consumption KPI for cloud-native network functions, where industry solutions for management of cloud-native network functions is used (see clause 5.2 of TR 28.869 [X]).

Editor’s note: The terminology related to cloud-native network function is under discussion in Rel-19 SI FS_Cloud_OAM. The terms in this use case needs to be aligned with the outcome documented in TR 28.869 [4].

### 5.A.2	Potential requirements

REQ-Energy_Consumption_Containerized-CON-1: The 3GPP management system should be able to measure or estimate the energy consumption of 5G NFs realised as cloud-native network functions.

### 5.A.3	Potential solutions

#### 5.A.3.i	Potential solution #<i>: <Potential Solution i Title>

##### 5.A.3.i.1	Introduction

Editor's Note:	This clause describes briefly the potential solution at a high-level.

##### 5.A.3.i.2	Description

Editor's Note:	This clause further details the potential solution and any assumptions made.

### 5.A.4	Evaluation of potential solutions

Editor's Note:	This clause provides the evaluation of potential solutions.

End of change