3GPP TSG-SA5 Meeting #156	S5-244408

Maastricht, Netherlands, 19 - 23 August 2024

# Source:	Nokia

# Title:	Versioning of management interfaces

# Document for:	Approval

Agenda Item:	6.19.8

# 1	Decision/action requested

The group is asked to discuss and approve the pCR below.

# 2	References

[1]	3GPP TR 28.871 Study on Service Based Management Architecture (SBMA) enhancement phase 3

[2]	3GPP TS 29.501: 5G System; Principles and Guidelines for Services Definition

[3]			Semantic Versioning Specification: https://semver.org.

# 3	Rationale

Interoperability of MnS consumers and MnS producers depends on the versions of both. This is relevant for the operations during usage of a specific interface as well as during the phase of discovery of management services since the MnS consumer needs to find interoperable MnS producers. Versioning shall be covered by [1].

Currently none of the versions is explicitly mentioned in stage 2. Solution set specific artifacts of OpenAPI are versioned according to the latest 3GPP release of the originating technical specification. This has the drawback that the version of stage 3 artifacts changes as soon as a new release of the originating TS is available -even in case the technical contant has not changed. In contrast CT is using a versioning scheme of stage 3 artifacts that is independent from the release cycle of 3GPP [2] and is based on [3].

Further, for a working interface not only the artifacts of 3GPP need to be compatible in producer and consumer, but the vendor-specific extensions have to be compatible too. Current specifications of SA5 do not consider any versioning of vendor-specific artifacts like VsDataContainers.

# 4	Detailed proposal

First change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[3]	3GPP TS 28.537: "Management and orchestration; Management capabilities".

[4]	3GPP TS 28.623: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions".[5]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3"

[6]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics (MDA) "

[7]	3GPP TS 28.105: "Management and orchestration; Artificial Intelligence/ Machine Learning (AI/ML) management"

[8]	3GPP TS 28.111: "Fault management"

[9]	3GPP TS 28.311: "Management and orchestration; Network policy management for mobile networks based on Network Function Virtualization (NFV) scenarios"

[10]	3GPP TS 28.312: "Management and orchestration; Intent driven management services for mobile networks"

[11]	3GPP TS 28.317: "Management and orchestration; Self-configuration of Rad io Access Network Entities (RAN NEs)"

[12]	3GPP TS 28.318: "Management and Orchestration; Network and services operations for energy utilities"

[13]	3GPP TS 28.319: "Management and orchestration; Access Control for Management services"

[14]	3GPP TS 28.532: "Management and orchestration; Generic management services"

[15]	3GPP TS 28.536: "Management and orchestration; Management services for communication service assurance; Stage 2 and stage 3"

[16]	3GPP TS 28.538: "Management and orchestration; Edge Computing Management"

[17]	3GPP TS 28.556: "Management and orchestration; Network policy management for 5G mobile networks; Stage 2 and stage 3"

[18]	3GPP 28.404 "Telecommunication management; Quality of Experience (QoE) measurement collection; Concepts, use cases and requirements)"

[19]	3GPP 28.405 "Telecommunication management; Quality of Experience (QoE) measurement collection; Control and configuration)"

[20]	3GPP 28.406 "Telecommunication management; Quality of Experience (QoE) measurement collection; Information definition and transport"

[21]	3GPP 28.520 "Telecommunication management; Performance Management (PM) for mobile networks that include virtualized network functions; Requirements"

[22]	3GPP 28.521 "Telecommunication management; Performance Management (PM) for mobile networks that include virtualized network functions; Procedures

[23]	3GPP 28.522 Telecommunication management; Performance Management (PM) for mobile networks that include virtualized network functions; Stage 2

[24]	3GPP 28.523 Telecommunication management; Performance Management (PM) for mobile networks that include virtualized network functions; Stage 3"

[25]	3GPP 28.550 "Management and orchestration; Performance assurance"

[26]	3GPP 28.552 "Management and orchestration; 5G performance measurements"

[27]	3GPP 28.554 "Management and orchestration; 5G end to end Key Performance Indicators (KPI)"

[28]	3GPP 28.558 "Management and orchestration; UE level measurements for 5G system"

[29]	3GPP 32.401 "Telecommunication management; Performance Management (PM);Concept and requirements"

[30]	3GPP 32.404 "Telecommunication management; Performance Management (PM); Performance measurements; Definitions and template"

[31]	3GPP 32.421 "Telecommunication management; Subscriber and equipment trace; Trace concepts and requirements"

[32]	3GPP 32.422 "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management"

[33]	3GPP 32.423 "Telecommunication management; Subscriber and equipment trace; Trace data definition and management"

[34]	3GPP TS 29.501: 5G System; Principles and Guidelines for Services Definition

[35]	Semantic Versioning Specification: https://semver.org.

[36]	3GPP 28.533 “Management and orchestration; Architecture framework”

Second change

## 5.x	Versioning of MnS

### 5.x.1	Description

Communication between MnS consumer and MnS producer depends on a common understanding of operations (component type A), managed entities (component type B), and management data (component type C).

Additionally, in many real-life systems the management interfaces need to carry parameters that have been standardized by other groups of 3GPP but are not covered by the NRM as of SA5. Further, in many implementations the management interfaces need to carry parameters and management data that relate to entirely vendor-specific behavior of managed entities. For both, 3GPP SA5 offers to model additional objects and parameters as vendor-specific data containers (VsDataContainer) or to add vendor-specific measurements.

Modifications of these artifacts are introduced on one hand due to the evolution of the 3GPP specifications by changed management operations, modified standardized network resource model, and modified measurements and alarms that 3GPP SA5 introduces with each new release. On the other hand, the additional implementation-specific objects, attributes, and measurements also change due to new functionality of network functions and management systems.

Note: For the time being SA5 does not consider to describe or to standardize vendor-specific operations.

In general, the release cycles of 3GPP are independent from the cycles of the implementation-specific artifacts. Due to technical and economical constraints, not necessarily all interfaces of an existing management system are changed at the same time. As consequence, a real life system might face a multitude of different combinations of releases of 3GPP and of implementation-specific versions. E.g. as depicted in Fig. 5.x.1-1, the overal system might be composed by interfaces of the 3GPP Releses 16, 17, and 18 and implementation-specific versions A-F.

Figure 5.x.1-1 A complex deployment of management systems management interfaces according to various releases of 3GPP (e.g. Rel-16, 17, and 18) might combine managed systems of different implementation-specific versions (e.g. Version A – F).

As a matter of fact, one interface (between NMS-1 and NMS-2 in the example of Fig 5.x.1-1) might be of Rel-18, while it has to carry information elements of network functions with interfaces of Rel-16, 17, and 18. In addion, it has to cope with several vendor-specific extensions of the network resource model. In the reply to operation getMOIAttributes the NMS-1 has to combine objects of different releases and versions into one payload, where the detailed format is specific to the solution set.

#### 5.x.1.1	Observations

For a given interface, the release of the operations (component type A) is independent from the release of the managed entities (NRM, component type B) and independent from the management data (component type C).

For a given interface, the release of the standardized managed entities (NRM, component type B) is independent from the vendor-specific extension of the NRM. Similarly, for a given interface the set of standardized management data is independent from the vendor-specific set of management data.

3GPP introduces new releases while content of technical specification remains unchanged.

The different artifacts of a given interface show a hierarchy of versions:

- The operations of the interface are defining the base version for the interface.

- Individual object instances carried across one interface might be of different 3GPP releases and different individual implementation-specific versions than the operations of the interface.

- If not specifically indicated, the object instances of component type B are of the same 3GPP version and release as the operations of the interface.

- If not specifically indicated, the management data of component type C are of the same 3GPP version and release as the operations of the interface.

- If not specifically indicated, name-contained object instances are of the same release and vendor-specific version as the containing object.

- Individual management data carried across one interface might be of different 3GPP release and individual implementation-specific versions than the operations of the interface.

.

### 5.x.2	Potential requirements

REQ-MNS-versioning-1: Artifacts of the management interfaces, including stage 2,  shall contain information elements to identify:

- Release number of the originating 3GPP TS that describes the interface.

- Version number of the artifact according to 3GPP, independent from the release number of the originating TS in order to avoid version bumps without technical implications.

REQ-MNS-versioning-2: The operations of an interface (component type A) shall indicate their originating 3GPP TS and a 3GPP version number which is independent from the release of the originating specification.

REQ-MNS-versioning-3: Individual instances of IOC from component type B (details depend on the solution set) require optional information elements to indicate the originating 3GPP release, the 3GPP version number, and an implementation-specific version. If an individual oject instance does not show a dedicated version information, it inherits this missing version information either from the operations (component type A) or from the containing object instance of component type B.

REQ-MNS-versioning-4: Individual management data (component type C) need to offer optional information elements to indicate the 3GPP release, the 3GPP version number, and an implementation-specific version. If managementan data does not indicate a specific version information, it inherits this version information either from the operations (component A) or from the related object instance (component type B).

Editors note: According to TS 28.532 clause 11.3.2.1.2 “Performance data file content description” PM files already contain the fields:

- vendorName = “Vendor of the the entity, that generated and sent the file” and

- measObjRootSwVersion = “Software version of the measured object root, allowing post-processing systems to take care of vendor specific performance metrics. It is either the software version of a managed element or of a management node”.

REQ-MNS-versioning-5: MnsRegistry and MnsInfo shall be able to handle the version info composed by originating 3GPP release, 3GPP version number, and implementation-specific versions according to REQ-MNS-versioning-2 - 4.

### 5.x.3 Potential solutions

#### TBD.

### 5.x.4	Evaluation of potential solutions

#### TBD.

End of change