3GPP TSG-SA5 Meeting #156 	S5-244423

Maastricht, The Netherlands, 19 - 23 August 2024

# Source:	Ericsson España S.A.

# Title:	pCR TR 28.879 Registration use case – add two new solutions and their evaluations

# Document for:	Approval

Agenda Item:	6.19.21

# 1	Decision/action requested

The group is asked to discuss and approve the proposal.

# 2	References

[1] 3GPP TR 28.879: " Study on OAM for service management and exposure to external consumers".

[2] S5-243250: “pCR TR 28.879 Registration of MnS producer into CAPIF”

# 3	Rationale

Clause 5.1.1 in TR 28.879 [1] focuses on registration use case, which describes the ability of the MnS producer to register into the CAPIF. Once registered, the MnS producer is able to communicate with the CCF for the subsequent procedures needed to configure accessibility to MnS(s) from external MnS consumers. These procedures include publication, discovery and access control, among others.

Status: At SA5#155, a first draft of publishing use case was agreed in S5-243250 [2]. This first draft contains:

- Description (clause 5.1.2.1)

- Potential requirements (clause 5.1.2.2)

- Potential solutions (clause 5.1.2.3). Specifically, one potential solution was agreed: solution #1. This solution proposes how to capture MnS producer information (SA5 definition) into APIProviderEnrolmentDetails data type (SA6 definition).

Gaps: When MnS producer is registered into the CCF, it is needed to specify which API provider domain function(s) the MnS producer will play. However, in the latest version of TR 28.879 [1]:

- There is no requirement that captures this need

- No potential solutions have been proposed to fulfil this requirement.

# 4	Detailed proposal

The present pCR aims to bridge this gap, by making the following changes in the latest version of TR 28.879 [1]

- Adding a new requirement in clause 5.1.2.2.

- Adding two new potential solutions in clause 5.1.2.3, that allow fulfilling this requirement

- Evaluate the two potential solutions, and report evaluation in clause 5.1.2.4.

Begin Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]            	3GPP TS 28.533: “Management and orchestration; Architecture Framework”.

[3]	3GPP TS 28.622: “Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)”

[4]           	3GPP TS 28.537: “Management and orchestration; Management capabilities”.

[5]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2"

[6]	SP-231669: "LS on collaboration and alignment of 3GPP defined application enablers with GSMA Open Gateway".

[7]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[8]	3GPP TS 23.255: "Application layer support for Vehicle-to-Everything (V2X) services; Functional architecture and information flows".

[9]	3GPP TS 23.286: "Application layer support for Uncrewed Aerial Systems (UAS) services; Functional architecture and information flows".

[10]	3GPP TS 23.545: "Application layer support for Factories of the Future (FF) ".

[11]	3GPP TS 23.542: "Application layer support for Personal IoT Networks".

[12] 	3GPP TS 23.554: "Application architecture for MSGin5G Service; Stage 2".

[13]	3GPP TS 29.222: "Common API Framework for 3GPP Northbound APIs; stage 3".

[14]	3GPP TS 33.122: "Security aspects of Common API Framework (CAPIF) for 3GPP Northbound APIs".

[15]	 "The Ecosystem for Open Gateway NaaS API Development", white paper, June 2023 [link]

[16]	"GSMA Operator Platform Group – Requirements and Architecture", version 5.0, July 2023 [link]

[17]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[18]	3GPP TS 28.531: "Management and orchestration; Provisioning"

[19]	3GPP TS 23.435: "Procedures for Network Slice Capability Exposure for Application Layer Enablement Service"

[x]	3GPP TR 28.824: “Study on network slice management capability exposure”

Second Change

## 5.1	Exposure of management services

### 5.1.1	Use case #1:  MnS producer registration into CAPIF

#### 5.1.1.1	Description

For a MnS to be made available for consumption through an exposure framework, it is needed that this MnS producer registers into that exposure framework. Once registered, the MnS producer is able to communicate with the CCF for the subsequent procedures needed to configure accessibility to MnS from external MnS consumers. These procedures include publication, discovery and access control, among others.

In the CAPIF, the scope of registration is limited to API provider domain functions, which include API Exposing Function (AEF), API Publishing Function (APF) and API Management Function (AMF). The registration is a procedure whereby these functions become recognized users of the CCF. This procedure is described in clause 8.28 of TS 23.222 [5], with stage 3 solution set detailed in clause 8.9 of TS 29.222 [13].

To register (and/or update the registration information of) of an API provider domain functions on the CCF, the AMF from this API provider domain communicates with the CCF over CAPIF-5 interface. This means that:

-	This API provider domain shall have an AMF.

-	The AMF of this API provider domain shall be an authorized user of CCF.

-	The AMF security information for CCF to validate the registration request is provisioned by the CAPIF administrator.

To register a MnS producer on the CCF, it is needed for the MnS producer to support some API provider domain functionality, as detailed below:

-	AEF. If supported, this means that the MnS producer will need to support:

-	CAPIF-2/2e interface, so that the API invokers acting as external MnS consumers can access service APIs, when required. The API operations that will be implemented in this interface (see TS 29.222 [13], clause 9.1) can be configured later, depending on the intended usage of MnS producer.

NOTE: The service APIs can convey different MnS information for different API invokers. This issue will be discussed as a separate use case.

-	CAPIF-3 interface, so that the MnS producer can communicate with CCF to exercise access and policy related control for service API invocations initiated by the API invoker. The API operations that will be implemented in this interface (see TS 29.222 [13], clauses 8.3, 8.5, 8.7, 8.6) can be configured later, depending on the intended usage of MnS producer.

-	APF. If supported, this means that the MnS producer will need to support CAPIF-4 interface, so that it can communicate with CCF to publish (and manage the published) MnS information. The API operations that will be implemented in this interface (see TS 29.222 [13], clause 8.2) can be configured later, depending on the intended usage of MnS producer.

-	AMF. If supported, this means that MnS producer will need to support CAPIF-5 interface. The API operations that will be implemented in this interface (see TS 29.222 [13], clause 8.3 and 8.9) can be configured later, depending on the intended usage of MnS producer.

The MnS producer shall always support at least the AEF; otherwise, the MnS producer is not reachable by the CCF nor API invokers, and thus not eligible for integration into the CAPIF.

The AMF communicates the MnS producer details (including supported API provider domain functions) to the CCF using CAPIF_API_Provider_Management_API (see TS 29.222 [13], clause 8.9). Upon reception of these details, the CCF registers the MnS producer with the supported API provider domain function(s).

#### 5.1.1.2	Potential requirements

PREQ-FS_MExpo-Reg-01: The 3GPP management system shall provide the capability to register a MnS producer on the CCF.

PREQ-FS_MExpo-Reg-02: The 3GPP management system shall provide the capability to deregister a MnS producer on the CCF.

PREQ-FS_MExpo-Reg-03: The 3GPP management system shall provide the capability to update the registration details of a MnS producer on the CCF.

PREQ-FS_MExpo-Reg-0x: The MnS producer shall be able to support one or more CAPIF API provider domain functions, with at least one of them being AEF.

Third Change

#### 5.1.1.3	Potential solutions

##### 5.1.1.3.x	Potential solution #x: MnS producer registered with AEF, APF and AMF functionality.

###### 5.1.1.3.x.1	Introduction

When registering a MnS producer on the CCF, it is needed to declare which API provider domain functions the MnS producer will play. This information is stored into the “APIProviderEnrolmentDetails/apiProvFuncs”.

This solution focuses on meeting the requirement PREQ-FS_MExpo-Reg-0x. It proposes MnS producer is configured to support all API provider domain functions: AEF, APF and AMF. This solution corresponds to the alternative 2 described in TR 28.824 [x], which was recommended as baseline for the normative work.

Figure 5.1.1-x: MnS producer configured with AEF, APF and AMF functionality.

###### 5.1.1.3.x.2	Description

The table below summarizes what interfaces the MnS producer need to support according to this solution.

Table 5.1.1.3.x.2-1 MnS producer supported interfaces

| CAPIF interface | Support | Description |
| --- | --- | --- |
| CAPIF-2/2e | M | The MnS producer shall use this interface to offer MnSs to external MnS consumers. The MnS producer can optionally implement the AEF_Security_API service (see TS 29.222 [13], clause 9.2) in this interface. |
| CAPIF-3/3e | M | The MnS producer shall implement the consumer of the CAPIF_Security_API service (see TS 29.222 [13], clause 8.6), so that the MnS producer can: - obtain from the CCF the authentication or authorization information related to external MnS consumer. To that end, the MnS producer invokes “Obtain_API_Invoker_Info” operation. - invalidate the authorization of an external MnS consumer. To that end, the MnS producer invokes “Revoke_Authorization” operation. The MnS producer can optionally implement the consumer of the CAPIF_Events_API service (see TS 29.222 [13], clause 8.3), in case the MnS producer wants to subscribe/unsubscribe from CAPIF events and receive notifications accordingly. The event criteria include failure API invocation event, new API available event, API version change event, API location change event, etc. The MnS producer can optionally implement the consumer of the CAPIF_Access_Security_API (see TS 29.222 [13], clause 8.6), in case the MnS producer wants to obtain access control policy from the CCF for access control enforcement. Examples of this policy include limits (rate limit, volume limit, time limit) on MnS invocation per external MnS consumer. The MnS producer can optionally implement the consumer of the CAPIF_Logging_API_Innovation_API (see TS 29.222 [13], clause 8.7), in case the MnS producer wants to log the information related to service API invocations on the CCF. |
| CAPIF-4 | M | The MnS producer shall implement the consumer of the CAPIF_Publish_Service_API service (see TS 29.222 [13], clause 8.2), so that the MnS producer can - publish/unpublish MnS(s) on the CCF. To that end, the MnS producer invokes “Publish_Service_API” and “Unpublish_Service_API” operations. - Update information on published MnS(s) on the CCF. To that end, the MnS producer invokes “Update_Service_API” operation. - Retrieve information on published MnS(s) from the CCF. To that end, the MnS producer invokes “Get_Service_API” operation. |
| CAPIF-5 | M | The MnS producer shall implement the consumer of the CAPIF_API_Provider_Management_API (see TS 29.222 [13], clause 8.9), so that the MnS producer can register itself on CCF as an CAPIF API provider. The MnS producer can optionally implement the consumer ,CAPIF_Auditing_API (see TS 29.222 [13], clause 8.8), in case the MnS producer wants to query the log information stored on the CCF. |

##### 5.1.1.3.y	Potential solution #y: MnS producer registered with AEF and APF functionality.

###### 5.1.1.3.y.1	Introduction

This solution focuses on meeting the requirement PREQ-FS_MExpo-Reg-0x. It proposes that MnS producer is configured only to support AEF and APF, but not AMF. It is assumed that other entity performs this functionality.

###### 5.1.1.3.y.2	Description

The solution is the same as solution #x (see Table 5.1.1.3.x.2-1), with the exception that MnS producer does not require to support CAPIF-5 interface.

#### 5.1.1.4	Evaluation of potential solutions

Editor's Note:	This clause provides the evaluation of potential solutions.

##### 5.1.1.4.x	Evaluation of potential solution #x: MnS producer registered with AEF, APF and AMF functionality

By configuring a MnS producer to support AEF, APF and AMF functionality, the MnS producer becomes a self-contained API provider domain. The pros and cons of this solution are detailed below.

Pros:

- Well-specified solution. It is assured that each API provider domain function (AEF, APF and AMF) is played by a concrete entity in the 3GPP management system, in this case the MnS producer. If APF or and/or AMF were left out of the MnS producer, then it would mean someone else must play them instead. However, who is this entity is an open question, resulting in an incomplete solution from standardization viewpoint.

- Less integration efforts on operator side. When the operator decides that a MnS will be exposed using CAPIF, the operator is assured that for all the subsequent procedures associated to this MnS (e.g., publication, discovery, access control, etc.), only the producer of that MnS is needed. The reason is that all API provider domain functions involved in such procedures are performed by the MnS producer. With this approach, the operator does not need to worry about integrating MnS producer with other entities (e.g., 3rd party tools/solutions); it only needs to rely on the completeness of the MnS producer solution provided by the vendor.

- Innovation protection on vendor side. In case the MnS producer did not perform a given API provider domain function, the MnS producer would need to interact with the entity performing such missing function. This interaction means disclosing low-level information to this entity, which would put at risk the innovation capacity on vendor side in case that entity is a 3rd party tool/solution. With a self-contained implementation on MnS producer, this risk is mitigated.

- Simplified backend implementation on CCF side, since the MnS producer acts as single entry-point for CAPIF-3/4/5 interfaces. For procedures like API provider authentication and authorization, this gets greatly simplified.

Cons:

- More implementation complexity on vendor side. The developer of the MnS producer shall scope up the original functionality of that MnS producer, by:

  - implementing the interface operations summarized in Table 5.1.1.3.x.2-1

  - implementing the logic to map SA5 defined data types into CAPIF information elements. This mapping does not only include solution #1, but to the rest of procedures that will be detailed in upcoming use cases.

##### 5.1.1.4.y	Evaluation of potential solution #y: MnS producer registered with AEF and APF functionality

Using the solution #x as baseline, the pros and cons of the new solution are detailed below.

Pros:

- Less complexity on vendor side. The fact that MnS producer is not configured to support AMF means that the developer no longer requires to implement CAPIF-5 interfaces nor the mapping solution defined in solution #1.

Cons

- For the MnS producer to get registered on the CCF, AMF is needed. The entity performing AMF functionality needs to exist before MnS producer can be registered. This entity can be:

  - Another MnS producer implemented according to solution #x. This would mean that at least one of the MnS producers in the operator domain shall implement solution #x.

  - A non-specified entity within 3GPP management system. By leaving it intentionally open, the solution is not complete.

- Integration efforts on operator side. The MnS producer needs to pass registration details to the AMF, which puts on operator side the responsibility to ensure AMF and MnS producer can exchange this information.

End Change