3GPP TSG-SA5 Meeting #156	S5-244439

Maastricht, Netherlands, 19 - 23 August 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0602 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.552 Correction of DL ITI Time Domain Proportion |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph4 |  |  |  |  |  | Date: |  |  | 2024-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The description of the denominator in the calculation formula of DL ITI Time Domain Proportion is not accurate, and it is more appropriate to change 'DL data scheduled' to 'DL data transmitted'. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the description of the denominator in the calculation formula of DL ITI Time Domain Proportion from 'DL data scheduled' to 'DL data transmitted'. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description of the denominator in the calculation formula of DL ITI Time Domain Proportion is not accurate. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.2.20 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

#### 5.1.1.2.20 DL ITI Time Domain Proportion

a)	This measurement provides the proportion of time domain resources that invoke the downlink interrupted transmission indication feature (see clause 11.2 of TS 38.213 [64], and it will be represented as ITI (Interrupted Transmission Indication) in the following) in the statistical period. Taking a fixed time duration as one sampling occasion, the numerator of this measurement is the number of sampling occasions that invoke the ITI feature (when the number of preempted PRBs is greater than 0) and the denominator is the number of sampling occasions with DL data transmitted (eMBB, URLLC, etc.).

b)	SI

c)	This measurement is obtained as: PITIT=ITITNDTT*100,

where

PITI(T) denotes the proportion of time domain resources that invoke the ITI feature during the time period T, with value range: 0-100%;

ITI(T) is the number of sampling occasions that invoke the ITI feature during the time period T,  ITI(T)=iITIi(T);

ITIiT is the invoking ITI feature result of sampling occasion i,  when the number of physical resource blocks (PRBs) that invoke the ITI feature at sampling occasion i is greater than 0, ITIi(T) = 1, and when the number of PRBs that invoke the ITI feature at sampling occasion i is equal to 0, ITIi(T) = 0;

NDT(T) is the number of sampling occasions with DL data transmitted during the time period T,  NDT(T)=iNDT,i(T)；

NDT,i(T) is the DL data transmitted result of sampling occasion i, when there is DL data transmitted at sampling occasion i, NDT,i(T)=1, and when there is no DL data transmitted at sampling occasion i, NDT,i(T)=0;

NOTE: DL data transmitted is transmitted data of user plane, such as eMBB data, URLLC data, etc.

T denotes the time period during which measurement is performed;

i denotes sampling occasion (e.g. 1 slot) during time period T.

d)	A single integer value from 0 to 100.

e)	RRU.ITiDtDl, which indicates the proportion of time domain resources that invoke the ITI feature

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for evaluating the resource load of URLLC services under eMBB and URLLC 		multiplexing scenarios.

End of Change