3GPP TSG-SA5 Meeting #157	S5-245222

Hyderabad, India, 14 - 18 October 2024

Title:	Reply LS on AIML data collection

Response to:	LS (RP-242389) on AIML data collection from RAN

Release:	Rel-19

Work Item:	FS_AIML_MGT_Ph2

Source:	SA5

To:	RAN

Cc:	RAN1, RAN2, RAN3

Contact person:	Pengxiang Xie

xie.pengxiang@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1	Overall description

1. Overall Description:

SA5 thanks RAN for the LS on AIML data collection in document RP-242389.

SA5 would like to response to the questions regarding to option 3 with the following:

| Aspects | Option 3 |
| --- | --- |
| First termination entity | Inside OAM domain |
| AI/ML-specific Data Transfer Path | UE-> gNB->OAM -> Server for data collection for UE-side model training/OTT server (Note 4) |
| UP/CP tunnel | CP tunnel (provided that the data volume remains within the RRC signalling capacity) FFS: UP tunnel (Note 7) |
| Protocol layer for data transfer | RRC layer for CP tunnel FFS: the protocol layer for UP tunnel |
| Controllability of MNO on data transfer (Note 1) | Full controllability |
| Solution for network controllability | Via RRC procedure |
| Possible Options for Visibility of data content in MNO and Data format (Note 2, Note 3) | Opt A) Full visibility for standardized data contents. Opt B) Partial visibility for partially standardized data contents. (Note 6) Opt C) No standardized visibility. (Note 6) |
| Impacted WGs | RAN2, RAN3, SA3, SA5, SA2 |
| - Note 1: Full controllability: The MNO can manage data transfer to the server for UE-side data collection, without the need of SLA. This includes initiating, terminating, and fully managing data transfer. - Note 2: Visibility of data content signifies that the MNO can, at least, be aware of, access, and comprehend the data without the need of SLA. - Note 3: The following options are identified to realize the different levels of data content visibility to the MNO: - Full visibility for standardized data content. - Partial visibility for partially standardized data content (e.g. UE proprietary information can be included transparently together with the standardized data message). - No standardized visibility (e.g. only UE proprietary information can be included transparently). - Note 4: The potential involvement of NF or other higher layers entities/functionalities should be discussed in other WGs. Impact on the OTT server is not in the scope of RAN2 discussion. - Note 5: RAN2 cannot reach consensus on the level of MNO controllability and visibility possible via solution 1b without input from SA groups. - Note 6: RAN2 has not concluded on the need for partial and no visibility options. Note 7: RAN2 could not reach consensus on the feasibility of UP solution in options 2 and 3. |  |

Management of UE is out of the scope of SA5 and AIML data collection is not specified in the ML model lifecycle in TS 28.105. MnS Consumer can indicate the address(es) of the candidate training data source but the detailed training data format is vendor-specific.

In RP-242389, the content of UE side data collection is not elaborated. SA5 has defined the following measurement collection mechanism regarding UE data collection.

- Trace & MDT. UE level measurements for the 5G system are documented in TS 28.558. NG-RAN UE level measurements collection and reporting are supported by MDT mechanisms for both signalling and management-based activation, which are detailed in clause 4 of TS 32.422

- QoE measurements. The measurements collected are DASH, MTSI, and Virtual Reality measurements, which include collecting QoE data from UEs in the specified area in case of Management Based Activation and an individual UE in case of Signalling Based Activation. The corresponding collection procedure and QoE measurement collection record content are detailed in TS 28.405 and TS 28.406, respectively.

- SA5’s view on aspects of Option 3 in the table above is as follows:

SA5’s view on aspects of Option 3 in the table above is as follows:

- First termination entity  & AI/ML-specific Data Transfer Path

For Trace & MDT, the gNB will send collected data to the data consumer (e.g., TCE) directly or via the management system by indicating the traceCollectionEntityIPAddress or traceReportingConsumerUri in the TraceJob instance. Details can be found in clause 4.3.30 of TS 28.622. For QoE Measurements, the gNB will send QMCRecord to the Measurement Collector Entity (MCE) associated with the qoECollectionEntityAddress specified in A QMCJob instance. Details can be found in clause 4.3.54 of TS 28.622. The first termination entity for Trace, MDT, and QoE measurements can be inside the OAM domain.

- UP/CP tunnel & Protocol layer for data transfer & Controllability of MNO on data transfer & Solution for network controllability

- CP Tunnel. For the data collection mechanism (Trace, MDT, QMC) above, only the control plane tunnel is supported. Data collection configuration parameters and corresponding data measurements reported are within the RRC signalling.

- UP Tunnel. Data collection via user plane is not discussed in SA5.

The existing data collection mechanism only supports data transferred within the RRC signalling in the CP tunnel, which is fully controlled.

- Possible Options for Visibility of data content in MNO and Data format

- Trace. gNB-CU-CP Trace Record Content defines the collected data on the Uu interface and detailed information can be found in TS 32.423. When the Trace Depth is at Minimum level or Medium level, IE extracted from RRC messages is as per TS 38.331. When the Trace Depth is Maximum level, raw Uu messages are provided.

- MDT. Trace Record for immediate MDT is as per different MDT Measurement (e.g., M1).

- QoE Measurement Collection Content. QoE measurements are contained in the measReportAppLayerContainer, which is sent to the gNB by the Access Stratum from the UE application layer in the message MeasurementReportAppLayer. QoE metrics for 3GP-DASH, VR, and MTSI are defined by SA4 in TS 26.247 and TS 26.114.

The definition of AIML data in the received LS is unclear. According to the observation above, currently, only standardized metrics can be collected from UE.

If QoE metrics are in the scope of AIML data collection, SA4 should be involved in the discussion. If trace and MDT mechanisms are used for AIML data collection from UE, the possible negative impact on gNB needs to be evaluated holistically. For example, the transferred data volume, the number of impacted UE, data collection frequency, and the potential negative impact on the performance of gNB all should be taken into consideration. Besides, security issues on AIML data collection from UE in option 3 should be discussed in SA3.

2	Actions

To RAN

ACTION: 	SA5 kindly asks RAN to take the SA5’s reply above into consideration.

3	Dates of next TSG SA WG5 meetings

SA5#158		18 November - 22 November 2024		Orlando, USA

SA5#159		17 February - 21 February 2025		Sophia-Antipolis, France