3GPP TSG-SA5 Meeting #157	S5-245293

Hyderabad, India, 14 - 18 October 2024

# Source:	Huawei

# Title:	Conclusion and recommendation for multi-dimensional energy efficiency metrics

# Document for:	Approval

Agenda Item:	6.19.20

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1]		3GPP TR 28.880 v1.0.1: Study on energy efficiency and energy saving aspects of 5G networks and services

# 3	Rationale

This contribution proposes to add conclusion and recommendation for multi-dimensional energy efficiency metrics in TR 28.880 [1].

# 4	Detailed proposal

This document proposes the following changes in TR 28.880 [1].

1st Change

# 6	Conclusions and recommendations

## 6.6	Use case #6: Multi-dimensional network energy efficiency metrics

The following bullets are recommended for normative work for use case #6:

1)	Define multi-dimensional EE metrics as per the following categories:

-	Network data traffic, which measures the number or amount of network traffic provided.

-	For example, the amount of data transferred as the Data Volume used in EE KPI, the number of connections supported, etc.

-	As proposed by potential solution #1, no need to introduce new EE KPIs for this category in normative work.

-	Network quality, which measures the quality of network provided.

-	Such as user throughput, coverage quality, or connection quality, etc.

-	New EE KPIs, as proposed by potential solution #3, need to be introduced for this category in normative work.

-	Network availability, which measures the availability of network.

-	A typical example in wireless networks is the cell availability.

-	New EE KPIs, as proposed by potential solution #2, need to be introduced for this category in normative work.

2)	The new EE KPIs to be defined in normative work are listed in Table 6.6-1.

Table 6.6-1: New EE KPIs evaluated from network quality and network availability performance dimensions

| Performance dimension to evaluate EE KPIs |  | New EE KPIs | Note |
| --- | --- | --- | --- |
| Network quality | UE throughput as the network quality performance indicator | DL RAN UE throughput for a NRCellDUECcell | This new EE provides the energy efficiency of a cell. This new EE KPI is obtained by DL RAN UE throughput for a NRCellDU KPI divided by the energy consumption of the cell over the same observation period. |
|  |  | DL RAN UE throughput for a sub-networkECcell | This new EE provides the energy efficiency of a sub-network. This new EE KPI is obtained by DL RAN UE throughput for a sub-network KPI divided by the energy consumption of the sub-network over the same observation period. |
|  |  | DL RAN UE throughput for a network slice subnetECnetwork slice subnet | This new EE provides the energy efficiency of a network slice subnet. This new EE KPI is obtained by DL RAN UE throughput for a network slice subnet KPI divided by the energy consumption of the network slice subnet over the same observation period. |
|  |  | UL RAN UE throughput for a NRCellDUECcell | This new EE provides the energy efficiency of a cell. This new EE KPI is obtained by UL RAN UE throughput for a NRCellDU KPI divided by the energy consumption of the cell over the same observation period. |
|  |  | UL RAN UE throughput for a sub-networkECsub-network | This new EE provides the energy efficiency of a sub-network. This new EE KPI is obtained by UL RAN UE throughput for a sub-network KPI divided by the energy consumption of the sub-network over the same observation period. |
|  |  | UL RAN UE throughput for a network slice subnetECnetwork slice subnet | This new EE provides the energy efficiency of a network slice subnet. This new EE KPI is obtained by UL RAN UE throughput for a network slice subnet KPI divided by the energy consumption of the network slice subnet over the same observation period. |
| Network availability | Availability as the network availability performance indicator | Cell AvailabilityECcell | This new EE KPI provides the energy efficiency of a cell. This new EE KPI is obtained by Cell Availability KPI divided by the energy consumption of the cell over the same observation period. |
|  |  | Radio access network availabilityECNG-RAN | This new EE KPI provides the energy efficiency of an entire RAN sub-network. This new EE KPI is obtained by Radio access network availability KPI divided by the energy consumption of the entire RAN sub-network over the same observation period. |

End of change