3GPP TSG SA WG5 Meeting #157 	S5-245335

Hyderabad, India, 14 - 18 October 2024

# Source:	CATT

# Title:	Add evaluation for store and forward satellite operation charging

# Document for:	Approval

# Agenda Item:	7.5.1

1	Decision

The group is asked to discuss and agree on the proposal.

2	References

[1]		3GPP TR 28.846: " Study on charging aspects of satellite access Phase 3 ".

3	Rationale

This pCR proposes to add evaluation for S&F operation charging.

4	Detailed proposal

The following changes are proposed to be incorporated into the new TR.

1st Modified Section

6.1.5	Evaluation

5.1.5.1	Solutions evaluation for Key issue #1.1

Solutions #1.1 addresses Key issue #1.1, for the SMS service using the satellite access running in the S&F mode, with the enhancement of the new trigger events and charging information. The new trigger events are Short Message received by a UE via the MME on-board (MT direction) from the SMSC and Short Message received by a UE via the MME on-board (MT direction) from the SMSC. The enhancement of the charging information including RAT type, Satellite Access Indicator, Store and Forward indicator and Satellite information.

Solutions #1.2 addresses Key issue #1.1, for the Control Plane CIoT using the satellite access running in the S&F mode, with the new trigger events and charging information. The new trigger events are Attach Complete via the MME onboard for the CIoT CP Optimizations and Service Request via the MME onboard for the CIoT CP Optimizations. The charging information including RAT type, Satellite Access Indicator, Store and Forward indicator and Satellite information.

5.1.5.2	Solutions evaluation for Key issue #1.2

Both solutions #1.1 and #1.2 address Key issue #1.2. Solution #1.1 enhances the MME over SMS charging that MME provides the charging information. Solution #1.2 directly uses the MME provides the charging information and provides the message flow.

End of Modified Sections