# 3GPP TSG-SA5 Meeting #157	S5-245408

# 14 - 18 October 2024, Hyderabad, India

# Source:	Nokia

# Title:	pCR 28.867 Enhance solution on Triggered CCL

# Document for:	Approval

# Agenda Item:	6.19.4

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1] 	3GPP TR 28.867-041 “Closed control loop management”.

# 3	Rationale

TR28.867 defines a use cae of historicl CCL which provides data to enale a CCL to be instantiated based on data from previous CCLs. The usecase and solution does not show how that data can be used.  This pCR enhances the  UC show how that data can be used for instantiating the new CCL.

# 4	Detailed proposal

Start of modification

# 5. 			Use Cases

## 5.2	Use case 2: Triggered CCL

### 5.2.1	Description

The existing CCL mechanism enables consumer to request the initiation of a CCL with the goal to maintain particular SLS (indicated by the AssuranceGoal). The CCL is expected to monitor the network to see if there have been some goal breaches. If there is, the consumer is notified, and the appropriate actions can be taken to mitigate the breach by the consumer. The consumer may also decide to update the existing CCL or create a new one to mitigate the breach. A CCL is always instantiated, updated and deleted on an explicit request from the consumer.

#### 5.2.1.1	Conditional instantiation of a CCL

Considering the autonomous nature of CCL, it is beneficial to study possible improvements to CCL management including automated instantiation, update and deletion of a CCL based on information provided by the consumer that could be used by the system to trigger CCL management. The existing CCL mechanism places a burden on the consumer to monitor the network and decide whether to instantiate a CCL, update a CCL, or delete a CCL. A possible improvement may be to allow the consumer to define trigger conditions for automated instantiation, update and deletion of a CCL.

The MnS consumer may want to request for a CCL to be dynamically instantiated when certain conditions are met. For example, the MnS consumer may want that for a CCL of a stated type or that matches a set of stated characteristics (e.g. goal) to be instantiated under conditions A and another with variations in goals to be instantiated under other conditions. The MnS consumer should be enabled to define those conditions so that the CCL is instantiated when the stated conditions are met.

The ConditionMonitor[x], post appropriate extensions, can be utilized to define triggering conditions for CCL management.

#### 5.2.1.2	Conditional execution of CCLs network changes

For the CCLs that have been instantiated, the MnS consumer may want to request for a CCL to be triggered to execute when certain conditions are met, e.g. when the performance on a certain threshold is crossed, or when the confidence is the decision is above a stated threshold. The consumer does not need to be aware of all decision, but providing conditions under which decisions may be activated or not, it is able to have supervision over the CCL without having to continuously track the decisions. The MnS consumer should be enabled to define those conditions for executing the CCL and that the CCL is triggered to execute when the stated conditions are met. Otherwise, the consumer should be enabled to define alternative actions, e.g. to notify the consumer of the decision that is not executed.

By supporting this, the execution can be affected by producer based on consumer's conditions or requirements.

### 5.2.2	Potential Requirements

REQ-TRI-FUN-01: The 3GPP management system shall enable authorized consumers to provide information that can be used to trigger CCL instantiation.

REQ-TRI-FUN-02: The 3GPP management system shall enable authorized consumers to provide information that can be used to trigger CCL update.

REQ-TRI-FUN-03: The 3GPP management system shall enable authorized consumers to provide information that can be used to trigger CCL deletion.

REQ-TRI-FUN-03: The 3GPP management system shall enable authorized consumers to provide information that can be used to determine effectuation of CCL actions.

### 5.2.3	Potential solutions

#### 5.2.3.1	Overview of LoopTrigger object

This solution proposes LoopTrigger object that would contain information a producer would use to trigger a CCL. The clause 5.2.3.1 specify the potential information to be present in this object. Clause 5.2.3.2 specify the usage of condition monitor to implement the LoopTrigger object.

The MnS producer receives a request from the MnS consumer to instantiate the Loop trigger object, which holds the conditions and information needed to create an instance of CCL of a given type, e.g. of type energy efficiency optimization. The LoopTrigger can be named contained in a subnetwork or managed function. When the conditions are fulfilled, the MnS producer creates the instance of the CCL and configures the CCL instance based to the configuration information provided in the loop object. Then when the appropriate conditions are fulfilled, the MnS producer triggers execution of the CCL to derive network configuration changes, e.g. the MnS producer activates the CCL. Then for an activated CCL which has derived actions, if the execution of actions is not conditional, the CCL executes the actions. Other the CCL holds the actions and only when the appropriate conditions for execution of actions on to the network are fulfilled, the CCL triggers execution of the actions onto the network,

Figure 5.2.3.1-1: triggering CCL instantiation, execution and effectuation of actions

#### 5.2.3.1	Information to be present in LoopTrigger object

Performance based criteria: This will define information related with performance measurements and KPIs that need to be monitored by the producer to see if the values have crossed the thresholds defined. This will include:

-	Target Node: The identification of the Managed Object for which the performance is to be monitored.

-	Measurement/KPI Name: Name of the measurement or the KPI.

-	Trigger Value range: The CCL shall be triggered when the value of the measurement or KPI exceeds more that the trigger value or when the value decreases below the trigger value.

Once the Trigger Value has reach, the producer will send a notification to the consumer stating that an CCL is required. The notification will contain information needed to instantiate an CCL. The CCL shall be triggered appropriately.

Provisioning based criteria: This will define various provisioning events that need to be monitored by the producer to see if an CCL is to be initiated:

-	Target Node: This can be a particular object or a DN, e.g. Intent.

-	Provisioning Location: The CCL will be created only when the object created is targeting a specific location.

-	Provisioning Event (e.g. Create{in case of an object}, Modify, Delete): The CCL will be created when the given event occurs on the given DN.

-	Provisioning Time: The CCL will be created only when the given event occurs at a specified time.

-	PreOrPostProvEvent: This will define if the CCL is to be instantiated before or after the provisioning event is completed.

Fault based criteria: This will define various fault related info that need to be monitored by the producer to see if an CCL is to be initiated:

-	Target Node: This will define the node which need to be monitored for the emitted alarms (i.e. objectInstance in AlarmInformation).

-	AlarmSeverityThreshold: This will define the "perceivedSeverity" threshold (i.e. threshold for each Severity). If total number of alarms, belonging to particular perceivedSeverity (e.g. critical, major etc.), goes beyond the threshold, an CCL will be instantiated.

-	AlarmTypeThreshold: This will define the "AlarmType" threshold (i.e. threshold for each AlarmType). If total number of alarms, belonging to a particular alarmType, goes beyond the threshold, an CCL will be instantiated.

#### 5.2.3.2	Usage of ConditionMonitor to realize LoopTrigger object

This LoopTrigger object can be inherited from ConditionMonitor. The existing condition attribute will be extended to include various type of triggers provided in clause 5.2.3.

The condition will be defined as a datatype containing following information:

- conditionObject: This is to represent the target node i.e. object for which the performance and fault is to be monitored.

- conditionInfo: This is a set of multiple conditions that should be satisfied for a CCL to be instantiated:

-	conditionItem: This will be the PM data name.

-	conditionValue: This is to represent the expected value of the measurement or KPI.

-	conditionString: This will be the logical assertion related to conditionItem and conditionValue ("is equal to", "is less than", etc.).

ConditionMonitor can be used to define the fault-based criteria as follows: the existing condition attribute will be defined as data type including the following information:

- conditionObject: This is to represent the target node. This will define the node which need to be monitored for the emitted alarms (i.e. objectInstance in AlarmInformation).

- conditionInfo: This is a set of multiple conditions that should be satisfied for a CCL to be instantiated:

-	conditionItem: This may represent the total number of alarms with particular alarmType or perceivedSeverity.

-	conditionValue: This is to represent the expected value.

-	conditionString: This will be the logical assertion related to conditionItem and conditionValue ("is equal to", "is less than", etc.).

ConditionMonitor can be used to define the provisioning-based criteria as follows: the existing condition attribute will be defined as data type including the following information:

- conditionInfo: This is a set of multiple conditions that should be satisfied for a CCL to be instantiated:

-	conditionItem: This may represent the following a) The DN at which the provisioning operation is performed. b) the location of the instantiated DN c) the provisioning operation executed d) the time at which the provisioning operation is executed d) time detail specifying where it is the pre or post provisioning operation.

-	conditionValue: This is to represent the expected value.

- conditionString: This will be the logical assertion related to conditionItem and conditionValue ("is equal to", "is less than", etc.).

#### 5.2.3.3	Expressing trigger conditions using existing SA5 defined mechanisms

##### 5.2.3.3.1	Introduction

SA5 has established notations to express conditions:

-	Jex for the HTTP/JSON solution.

-	XPath for the NETCONF/YANG solution.

Conditions are expressed based on data nodes in the data node tree on a MnS producer.

Currently there are no data nodes defined for performance metrics and trace metrics. Therefore, it is not possible to express conditions based on these metrics.

Furthermore, it is not possible to build conditions that evaluate structural changes of the data node tree such as the creation or deletion of a data node.

This clause proposes mechanisms to address these gaps. The motivation for doing so is to have a single notation allowing to express conditions of all kinds and for all use cases, and to avoid silo solutions for the different kinds of conditions or silo solutions for some use case like triggering the creation of a CCL.

##### 5.2.3.3.2	Conditions based on performance metrics

Stage 2 considerations

Data nodes for performance metrics need to be introduced to allow for conditions based on performance metrics. For this purpose, the stage 2 NRM definitions are conceptually extended with pseudo attributes representing these performance metrics. These attributes are not readable and not writable by MnS consumers. They can be used only in condition expressions that are evaluated on MnS producers. The name of these attributes is equal to a measurement name defined in 3GPP TS 28.552 [8] or the KPI name defined in 3GPP TS 28.554 [9]. Its value is equal to the current value of the measurement or the KPI. A pseudo attribute for a specific performance metric can be (conceptionally) added only to the object class that is equal to the measurement object class of that measurement.

Pseudo attributes are not added explicitly to NRM stage 2 definitions. Alternatively, as a more formal approach, they could be added also to the stage 2 NRM definitions. Adding them directly to all measurement classes is feasible but not practical. A practical solution could be to add them to the "TopX" class, from which every other class inherits.

Table 5.2.3.3.2-1: Attributes of "TopX"

| Attribute Name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| objectClass | M | T | T | T | T |
| objectInstance | M | T | T | T | T |
| <measurementName1> | O | T (see note) | F | F | F |
| <measurementName2> | O | T (see note) | F | F | F |
| … | O | T (see note) | F | F | F |
| <measurementNameX> | O | T (see note) | F | F | F |
| NOTE: Attribute is not readable using CRUD operations. It can be accessed only by condition expressions. |  |  |  |  |  |

In a condition expression the measurement pertaining to a measured object is identified by concatenating the "objectInstance" of the measured object with the "measurementName".

The MnS producer evaluating the condition expression must have access to the performance metric values. This is assumed here.

NOTE 1:	So far conditions are designed to switch on and off a process: When the condition evaluates to false the process is (switched) off, when the condition evaluates to true, the process is (switched) on. For the present use case the condition needs to trigger an action. The triggering condition is therefore the transition of the condition evaluation result from false to true or true to false. The triggering condition needs to be specified together with the condition expression.

In the context of CCL, a CCL can be created when the condition expression transits from false to true and deleted when the condition expression transits from true to false.

Stage 3 considerations

Jex is used in the HTTP/JSON solution for expressing conditions. XPath is used in the NETCONF/YANG solution for expressing conditions.

Jex example:

The following example shows a Jex conditions expression with a pseudo attribute for the measurement "measurementA".

SubNetwork[id="SN1"]/ManagedElement[id="ME1"]/XyzFunction[id"=YXZF1"]/attributes/measurementA >10

When hysteresis is needed, the "ThresholdMonitor" can be used. It needs to be extended with the Boolean attribute "flag" flagging if the threshold is not crossed or crossed. For multi-level thresholds integers need to be used to flag the status. The "flag" attribute can be used in Jex conditions.

SubNetwork[id="SN1"]/ThresholdMonitor[id="TM1"]/attributes/flag = true

NOTE 2:	The MnS producer evaluating the condition expression must have access to the performance metric values. This is assumed here.

XPath example:

The XPath notation is the same as the Jex notation. Therefore, the Jex examples are examples for XPath, too.

##### 5.2.3.3.3	Conditions based on Trace metrics

Stage 2 considerations

To allow for conditions based on trace metrics the NRM is conceptually extended with pseudo attributes. These attributes are not readable and not writable by MnS consumers. They can be used only in condition expressions that are evaluated on MnS producers. The name of these pseudo attributes is equal to the trace metric identifier.

Pseudo attributes are not added to stage 2 NRM definitions. Alternatively, the approach described for measurements may be used:

-	Adding the trace metric identifiers to "TopX".

NOTE:	The MnS producer evaluating the condition expression must have access to the trace metric values. This is assumed here.

For trace metrics there are two use cases, which are described in the following. For signalling based MDT only use case 1 applies.

Stage 3 considerations

Jex is used in the HTTP/JSON solution for expressing conditions. XPath is used in the NETCONF/YANG solution for expressing conditions.

Use case 1: MnS consumer is aware of corresponding "TraceJob" MOI

The pseudo attribute is attached to the corresponding "TraceJob" MOI. The pseudo attribute name is constructed according to the metric identifier specified in 3GPP TS 32.422 [6]. Its value is equal to the current value of the measurement or information element.

Examples (stage 3):

The following example shows a Jex conditions expression to verify whether "REGISTRATION REQUEST" message on N1 interface between AMF and UE has been traced. It uses a pseudo attribute for the "REGISTRATION REQUEST" message.

SubNetwork[id="SN1"]/ManagedElement[id="ME1"]/TraceJob[id="YXZF1"]/attributes/ trace.amf.n1.registrationRequest=true

The following example shows a Jex conditions expression to verify whether the Information Element (IE) "Requested NSSAI" of the "REGISTRATION REQUEST" message on N1 interface between AMF and UE has been included in the message content. It uses a pseudo attribute for Information Element (IE) "Requested NSSAI".

SubNetwork[id="SN1"]/ManagedElement[id="ME1"]/TraceJob[id="YXZF1"]/attributes/ trace.amf.n1.registrationRequest.requestedNssai=true

The following example shows a Jex conditions expression with a pseudo attribute for the RRC counter "countMSB-Uplink".

SubNetwork[id="SN1"]/ManagedElement[id="ME1"]/TraceJob[id="YXZF1"]/attributes/ trace.gnbCuCp.uu.CounterCheck.counterCheck.drb-CountMSB-InfoList.countMSB-Uplink>500

The following example shows a Jex conditions expression with a pseudo attribute for the RSRP value.

SubNetwork[id="SN1"]/ManagedElement[id="ME1"]/TraceJob[id="YXZF1"]/attributes/ immediateMdt.nr.m1.rsrp<90

Use case 2: MnS consumer knows the corresponding network entity MOI

The containment tree is used to identify the corresponding network entity (network element/interface). The pseudo attribute is attached to the corresponding network entity MOI. The pseudo attribute indicates the message name, message name and IE name or measurement name. The pseudo attribute name is constructed according to the metric identifier specified in 3GPP TS 32.422 [6] omitting, in case of trace, the items of job type, network element and interface.

If there is no corresponding interface IOC defined, the interface name is included in the pseudo attribute name.

Examples (stage 3):

The following example shows a Jex conditions expression to verify whether a "Nudm_UECM_Registration_Request" message on N8 interface between AMF and UDM has been traced. It uses a pseudo attribute for message "Nudm_UECM_Registration_Request".

SubNetwork[id="SN1"]/ManagedElement[id="ME1"]/UDMFunction[id="YXZF1"]/ EP_N8[id="XXZ2"]/attributes/Nudm_UECM_Registration_Request=true

The following example shows a Jex conditions expression to verify whether a "REGISTRATION REQUEST" message on N1 interface between AMF and UE has been traced. As there is no IOC specified for the N1 interface, the N1 interface is included in the pseudo attribute name "n1.registrationRequest".

SubNetwork[id="SN1"]/ManagedElement[id="ME1"]/AMFFunction[id="YXZF1"]/attributes/ n1.registrationRequest=true

The following example shows a Jex conditions expression to verify whether the information element (IE) "Requested NSSAI" of the "REGISTRATION REQUEST" message on N1 interface between AMF and UE has been included in the message content.

SubNetwork[id="SN1"]/ManagedElement[id="ME1"]/AMFFunction[id="YXZF1"]/attributes/ n1.registrationRequest.requestedNssai=true

The following example shows a Jex conditions expression to verify whether the immediate MDT measurement M1 RSRP in NR is above a certain threshold.

SubNetwork[id="SN1"]/ManagedElement[id="ME1"]/GNBCUCPFunction[id="XYY"]/NRCellCU[id="YXZF1"]/ attributes/immediateMdt.nr.m1.rsrp>90

##### 5.2.3.3.4	Conditions based on data node tree changes

XPath allows to express conditions based on data node tree changes with the "count" function.

Jex does not feature the "count" function yet. It is proposed to add the "count" function as follows:

-	Equality and relational expressions may have on the left side of the operator also the "count" function, defined in clause 4.1 of the W3C XPath1.0 specification. The "count" function returns the number of nodes in the argument node-set. The argument is an absolute location path or a relative location path:

EqualityExpr   ::= CompOperandExpr EquaComp (String| Number | true | false | null)

RelationalExpr ::= CompOperandExpr RelaComp Number

CompOperandExpr ::= LocationPath | 'count' '(' LocationPath ')'

EquaComp ::= '=' | '!='

RelaComp ::= '<' | '>' | '<=' | '>='

Examples:

The "count" function can be used to test for the presence of objects. The following example evaluates to true only when at least one "ManagedElement" object exists under the "SubNetwork" object. The "ManagedElement" indicates the particular object or a DN, e.g. Intent.

count(/SubNetwork[id="SN1"]/ManagedElement)>=1

NOTE:	An expression using the proposed count function can be combined with expressions testing other things (defined as part of Provisioning based criteria in clause 5.2.3.1) such as the value of one or more attribute. Note that the Jex notation currently specified in 3GPP TS 32.161 [10] already allows for testing these other things.

count(/SubNetwork[id="SN1"]/ManagedElement)>=1\ and /SubNetwork[id="SN1"]/ManagedElement/attributes/location="TV Tower"\ and /SubNetwork[id="SN1"]/ManagedElement/attributes/vendor="Company XY"

In the context of the present use case testing on the presence of an intent object that triggers the creation of a closed loop is a realistic deployment scenario.

#### 5.2.3.4	Potential solution for Conditional execution of CCLs network changes

To enable dynamic Conditional decision activation of the CCL:

- Introduce on the CCL IOC, an attribute representing the set of conditions to be monitored for activation of the CCL decisions. The condition may represent the context under which the CCL may execute actions or not. The attribute may be of type threshold monitor defined in 3GPP TS 28.622 [11], condition monitor as defined in 3GPP TS 28.622 [11], the condition expressed in form of a JEX/XPATH expression.

NOTE 1:	The conditions could alternatively be part of the goal.

- Introduce on the CCL IOC, an attribute representing the triggered behavior. This will define the corresponding behavior of the CCL. The behaviors can be represented by an ENUM to include:

-	DECISION_ACTIVATION: The Loop executed the recommendations that it derives on to the network.

-	NOTIFY_RCOMMENDATION: The Loop starts processing input to derive recommendations but without the corresponding actions executed on the network. Instead, the recommendation is notified to the consumer who then considers whether it should be applied or not.

NOTE 2:	The internal action of the CCL is not communicated to the MnS consumer.

-	DO_NOTHING: do not do anything.

NOTE 3:	The trigger behavior may be added into the MnS consumer's defined condition.

### 5.2.4	Evaluation of solutions

It is recommended to express conditions triggering the creation of a CCL using the method that is used to express conditions for other use cases: Jex in HTTP/JSON and XPath in NETCONF/YANG.

For enabling to express all conditions required in the context of CCL creation, the stage 2 and stage 3 enhancements outlined in clause 5.2.3 are recommended to be standardized.

Furthermore, it is recommended to use conditional activation of planned configurations (3GPP TR 28.872 [7], clause 5.6) as a solution for the use case Triggered CCL.

Next modifications

# A.1	Procedures for CCL Management

# A.1.X	Detection and avoidance of potential goal targets conflicts (figure 6.2.1.1.1-2)

@startuml Detection and avoidance of potential goal targets conflicts

skinparam Shadowing false

autonumber

skinparam monochrome true

skinparam maxMessageSize 200

participant "CCL MnS Consumer" as CX

participant "CCL MnS producer" as CP

participant "CCL" as CCL

participant "Network" as Net

CX -> CP: createMOI (LoopTrigger, LoopTriggerAttributesData)

CP -> CP: evaluate CCL instantiation conditions in LoopTrigger

Alt CCL instantiation conditions are fulfilled

CP -> CP: Instantiate the CCL

CP -> CCL: configure the CCL with information in LoopTrigger

CP -> CP: evaluate CCL activation conditions in LoopTrigger

Alt CCL activation conditions are fulfilled

CP -> CCL: Trigger execution of CCL

CCL -> CCL: derive network control actions

Alt Execution of CCL network control actions is not conditional

CCL -> Net: Execute actions onto network

Else Execution of CCL network control actions is conditional

CP -> CP: evaluate CCL decision-execution conditions in LoopTrigger

Alt CCL decision-execution conditions are fulfilled

CP -> CCL: Trigger execution of CCL

CCL -> Net: Execute actions onto network

End

end

end

End

@enduml

End of modifications