3GPP TSG-SA5 Meeting #157	S5-245637

Hyderabad, India, 14 - 18 October 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.554 | CR | 0208 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 28.554 Energy efficiency per node type for NwSlice |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph3 |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The energy efficiency can be calculated as dependent on several things, the current main things specified are data volume, registered subscribes, and PDU Sessions. Having the AMF or SMF energy consumption as depending the data volume will be miss leading since the energy consumption for these will not change depending on the number of bytes transferred. Having them separated based on how their usage dependency are will make it easier to calculate a better and more relevant view of the energy efficiency for the network slice. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding separate energy efficiency calculations for the different 5GC NFs depending on their type of usage for, AMF the number of registered subscribers, SMF the number of PDU sessions and gNB, UPF, and PSA UPF the data volume. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It will be not be possible to calculate the energy efficiency of a network slice based on different aspects. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.2.x (new), 6.7.2.y (new), and 6.7.2.z (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS 28.554 CR 0207 |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

#### 6.7.2.x	Energy Efficiency of data volume related network slice network functions

a)	EE,ns,NF,DV

b)	A KPI that shows the energy efficiency of network functions composing the network slice that have efficiency related to data volume (i.e., gNB, UPF and PSA UPF). This KPI is obtained by the summing up UL and DL data volumes at N3 interface(s) and dividing this with the energy consumption of the network functions (gNB, UPS and PSA UPF respectively). The unit of this KPI is bit/J.

c)

EEns,NF,DV=UPFGTP.InDataOctN3UPF.SNSSAI+GTP.OutDataOctN3UPF. SNSSAI×1gNBECns,gNB+1UPFECns,UPF+1PsaUpfECns,PsaUpf×8

d) NetworkSlice

Second change

#### 6.7.2.y	Energy Efficiency of registered subscribers related network slice network functions

a)	EE,ns,NF,RegSubs

b)	A KPI that shows the energy efficiency of network function composing the network slice that have efficiency related to number of registered subscribers (i.e., AMF). This KPI is obtained by the summing up the mean number of registered subscribers of the network slice relatively divided by the energy consumption of the network functions and dividing this by the number of network functions (AMFNbr). The unit of this KPI is RegSubs/J.

c)

EEns,NF,RegSubs=AMFRM.RegisteredSubNbrMean.SNSSAIECns,AMF AMFNbr

d) NetworkSlice

Third change

#### 6.7.2.z	Energy Efficiency of PDU sessions related network slice network functions

a)	EE,ns,NF,PDUSes

b)	A KPI that shows the energy efficiency of network function that have efficiency related to number of PDU sessions (i.e., SMF), that compose the network slice. This KPI is obtained by the summing up mean number of PDU sessions of the network slice relatively divided by the energy consumption of the network functions and dividing this by the number of network functions (SMFNbr). The unit of this KPI is PDUSes/J.

c)

EEns,NF,PDUSes=SMFSM.SessionNbrMean.SNSSAIECns,SMF SMFNbr

d) NetworkSlice

End of changes