3GPP TSG-SA5 Meeting #157	S5-245651

Hyderabad, India, 14 - 18 October 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.255 | CR | 0566 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 32.255 Adding use of charging characteristics for CHF Group |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | CHFSeg |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The handling of Charging Charateristics are unclear and is m issing CHF Group ID. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding relavent information the charging characteristics may be used for, including CHFGroup ID. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The use of charging characteristscs is unclear and could lead to interoperability issues.. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

# A.1	General

A subscriber may have Charging Characteristics assigned to the subscription and/or the subscribed DNNs. Default Charging Characteristics may also be pre-provisioned on the SMF.

During PDU session establishment, when the SMF retrieves subscription data from the UDM, if a subscribed Charging Characteristics for the requested DNN is identified, it may override the SMF pre-provisioned Charging Characteristics.

The Charging Characteristics parameter consists of a string of 16 bits designated as behaviours , freely defined by Operators.

These behaviours may be things like:

-	CHF addresses: to be used by the SMF, optionally with associated CHF instance ID(s), CHF set ID(s), CHF Group ID

-	CHF selection method: this indicates how the SMF is to select the CHF instance e.g., local configuration, NRF

-	Charging method: indicates what charging method to be used for every PCC rule of the PDU Session, i.e., "online", "offline", or “offline only”

-	Charging service: indicates if converged charging or offline only charging service is to be used

-	Metering method: indicates if duration, volume, or duration and volume of the service data flow traffic is to be metered

-	Service data flow handling: wherever the service data flow is allowed to start while the SMF is waiting for the response to the converged charging request

-	Triggers: the default triggers for the PDU session e.g., tariff time, time limit, volume limit, number of charging condition changes

One usage may be as a behaviour index associated to the PDU session in converged charging as described by the example in the Table A.1.

Table A.1: Example of Charging Characteristics behaviours for SMF

| Behaviour index | 5G data connectivity domain charging behaviours |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CHF address | CHF selection method | Charging method | Charging service | Metering method | Time Limit | Volume Limit | Cond. changes | Tariff time | … |
| 0 | CHF Set ID 1 | NRF | Online | Converged charging | Volume | 30 min | 2 MB | 2 | - | ... |
| 1 | Primary & secondary CHF addr. 1 & 2 | Local config. | Offline only | Converged charging | Volume | 15 min | 5 MB | 3 | 00:00 | … |
| 2 | CHF Group ID 1 | NRF | Offline | Converged charging | Volume | 30 min | 1 MB | 1 | - | ... |
| 3 | Primary & secondary CHF addr. 3 & 4 | Local config. | Offline only | Offline only charging | Volume | 15 min | 10 MB | 3 | 00:00 | ... |
| 4 | CHF Group ID 2 | NRF | Online | Converged charging | Volume | 15 min | 3 MB | 1 | - | … |
| … | … | … | … | … | … | … | … | … | … | … |

Any of the behaviours may be overridden by information provided by PCF or CHF.

End of changes