3GPP TSG-SA5 Meeting #157	S5-245817

Hyderabad, India, 14 - 18 October 2024

# Source:	Nokia

# Title:	Rel-19 pCR TR 28.879 Add use case, requirements, potential solution and evaluation for service API consumption by an external MnS consumer

# Document for:	Approval

Agenda Item:	6.19.21

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

[1] 3GPP TR 28.879, " Study on OAM for service management and exposure to external consumers".

# 3	Rationale

This pCR proposes to add a new use case to clause 5.1 of TR 28.879 [1] to cater for the service API consumption by an external MnS consumer at the MnS producer. It’s important to add the use case because the service API invocation request from the API invoker/external MnS consumer will mandatorily indicate the API invoker Id and the service API identification information. Accordingly, the MnF hosting the MnS producer needs to perform the mapping from the specified service API identification information to the published management service.

# 4	Detailed proposal

It is proposed that the following changes be made to clause 5.1 of TR 28.879 [1].

Begin Change

### 5.1.x	Use case #<X>: Service API consumption by an external MnS consumer

#### 5.1.X.1	Description

After the external MnS consumer has received the authorization information to consume the exposed management services as described in clause 5.1.4, the external MnS consumer can send a “service API invocation request” to the AEF (see clauses 8.16 and 8.17 of TS 23.222[5]). The service API invocation request includes two mandatory attributes, i.e., API invoker identification information and the Service API identification, as shown in Table 8.16.2.1-1 of TS 23.222[5]. The API invoker identification information is data that can be configured at the AEF (in this case, the MnF hosting the MnS producer providing the AEF functionality) by the CCF during the API invoker onboarding or later. However, the service API identification information is an attribute that is provided to the MnF hosting the MnS producer during the publishing of the management services to the CCF (see clause 5.1.2) as the apiId (see Table 5.1.2.3.1.2-1).

Accordingly, the MnF hosting the MnS producer while providing the AEF functionality should provide a mapping between the service API identification information (i.e., the apiId attribute) and the published management service to support the invocation of the specific service API by the external MnS consumer.

#### 5.1.X.2	Potential requirements

PREQ-FS_MExpo-Invoke-01: The 3GPP management system shall have the capability to map the service API identification information to the specific management service that was published to the CCF.

NOTE: The service API identification information is described by the apiId attribute of the CAPIF ServiceAPIDescription information element during the publishing of the management services to the CCF (see clause 5.1.2)

#### 5.1.X.3	Potential solutions

##### 5.1.X.3.i	Potential solution #<i>: The MnF acting as the AEF maintains a mapping table between the published management service and the service API identification information at the CCF

###### 5.1.X.3.i.1	Introduction

The potential solution proposes that the MnF acting as the AEF maintains a mapping table between the published management service and the service API identification information.

###### 5.1.X.3.i.2	Description

The MnF acting as the AEF maintains a mapping table between the apiId and the following MnSInfo IOC attributes (i.e., MnSLabel, MnSAddress and MnSVersion) of the published management service. The mapping table between the published management service and the service API identification information can be as indicated in Table 5.1.X.3.i.2-1. The solution assumes that the identified MnSInfo IOC attributes can provide a unique combination to aid the MnF to identify which published management service the external MnS consumer wants to consume.

Table 5.1.X.3.i.2-1: Mapping between the published management service and the serviceAPIDescription information element

| Published Management service(MnSInfo IOC attributes) | Corresponding serviceAPIDescription information element attribute |
| --- | --- |
| mnSLabel | apiId |
| mnSAddress |  |
| mnSVersion |  |

When the MnF acting as the AEF receives the service API invocation request with the apiId specified (as the value of the service API identification information), the MnF maps the apiId to the published management service using Table 5.1.X.3.i.2-1. Upon successful mapping, the specific published management service is consumed by the external MnS consumer.

##### 5.1.X.3.y	Potential solution #<y>: Adding a new attribute to MnSInfo IOC to represent the MnS identifier

###### 5.1.X.3.y.1	Introduction

The potential solution proposes to add a new attribute to the MnSInfo IOC (see clause 4.3.42 in TS 28.622 [3]) to uniquely identify the management service instance within the 3GPP management system.

###### 5.1.X.3.y.2	Description

The potential solution proposes to add a new attribute “mnsId” to MnSInfo IOC to uniquely identify a specific management service as indicated in Table 5.1.X.3.y.2-1.

Table 5.1.X.3.y.2-1: MnSInfo IOC attributes table with proposed new attribute (see clause 4.3.42.2 in TS 28.622 [3] for the original MnSInfo IOC attributes table)

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| mnsLabel | M | T | F | F | T |
| mnsType | M | T | F | F | T |
| mnsVersion | M | T | F | F | T |
| mnsAddress | M | T | F | F | T |
| mnsScope | M | T | F | F | T |
| mnsId | M | T | F | F | T |

The “mnsId” attribute has the following attribute definitions as indicated in Table 5.1.X.3.y.2-2.

Table 5.1.X.3.y.2-2: Attribute properties for the mnsId attribute

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| mnsId | A unique identifier for a management service allowedValues: NA | Type: String multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |

To support the mapping from the service API identification information to the published management service, the following occurs:

- During the process of publishing of the management service to the CCF (see clause 5.1.2), the MnF acting as the APF, publishes the “mnsId” attribute as a new attribute to the CAPIF ServiceAPIDescription information.

- When the MnF receives the response from the CCF about the result of the publishing of the management service request, the MnF stores the returned apiId.

- The MnF maintains the mapping table between the apiId and the mnsId as shown in Table 5.1.X.3.y.2-3.

Table 5.1.X.3.y.2-3: Mapping between the published management service and the serviceAPIDescription information element

| Published Management service (MnSInfo IOC attributes) | Corresponding serviceAPIDescription information element attribute |
| --- | --- |
| mnsId | apiId |

When the MnF acting as the AEF receives the service API invocation request with the apiId specified (as the value of the service API identification information), the MnF maps the apiId to the published management service using Table 5.1.X.3.y.2-3. Upon successful mapping, the specific published management service is consumed by the external MnS consumer.

#### 5.1.X.4	Evaluation of potential solutions

The potential solution in clause 5.1.X.3.i has the following benefits: it does not require the addition of a new attribute to the MnSInfo IOC and does not require the enhancement of the CAPIF serviceAPIDescription information to support the new MnSInfo IOC attribute. However, the potential solution requires that the MnF acting as the AEF keeps track of the mapping table given in Table 5.1.X.3.i.2-1, which requires to keep track of more than one MnSInfo IOC attribute.

The potential solution in clause 5.1.X.3.y has the benefit that the MnF acting as the AEF needs only to keep track of the mapping between the apiId (of the CAPIF serviceAPIDescription information element) and the new “mnsId” attribute of the MnSInfo IOC as shown in Table 5.1.X.3.y.2-3. The mapping is much more convenient. However, the potential solution requires enhancements to both the MnSInfo IOC and the CAPIF serviceAPIDescription information element.

End Change