3GPP TSG-SA5 Meeting #157 	S5-245829

Hyderabad, India, 14 - 18 October 2024

# Source:	Ericsson España S.A.

# Title:	pCR TR 28.879 pCR TR 28.879 Publishing UC – updates on solution and evaluation

# Document for:	Approval

Agenda Item:	6.19.21

# 1	Decision/action requested

The group is asked to discuss and approve the proposal.

# 2	References

[1] 3GPP TR 28.879: " Study on OAM for service management and exposure to external consumers".

[2] S5-243252: “Rel-19 pCR TR 28.879 Add use case for publishing management services to the CCF”

# 3	Rationale

This pCR aims to provide solutions along with evaluations for the updated UC description and requirements in S5-246xxx. The updated requirements are as follows:

PREQ-FS_MExpo-Pub-01: The 3GPP management system shall have the capability to configure, for a given MnS, which endpoints of this MnS will be made available for external consumption through CAPIF. These are referred as to reachable MnS endpoints.

PREQ-FS_MExpo-Pub-02: The 3GPP management system shall have the capability to map reachable MnS endpoints into the endpoints of one or more service APIs.

PREQ-FS_MExpo-Pub-03: The 3GPP management system shall have the capability to populate ServiceAPIDescription data type with information available in the MnSInfo IOC.

PREQ-FS_MExpo-Pub-04: The 3GPP management system shall have the capability to perform APF functionality.

# 4	Detailed proposal

It is proposed to make the following changes in the latest version of TR 28.879 [1].

Begin Change

#### 5.1.2.3	Potential solutions

##### 5.1.2.3.x	Potential solution #<x>: Mapping MnS endpoints into service API endpoints.

5.1.2.3.x.1	Introduction

When publishing on the CCF, MnS endpoints need to be mapped into the endpoints of one or more service API. This potential solution compares the URI structure for MnS API and service API, to help understand how this mapping looks like.

5.1.2.3.x.2	Description

The table below compares the URI structure for Service API and MnS API

Table 5.1.2.3.x.2-1: URI structure for Service APIs and MnS APIs

| Service API | MnS API (see 3GPP TS 32.158) |
| --- | --- |
| URI: = <apiRoot>/<apiName>/<apiVersion>/<APISpecificSuffixes> See NOTE 1 | URI: = {MnSRoot}/{MnSName}/{MnSVersion}/{URI-LDN-first-part}/{className} = {id} See NOTE 2 |
| NOTE 1: <apiRoot>:= https://<authority>/<API-prefix>, with <API-prefix> being optional. NOTE 2: {MnSRoot} = https://{URI-to-DN-prefix}/{root}, with {root} being optional. |  |

It is needed that the URI structure of MnS endpoints follow the same format as defined for service APIs. This means:

- {URI-to-DN-prefix} corresponds to the <authority> (host and optional TCP port). The host name is constructed from the DN prefix as detailed in clause 4.2.3, 3GPP TS 32.158 [30].

- {root} corresponds to <API-prefix>.

- {MnSName} corresponds to <apiName>. For example, when <MnSName> := ProvMnS, then the apiName in the service API URI shall be ProvMnS.

- {MnSVersion} corresponds to <apiVersion>.

- {URI-LDN-first-part}/{className} = {id} corresponds to <apiSpecificSuffixes>

##### 5.1.2.3.y	Potential solution #y: Populating ServiceAPIDescription with MnS related information

5.1.2.3.y.1	Introduction

This potential solution shows how ServiceAPIDescription data type can be populated using MnS related information available in the 3GPP management system. This information is captured in the MnSInfo IOC, which is name-contained by MnSRegistry IOC.

5.1.2.3.y.2	Description

Table 5.1.2.3.y.2-1 shows how the data type attributes (see Table 8.2.4.2.2-1 in 3GPP TS 29.222 [13]) can be populated using MnS related information

Table 5.1.2.3.1.y-1:
Populating ServiceAPIDescription data type using MnS related information 
(Source: Table 8.2.4.2.2-1 in 3GPP TS 29.222 [13])

| Attribute name | Data type | P | Cardinality | Description | Correspondence |
| --- | --- | --- | --- | --- | --- |
| apiName | string | M | 1 | API name, it is set as {apiName} part of the URI structure as defined in clause 5.2.4 of 3GPP TS 29.122 [33]. | Corresponds to the following MnSInfo IOC attribute: mnsType |
| apiId | string | O | 0..1 | API identifier assigned by the CAPIF core function to the published service API. Shall not be present in the HTTP POST request from the API publishing function to the CAPIF core function. Shall be present in the HTTP POST response from the CAPIF core function to the API publishing function and in the HTTP GET response from the CAPIF core function to the API invoker (discovery API). |  |
| aefProfiles | array(AefProfile) | C | 1..N | AEF profile information, which includes the exposed API details (e.g. protocol). For CAPIF-4/4e interface, API publishing function shall provide this attribute to the CAPIF core function in service API publishing. For CAPIF-1/1e interface, the CAPIF core function shall provide this attribute to the API Invoker during service API discovery (see note 2) | See Table 5.1.2.3.1.2-2 |
| description | string | O | 0..1 | Text description of the API |  |
| supportedFeatures | SupportedFeatures | O | 0..1 | The supported optional features of the CAPIF API (see note 1). | Not applicable in the context of SA5 MnS. |
| shareableInfo | ShareableInformation | O | 0..1 | Represents whether the service API and/or the service API category can be published to other CCFs. | Not applicable in the context of SA5 MnS. |
| serviceAPICategory | string | C | 0..1 | The service API category to which the service API belongs to. This attribute is only applicable for CAPIF-6/6e interface (see note 2). |  |
| ccfId | string | C | 0..1 | CAPIF core function identifier which can be contacted further for discovering the details of service API information. This attribute is only applicable for CAPIF-6/6e interface and shall be provided with serviceAPICategory (see note 2). | Not applicable in the context of SA5 MnS. |
| apiSuppFeats | SupportedFeatures | O | 0..1 | Provided by the consumer to indicate the features supported by the service API. | Not applicable in the context of SA5 MnS. |
| pubApiPath | PublishedApiPath | C | 0..1 | It contains the published API path within the same CAPIF provider domain. it shall be provided by the CCF when publishing the service API to other CCF via the CAPIF-6 reference point. | Not applicable in the context of SA5 MnS. |
| NOTE 1: For CAPIF_Publish_Service_API, the supported features attribute shall be provided in the HTTP POST request and in the response of successful resource creation. In addition, the supportedFeatures attribute may include one or more the supported features as defined in clause 8.2.6. NOTE 2: For CAPIF-6/6e interface, at least one of aefProfiles or serviceAPICategory and the corresponding ccfId shall be provided. |  |  |  |  |  |

Table 5.1.2.3.1.y-2: Populating AefProfile data type (Source: Table 8.2.4.2.4-1 in 3GPP TS 29.222 [13]) using MnS related information

| Attribute name | Data type | P | Cardinality | Description | Correpondence |
| --- | --- | --- | --- | --- | --- |
| aefId | string | M | 1 | AEF identifier. | Corresponds to the AEF identifier provided by the CCF upon MnS producer registration (use case 5.1.1). |
| versions | array(Version) | M | 1..N | API version. | See Table 5.1.2.3.1.2-3 |
| protocol | Protocol | O | 0..1 | Protocol used by the API. | Only “HTTP_1_1” and “HTTP_1_2” are applicable in the context of SA5 MnS. |
| dataFormat | DataFormat | O | 0..1 | Data format used by the API. | Only “JSON” value is applicable in the context of SA5 MnS. |
| securityMethods | array(SecurityMethod) | O | 1..N | Security methods supported by the AEF for all interfaces. Certain interfaces may have different security methods supported in the attribute interfaceDescriptions. | Only “OAUTH” value (i.e. TLS with OAuth token) is applicable in the context of SA5 MnS. |
| domainName | string | O | 0..1 | Domain to which API belongs to. (See note 1.) | Not applicable in the context of SA5 MnS. |
| interfaceDescriptions | array(InterfaceDescription) | O | 1..N | Interface details (See note 1.) | See Table 5.1.2.3.1.2-5 |
| aefLocation | AefLocation | O | 0..1 | The location information (e.g. civic address, GPS coordinates, data center ID) where the AEF providing the service API is located. |  |
| NOTE 1: Only one of the attributes "domainName" or "interfaceDescriptions" shall be included. NOTE 2: Notification or callback type of resource is not included. |  |  |  |  |  |

Table 5.1.2.3.1.y-3: Populating Version datatype
(Source: Table 8.2.4.2.5-1 in 3GPP TS 29.222 [13]) with MnSInfo IOC attributes

| Attribute name | Data type | P | Cardinality | Description | Correspondence |
| --- | --- | --- | --- | --- | --- |
| apiVersion | string | M | 1 | API major version in URI (e.g. v1). | Corresponds to the following MnSInfo IOC attribute: mnsVersion |
| expiry | DateTime | O | 0..1 | Expiry date and time of the AEF service. This represents the planned retirement date as specified in clause 4.3.1.5 of 3GPP TS 29.501 [34]. | Not applicable in the context of SA5 MnS. |
| resources | array(Resource) | O | 1..N | Resources supported by the API. It may include the custom operations with resource association. | See Table 5.1.2.3.1.2-4. Each resource corresponds to a service API endpoint. |
| custOperations | array(CustomOperation) | O | 1..N | Custom operations without resource association. |  |

Table 5.1.2.3.1.y-4: Populating Resource data type
(Source: Table 8.2.4.2.6-1 in 3GPP TS 29.222 [13]) with MnSInfo IOC attributes

| Attribute name | Data type | P | Cardinality | Description | Correspondences |
| --- | --- | --- | --- | --- | --- |
| resourceName | string | M | 1 | Resource name. | {className}. |
| commType | CommunicationType | M | 1 | Communication type used by the API resource (see note 1). | Only “REQUEST_RESPONSE” value is applicable for SA5 MnS of type Provisioning. |
| uri | string | M | 1 | Relative URI of the API resource, it is set as {apiSpecificSuffixes} part of the URI structure as defined in clause 5.2.4 of 3GPP TS 29.122 [33]. | Corresponds to the following URI component in a MnS endpoint: {URI-LDN-first-part}/{className} = {id}. |
| custOpName | string | O | 0..1 | it is set as {custOpName} part of the URI structure for the case where there is only a single custom operation associated with this resource as defined in clause 5.2.4 of 3GPP TS 29.122 [33] (see note 2). |  |
| custOperations | array(CustomOperation) | O | 1..N | List of custom operations associated to this resource (see note 2). | . |
| operations | array(Operation) | C | 1..N | Supported HTTP methods for the API resource. Only applicable when the protocol in AefProfile indicates HTTP. | HTTP methods specified under {URI-LDN-first-part}/{className} = {id}. |
| description | string | O | 0..1 | Text description of the API resource. |  |
| NOTE 1: The communication type refers to the semantics of the resource or custom operation and is independent of the HTTP methods that are supported (e.g. if a resource is used for subscriptions then its CommunicationType shall be SUBSCRIBE_NOTIFY even if it supports also the GET method for retrieving the subscriptions). NOTE 2: The attributes "custOpName" and "custOperations" are mutually exclusive. |  |  |  |  |  |

Table 5.1.2.3.1.y-5: Populating InterfaceDescription datatype
(Source: Table 8.2.4.2.3-1 in 3GPP TS 29.222 [13]) with MnSInfo IOC attributes

| Attribute name | Data type | P | Cardinality | Description | Equivalent MnS Info IOC attribute/comments |
| --- | --- | --- | --- | --- | --- |
| ipv4Addr | Ipv4Addr | C | 0..1 | String identifying an IPv4 address (see note 1). | Not applicable in the context of SA5 MnS |
| ipv6Addr | Ipv6Addr | C | 0..1 | String identifying an IPv6 address (see note 1). | Not applicable in the context of SA5 MnS |
| fqdn | Fqdn | C | 0..1 | String containing a Fully Qualified Domain Name (see note 1). | Corresponds to the following URI component in a MnS: {URI-DN-prefix} This FQDN can be constructed from the DN prefix as detailed in clause 4.2.3, 3GPP TS 32.158 [30]. |
| port | Port | O | 0..1 | Port. |  |
| apiPrefix | string | O | 0..1 | A string representing an optional deployment-specific string (API prefix) in the form of a sequence of path segments that starts with a "/" character. | Corresponds to the following URI component in a MnS: “/” + {root} |
| securityMethods | array(SecurityMethod) | M | 1..N | Security methods supported by the interface. It takes precedence over the security methods provided in AefProfile, for this specific interface. | Only “OAUTH” value (i.e. TLS with OAuth token) is applicable in the context of SA5 MnS. |
| NOTE 1: Exactly one of the attributes "ipv4Addr", "ipv6Addr" and "fqdn" shall be included. NOTE 2: When the contents of this data type are used to construct the apiRoot of an API, they are used as described in clause 4.4.1 of 3GPP TS 29.501 [34]. |  |  |  |  |  |

The mapping described in these tables allow describing how to ServiceAPIDescription data type can be defined with MnS related information available in the 3GPP management system. To publish a service API on the CCF, the APF sends an HTTP POST operation carrying this data type over the CAPIF-4 interface (see clause 8.2 in 3GPP TS 29.222 [13]).

##### 5.1.2.3.z	Potential solution #<z>: MnS producer implements APF

5.1.2.3.z.1	Introduction

This potential solution proposes that APF is implemented by the MnS producer.

5.1.2.3.z.2	Description

The MnS producer implements the consumer of the CAPIF_Publish_Service_API service (see TS 29.222 [13], clause 8.2), so that the MnS producer can:

- publish/unpublish MnS(s) on the CCF. To that end, the MnS producer invokes “Publish_Service_API” and “Unpublish_Service_API” operations.

- Update information on published MnS(s) on the CCF. To that end, the MnS producer invokes “Update_Service_API” operation.

- Retrieve information on published MnS(s) from the CCF. To that end, the MnS producer invokes “Get_Service_API” operation.

##### 5.1.2.3.z	Potential solution #<a>: Agent implements the APF

5.1.2.3.z.1	Introduction

This potential solution proposes that APF is implemented by an agent.

5.1.2.3.z.2	Description

This agent communicates with:

- CCF, over CAPIF-4 interface. The agent implements the consumer of the CAPIF_Publish_Service_API service

- One or more MnS producers. The agent implements the MnS consumer(s).

#### 5.1.2.4	Evaluation of potential solutions

Editor's Note:	This clause provides the evaluation of potential solutions.

##### 5.1.2.4.x	Evaluation of potential solution #x

The potential solution #x shows that the URI components of MnS endpoint find a correspondence with the URI components with the service API endpoints. The solution fulfils PREQ-FS_MExpo-Pub-02.

The potential solution identifies that {MnSVersion} in MnS corresponds to <apiVersion> in service API. However, it is not worthing that <apiVersion> represents only the major release, e.g. “v1”, while the <MnSVersion> follow “v<major><minor><patch>” format, e.g. “v1501”. It shall be clarified how the versioning format in MnS matches with the major release format in service API.

##### 5.1.2.4.y	Evaluation of potential solution #y

The potential solution #y shows that it is feasible to build ServiceAPIDescription data type using MnS related information available in the 3GPP management system, including the MnSInfo IOC. The solution fulfils PREQ- PREQ-FS_MExpo-Pub-03.

As part of the evaluation of the solution, the following can be noted:

- Not all the MnSInfo IOC attributes are relevant to the ServiceAPIDescription data type. In particular, the following two attributes are not relevant: mnsLabel and mnsScope.

- The “description” attribute in Table 5.1.2.3.1.y-1 is specified (and maintained) by the API provider domain.

- The “ServiceAPICategory” attribute in Table 5.1.2.3.1.y-1 is specified (and maintained) by the CCF. The current usage is limited to CAPIF-6/6e interface, though SA6 plans to remove this limitation during Rel-19.

The ServiceAPIDescription data type will be used by external MnS consumers to discover and invoke the endpoints of published service API(s). For discovery purposes, the external MnS consumer can read the following attributes:

- “resources” (see  Table 5.1.2.3.1.y-4). This attribute provides information on the scope of a service API, i.e. constituent service API endpoints. Unlike SA5 solution, which is based on identifying MOIs accessible through the MnS (see mnsScope in the MnSInfo IOC), the solution here is based on the resource (along with supported operations) accessible through each endpoint. This solution requires that the external MnS consumer understand the schema representing the resources, and therefore IOCs in the NRM tree.

- “ServiceAPICategory” (see Table 5.1.2.3.1.y-1). This attribute provides additional information for a specific API, e.g. optional MnSInfo IOC attributes based on the MnSType. When the usage limitation to CAPIF-6/6e information is removed, this attribute can be for example used to accommodate MnsInfo.mnsCapability attribute. If present, this attribute complements “resource” attribute.

##### 5.1.2.4.z	Evaluation of potential solutions #z and #a

These potential solutions fulfil PREQ- PREQ-FS_MExpo-Pub-04. The pros and cons of these (competing) solutions are detailed below.

Table 5.1.2.4.z-1: Pros and cons of solutions #z and #a

| Solution | Pros | Cons |
| --- | --- | --- |
| #z | - Less integration efforts on the operator side: the MnS producer has built-in capabilities to communicate with the CCF, so the operator does not need to worry about integrating the MnS producer with other entities implementing APF; it only needs to rely on the completeness of the MnS producer solution provided by the vendor. - More innovation protection on MnS producer implementor side: in case the MnS producer does not perform APF, the MnS producer would need to interact with another entity performing APF. This interaction means disclosing low-level information to this entity, which would put at risk the innovation capacity on implementor side in case this entity is a 3rd party tool. With the APF implementation on MnS producer, this risk is mitigated. | - More complexity on MnS producer implementor side: the implementor does not only need to develop the MnS producer capabilities associated to MnS logic, but also the APF. - Risk of having multiple APF instances (one for each MnS producer), leading to unneeded duplicated functionality in 3GPP management system and multiple contact points for CCF. |
| #a | - Simplified backend implementation on CCF side, since this agent acts as single contact point for CAPIF-4 interface. - Less complexity on MnS producer implementor side: the implementor shall only focus on developing the MnS producer capabilities associated to MnS logic, as done traditionally. | - Increasing complexity on agent implementor side, as the number of MnS producer increases; the agent shall implement MnS consumer for each of them |

End Change