3GPP TSG-SA5 Meeting #157	S5-246034

Hyderabad, India, 14 - 18 October 2024										(revision of S5-245618)

# Source:	Nokia

# Title:	Rel-19 pCR TR 28.880 Potential solution for use case Handling of power shortages

# Document for:	Approval

Agenda Item:	6.19.20

# 1	Decision/action requested

The group is requested to discuss and approve the pCR below.

# 2	References

[1]          3GPP TR 28.880: "Study on energy efficiency and energy saving aspects of 5G networks and services".

# 3	Rationale

This pCR proposes a potential solution for Use case #11: Handling of power shortages, of TR 28.880 [1].

# 4	Detailed proposal

The following changes are proposed for TR 28.880[1].

1st Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[3]	ETSI GR NFV-IFA 029 (V3.3.1): "Network Functions Virtualisation (NFV) Release 3; Architecture; Report on the Enhancements of the NFV architecture towards "Cloud-native" and "PaaS"".

[4]	ETSI GS NFV-IFA 040 (V4.3.1): "Network Functions Virtualisation (NFV) Release 4; Management and Orchestration; Requirements for service interfaces and object model for OS container management and orchestration specification".

[5]	ETSI GS NFV-IFA 027 (V4.3.1): "Network Functions Virtualisation (NFV) Release 4; Management and Orchestration; Performance Measurements Specification".

[6]	ETSI GS NFV-IFA 027 (V5.1.1): "Network Functions Virtualisation (NFV) Release 5; Management and Orchestration; Performance Measurements Specification".

[7]	3GPP TS 22.261: "Service requirements for the 5G system".

[8]	ETSI GS OEU 020: "Operational energy Efficiency for Users (OEU); Carbon equivalent Intensity measurement; Operational infrastructures; Global KPIs; Global KPIs for ICT Sites".

[9]	ETSI EN 303 472: "Environmental Engineering (EE); Energy Efficiency measurement methodology and metrics for RAN equipment".

[10]	ISO/IEC 30134-3:2016: "Information technology -- Data centres -- Key performance indicators -- Part 3: Renewable energy factor (REF)".

[11]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[12]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[13]	3GPP TS 28.313: "Management and orchestration; Self-Organizing Networks (SON) for 5G networks".

[14]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[15]	Energy Benchmark 2023: Telco Insights and Industry Health Check.

[16]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

Editor's Note: reference [6] is not published yet, will be published soon, the latest draft of DGS/NFV-IFA027 is available in the following location: https://docbox.etsi.org/ISG/NFV/Open/Drafts/IFA027ed451

[17]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[18]	3GPP TS 22.104: "Service requirements for cyber-physical control applications in vertical domains".

[Y] 	3GPP TS 28.104: "Management and orchestration; Management Data Analytics (MDA)"

Next Change

## 5.11	Use case #11: Handling of power shortages

### 5.11.1	Description

In this use case, the 3GPP network management system becomes aware of current or upcoming power shortages. The network operator in turn takes actions (e.g. at service, network and/or resource level) to ensure the delivery of communication services such as emergency calls, first responder communication, voice calls etc. while reducing energy use.

### 5.11.2	Potential requirements

REQ-Energy_Shortage_Handling-01: The 3GPP management system, in case it is made aware of current energy shortages or upcoming energy shortages, should be able to take appropriate measures to conserve energy use in the network while ensuring delivery of important communication services.

NOTE:	Examples for communication services that need to be preserved include emergency calls, first responder communication, voice calls etc.

### 5.11.3	Potential solutions

#### 5.11.3.y	Potential solution #Y: Service adjustment based on available energy information

##### 5.11.3.y.1	Introduction

In this potential solution, it is proposed to enhance the MDA assisted Energy Saving use case (see clause 7.2.4 of TS 28.104 [Y]) to consider the power shortage information and re-use the existing inputs such as traffic trends and provides service adjustment recommendations for different periods of time.

##### 5.11.3.y.2	Description

MDA assisted Energy Saving use case is defined in clause 7.2.4 of TS 28.104 [Y], and the data definitions for the same are defined in clause 8.4.4 of TS 28.104 [Y]. This potential solution proposes enhancement to this MDA use case.

It is proposed to enhance the enabling data for this MDA use case with information on power shortage and outage information and enhance the analytics output from this MDA use case with service adjustment recommendations for different periods of time.

This potential solution proposes that new information element(s) be defined to enable the operator to associate the power outage and outage information with the managed functions that are managed by 3GPP management system.

The enabling data for the MDA use case for power shortage and outage information needs to be associated with the network functions. The analytics output from the MDA use case needs to be enhanced to include the recommendations of network service operability levels for different time periods, based on energy availability.

### 5.11.4	Evaluation of potential solutions

The potential solution #Y proposes to extend the MDA assisted Energy Saving use case to utilize power shortage and outage information, that is associated with the network functions, along with network analytics and to provide recommendations on the network service operability levels for different time periods.

This solution is feasible by identifying the that power shortage information needs to be provided to the 3GPP management system and by using MDA for providing the recommendations.

NOTE: The definition of the information elements (e.g., an IOC or attribute(s) representing the information) related to power shortage and outage information and how this is associated with the network functions (represented by ManagedFunction) will be part of the normative phase.

End of change