3GPP TSG-SA5 Meeting #157 	S5-246064

Hyderabad, India, 14 - 18 October 2024                                         revision of S5-245392

# Source:	Ericsson

# Title:	pCR TR 28.871 Alarm definition - filtering

# Document for:	Approval

Agenda Item:	6.19.8

# 1	Decision/action requested

Approve the proposal.

# 2	References

[1]	3GPP TR 28.871 Study on Service Based Management Architecture enhancement phase 3 (Release 19) v1.0.0

[2]	S5-238120	Discussion paper on Alarm definition

[3] 	3GPP TS 28.111 “Management and orchestration; Fault Management (FM)”

# 3	Rationale

The Study on Service Based Management Architecture enhancement phase 3 [1] includes requirement

| REQ-MS- FMAL-3 | It should be possible to filter alarm related notifications based on the alarming-condition. |
| --- | --- |

This document specifies a potential solution for this requirement.

# 4	Detailed proposal

First change

#### 5.10.3.2	Potential solution #part2: Filtering on alarming condition

TS 28.111[3] defines that the attributes alarmType, probableCause and specificProblem (if present) identify the alarming condition.

To allow filtering on alarming conditions all notifications related to individual alarms shall carry the parameters alarmType, probableCause and specificProblem.

SpecificProblem shall be added to the input parameters of notifyClearedAlarm, notifyChangedAlarm, notifyAckStateChanged, notifyComments.

All 3 parameters shall be added to notifyCorrelatedNotificationChanged.

### 5.10.4	Evaluation of potential solutions

The potential solution described in clause 5.10.3.2 fulfils the requirement REQ-MS- FMAL-3. It is recommended for normative work.

End of change