3GPP TSG-SA5 Meeting #157	S5-246123

Hyderabad, India, 14 - 18 October 2024

# Source:	Nokia, Ericsson

# Title:	Rel-19 pCR TR 28.869 Resolve editor's note in Annex D by updating the content and the related figure

# Document for:	Approval

Agenda Item:	6.19.6

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

[1] 3GPP TR 28.869, " Study on cloud aspects for management and orchestration."

# 3	Rationale

This pCR resolves the editor’s note in Annex D of TR 28.869 [1] by updating the content and Figure D-1 to show the interactions between the 3GPP management system and the orchestration and management entity for the creation of the cloud-native NF. In addition, this pCR replaces the term cloud-native NF with NF deployment as agreed.

# 4	Detailed proposal

It is proposed that the following changes be made to Annex D of TR 28.869 [1].

Begin Change

Annex <D>: Example scenario using interactions with orchestration and management entity for the creation of NF deployment instances

Figure D-1 depicts an example scenario for the interaction between the 3GPP management system and the orchestration and management entity for the creation of a  NF deployment instance.

An authorized NF provisioning MnS Consumer requests the creation of a 3GPP NF  by creating the ManagedFunction IOC on  the NF provisioning MnS producer specifying the network function related requirements. If the 3GPP NF  contains containerized or virtualized parts, the NF provisioning MnS producer derives the associated requirements for the NF deployment based on the network function related requirements.

Note:	For the network function related requirements, see information model definition for NR NRM in clause 4 and information model definition for 5GC NRM in clause 5 in TS 28.541[17].

Figure D-1: Example scenario for interaction with orchestration and management entity for  3GPP NF creation procedure

The NF provisioning MnS producer invokes interactions with an orchestration and management entity for the creation of the NF deployment instance. Following the completion of the required interactions with the orchestration and management entity, the NF provisioning MnS producer creates the MOI for the 3GPP NF  instance and configures the new MOI with corresponding information. The NF provisioning MnS producer sends the createMOI response to the NF provisioning MnS Consumer with the DN of the MOI of the 3GPP NF instance.

Note: If the orchestration and management entity is ETSI NFV MANO, the interactions over the deployment management reference point are as specified in clause 7.10 of 28.531[7].

End of Change