3GPP TSG-SA5 Meeting #157	S5-245224

Hyderabad, India, 14 - 18 October 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.405 | CR | 0038 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.405 Correct the procedure description of management based activation in NR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eQoE |  |  |  |  |  | Date: |  |  | 2024-09-25 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - There is a typo in step 13 of activation of measurement collection job and reporting of collected information in NR, which needs to be corrected. “MCE associated to the qoECollectionEntityAddress” should be changed to “MCE associated to the qoEReference”. - Wrong clause number |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correcet a typo. Correct “MCE associated to the qoECollectionEntityAddress” to “MCE associated to the qoEReference”. - Correct wrong clause number |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Typo may lead to misunderstanding. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4, 4.5.1, 4.5.3, 4.5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | No impact on stage 3. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

4.5.1	Activation of measurement collection job and reporting of collected information in NR

Management Based Activation enables collection of application layer measurements from the UEs in the specified area for specified end user service type.

The parameters for the network request session are sent from the MnS Consumer via the management system to the gNBs that host the cells that are included in the QoE Measurement Collection job request in the Create MOI QMC operation [15]. A QMC Job is activated by creating a QMCJob object instance in the MnS producer.

Figure 4.5.1-1 and the text below describe the activation of QoE measurement collection.

Figure 4.5.1-1: QMC activation example for Management Based Activation and Reporting in NR

1)	A QoE Measurement Collection Job begins when the MnS Consumer sends createMOI request for QMCJob to the MnS Producer serving the impacted gNB(s), and includes the parameters: serviceType, areaScope, qoECollectionEntityAddress, qoEReference, mDTAlignmentInformation, availableRANqoEMetrics and qMCConfigFile. Step 2 and step 3 are asynchronous in time with respect to Step 1; they may occur before or after step 1.

NOTE: The RAN visible QoE measurements are supported for the DASH streaming and VR services.

2)	Application level measurement configuration. The Application in the UE shall send AT command +CAPPLEVMCNR [5] containing the parameter <n> set to 1 to the Access Stratum. This enables the the Access Stratum to present an unsolicited result code to the Application at a later time. Step 2 and step 3 are asynchronous in time with respect to Step 1; they may occur before or after step 1.

3)	UE registration procedure. The UE shall register QMC capability with the gNB by setting qoe-Streaming-MeasReport to “supported” in UE-NR-Capability [11]. Step 2 and step 3 are asynchronous in time with respect to Step 1; they may occur before or after step 1.

4)	The gNB starts a network request session, with the QoE Reference [3] given in createMOI for QMCJob [3]. For the duration of the network request session, the gNB(s) checks for connections where the UE has qoe-Streaming-MeasReport set to “supported” [11] in step 2.

5)	If a UE has the wanted capability, the gNB stores the associated qoECollectionEntityAddress and starts a UE request session by sending RRCReconfiguration to the UE including serviceType, measConfigAppLayerId (corresponding to the value of qoEReference from step 4) and measConfigAppLayerContainer.

If QoE measurement configuration pertains to MBS communication service, the gNB translates the qoECollectionEntityAddress into qoECollectionEntityIdentity and includes qoECollectionEntityIdentity in the RRCReconfiguration.

NOTE: The IE measConfigAppLayerId indicates the identity of the application layer measurement configuration, see [11].

6)	The access stratum in the UE sends an unsolicited response to the Application including app-meas_service_type, meas_config_app_layer_id and app-meas_config_file. These IEs map directly to the IEs in step 5. The unsolicited response is for the AT command +CAPPLEVMCNR [5] which is sent from Application in step 2.

7)	The QMC Job remains inactive until the Application for the specified serviceType initiates service.

8)	When the Application begins service, it sends the AT command +CAPPLEVMRNR [5], including meas_config_app_layer_id and qoe_measurement_status to indicate a recording session has started, to the access stratum.

9)	The UE sends the message MeasurementReportAppLayer including measConfigAppLayerId,  appLayerSessionStatus, and qoECollectionEntityIdentity to the gNB to indicate a recording session has started.

10)	While active, the Application collects measurement reports. When the QMC is completed or at the end of period for periodic report, the recorded information is formatted in a QMC report, see [6] for DASH, [7] for MTSI or [13] for VR.

11)	When a formatted measurement report is available, the Application sends the AT command +CAPPLEVMRNR [5] including meas_config_app_layer_id, qoe_measurement_status and app-meas_report to the Access Stratum. If the QMC Job has ended, qoe_measurement_status is set to indicate the Job has ended.

12)	When the AT command +CAPPLEVMRNR is received from the Application, the Access Stratum sends the message MeasurementReportAppLayer including measConfigAppLayerId, appLayerSessionStatus, measReportAppLayerContainer, and qoECollectionEntityIdentity to the gNB.

13)	The gNB translates the qoECollectionEntityIdentity into qoECollectionEntityAddress if it is needed, and sends the QMCRecord to the MCE associated to the qoEReference. The report contains the QoE Reference from step 4 and measReportAppLayerContainer from step 12.

End of First change

Start of Second change

4.5.2	Deactivation of measurement collection job in NR

4.5.2.1	Forced deactivation

When the operator technician or the management application wants to deactivate a measurement collection job, the management system sends the deactivateQMCJob operation [3] to the gNB. The gNB sets the network request session to ended, but does not delete the UE request session id and the Collection Entity Address parameters [3], as the UE still may send reports which shall be sent to the collection centre. For UE request sessions which have reported that a recording session is started, the gNB sends the RRCReconfiguration message [11] to relevant UEs. The RRCReconfiguration message is including measConfigAppLayerId set to discard application layer measurement report information in otherConfig [11]. The Access Stratum sends +CAPPLEVMC AT command [5] to the application with the discard request. The application stops the recording session and stops recording of the requested information. The QoE Reference Id and the QoE Collection Entity Address parameters [3] in the gNB are deleted when the UE request session is ended.

4.5.2.2	Deactivation of recording session

The recording session continues to be active until the session for the Application is ended.

4.5.3	Temporary stop and restart of QoE information reporting during RAN overload in NR

In case of overload in RAN, the gNB may temporarily stop the reporting from the UE by sending the RRCReconfiguration message [11] to relevant UEs. The RRCReconfiguration message [11] includes pauseReporting set to TRUE in appLayerMeasConfig to temporarily stop application layer measurement reporting. The access stratum continues recording further information until the data storage capacity for the reporting is fully used or the UE request session ends. Then the recorded data is kept until it is reported to the gNB or when the UE request session is ended.

When the overload situation in RAN is ended the gNB restart the reporting from the UE by sending the RRCReconfiguration message [11] to relevant UEs. The RRCReconfiguration message is including pauseReporting set to FALSE in appLayerMeasConfig  to restart application layer measurement reporting.

The UE application level is not directly affected by the RAN overload.

End of Second change

Start of Third change

4.4	Signalling based activation in LTE

4.4.1	Activation of measurement collection for a UE in LTE

4.4.1.0	General

Activation of measurement collection for a UE can be done before UE attachment or after UE is attached.

4.4.1.1	Activation of QoE measurement task before UE attaches to the network

When HSS activates the QoE measurement collection for a job, to the MME the following configuration parameters shall be included in the message: serviceType, areaScope, qoECollectionEntityAddress, pLMNTarget, qoETarget, qoEReference and QMC configuration file.

At UE attachment the configuration parameters for the QoE measurement collection for a job is propagated to the UE.

The MME receives and stores QoE measurement collection job as part of Update Location Answer (or Insert Subscriber Information). Then the same procedure for activation of measurement collection job is applied as after UE is attached, described in see 4.4.2.2.

4.4.1.2	Activation of QoE measurement task after UE attachement to the network

The QoE measurement collection is sent to the UE from the OAM via CN. The messages from EMS propagated to HSS, MME and eNodeB are the same as previous section. QoE measurement collection activation request is propagated to UE finally.

When MME can send the QoE measurement collection to the UE through eNodeB, the following configuration parameters shall be included in the message: serviceType, areaScope, qoECollectionEntityAddress, pLMNTarget, qoETarget, qoEReference and QMC configuration file. QoE measurement collection activation request is propagated to UE.

1)	The EMS sends activateAreaQMCJob to HSS that controls the impacted eNB(s), and includes the parameters: serviceType, areaScope, qoECollectionEntityAddress, pLMNTarget, qoETarget, qoEReference and QMC configuration file.

2)	The HSS inserts subscriber related data and forwards it to the MME.

3)	The MME forwards activateAreaQMCJob to impacted eNB(s), and includes the parameters: serviceType, areaScope, qoECollectionEntityAddress, pLMNTarget, qoETarget, qoEReference and QMC configuration file.

4)	The eNB checks if the UE capability that matches the criteria for serviceType in the activateAreaQMCJob.

5)	If the UE has the wanted UE capability, the eNB starts a UE request session and stores the associated QoECollectionEntityAddress, sends the message RRCConnectionReconfiguration to the UE, and includes the following: serviceType, qoEReference and QMC configuration file.

6)	The access stratum in the UE sends the AT command +CAPPLEVMC to application level and includes the following: serviceType, qoEReference and QMC configuration file.

7)	When the application in the serviceType starts, the QMC is initiated.

8)	The application layer sends the AT command +CAPPLEVMR including a recording session indication that indicates that a session is started to the access stratum.

9)	The UE sends the message MeasReportAppLayer including the recording session indication to the eNB.

10)	The eNB sends a notification including the recording session indication to the NM.

11)	When the QMC is completed, the recorded information is collected in a QMC report [6], [7], including qoEReference and recordingSessionId. The qoEReference, Client Id [6] and [7] in the reporting container (that represent the UE request session), and recordingSessionId are needed in the QMC collection entity for post processing purposes.

12)	The application layer sends the AT command +CAPPLEVMR including qoEReference and the QMC report to the access stratum.

13)		The UE sends the message MeasReportAppLayer including qoEReference and the QMC report to the eNB.

14)		The eNB sends the QMC report to the MCE associated to the qoEReference.

4.4.2	Handling of measurement collection at handover in LTE

See clause 4.2.2.

4.4.3	Deactivation of measurement collection for a UE in LTE

4.4.3.1	Pre-set time has elapsed in LTE

When the pre-set time is elapsed, the MME and the eNB sets the network request session to ended, but do not delete the UE request session id and the Collection Entity Address parameters [3], due to that reports still can come from the UE that has to be transferred to the collection centre. The UE request session id and the Collection Entity Address parameters [3] are deleted when the session is ended.

4.4.3.2	Forced deactivation in LTE

When the operator technician or the management application wants to deactivate a measurement collection job, the management system sends the deactivateQMCJob operation [3] to the MME that propgates to the eNB . The eNB sets the network request session to ended, but does not delete the UE request session id and the Collection Entity Address parameters [3], as the UE still may send reports which shall be send to the collection centre. For UE request sessions which have reported that a recording session is started, the eNB sends the RRCConnectionReconfiguration message [8] to relevant UEs. The RRCConnectionReconfiguration message is including measConfigAppLayer set to discard application layer measurement report information in otherConfig [8]. The Access stratum sends +CAPPLEVMC AT command [5] to the application with the discard request. The application stops the recording session and stops recording of the requested information. The UE request session id and the Collection Entity Address parameters [3] in the eNB are deleted when the UE request session is ended.

4.4.3.3	Deactivation of recording session in LTE

See clause 4.2.3.2.

End of Third change