3GPP TSG-SA5 Meeting #157	S5-245281

# Hyderabad, India, 14th Oct 2024 - 18th Oct 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0605 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.552 Add PMF measurement of network-initiated PLR for ATSSS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom Corporation Ltd., ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph4 |  |  |  |  |  | Date: |  |  | 2024-09-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The purpose of the network-initiated PLR measurement procedure is to enable the UPF to measure the PLR of DL traffic to the UE over an access of an MA PDU session. The number of PLR measurement requests and successful responses will reflect the availability of PLR values which are used in ATSSS rules. Therefore, it is useful to know the performance of PMF PLR measurement procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add PMF related measurements： - Number of requested/successful network-initiated PLR measurement for MA PDU session |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cannot monitor the performance of PLR measurement for the MA PDU session. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 5.4.13.x(new), 5.4.13.y(new), A.133 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], TS 23.501 [4] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and TS 23.501 [4].

CHO	Conditional Handover

CLI	Cross Link Interference

DAPS	Dual Active Protocol Stack

GP	Guard Period

HO	Handover

ITI	Interrupted Transmission Indication

kbit	kilobit (1000 bits)

LHO	Legacy Handover

LTM	L1/L2 Triggered Mobility

MA PDU	Multi-Access PDU

MN	Master Node.

MPQUIC	Multi-Path QUIC

MPTCP	Multi-Path TCP Protocol

NG-RAN	Next Generation Radio Access Network

RNA	RAN-based Notification Area

PI	Performance Indicator

PMF	Performance Measurement Function

SA PDU	Single-Access PDU

SDT	Small Data Transmission

SN	Secondary Node.

SRS	Sounding Reference Signal

TEID	Tunnel Endpoint Identifier

PLR                     Packet Loss Rate

2nd change

5.4	Performance measurements for UPF

5.4.13	PMF related measurements

##### 5.4.13.x	Number of PLR measurement request for MA PDU session (network-initiated)

a)	This measurement provides the number of network-initiated PLR measurement attempt for MA PDU session.

b)	CC.

c)	On transmission of PMFP PLR COUNT REQUEST message from the UPF to UE (see clause 5.4.7.1 of TS 24.193 [67]).

d)	A single integer value.

e)	GTP. PLRMeasUpfInitReq

f)	EP_N3 (contained by UPFFunction); 
EP_N9 (contained by UPFFunction).

g)	Valid for packet switched traffic.

h)	5GS.

##### 5.4.13.y	Number of successful PLR measurement for MA PDU session (network-initiated)

a)	This measurement provides the number of successful network-initiated PLR measurement for MA PDU session.

b)	CC.

c)	On receipt of any PMFP PLR REPORT RESPONSE message over the same QoS flow with the PMFP PLR COUNT REQUEST message by the UPF from UE. (see clause 5.4.7.1 of TS 24.193 [67]).

d)	A single integer value. Multiple response messages (in case of the network-initiated PLR measurement procedure consists of more than one network-initiated PLR report procedure) are counted as one.

e)	GTP. PLRMeasUpfInitSucc

f)	EP_N3 (contained by UPFFunction); 
EP_N9 (contained by UPFFunction).

g)	Valid for packet switched traffic.

h)	5GS.

3rd change

A.133	Use case of measurements related to ATSSS rules

The ATSSS feature enables a multi-access PDU Connectivity Service, which can exchange PDUs between the UE and a data network by simultaneously using one 3GPP access network and one non-3GPP access network and two independent N3/N9 tunnels between the PSA and RAN/AN.

ATSSS rules/N4 rules are applied by UE/UPF for deciding how to distribute the downlink/uplink traffic across the two access networks. The PMF protocol enables messages to be exchanged between the PMF in the UE and the PMF in the UPF, e.g. RTT measurements for ATSSS-LL control, reporting of access availability/unavailability, PLR measurements for ATSSS-LL, etc.

The RTT and PLR measurements are defined to support several steering modes such as "Smallest Delay", "Load Balancing", "Priority-based" and "Redundant". The number of RTT/PLR measurements requests and responses will reflect whether the RTT/PLR values used in the ATSSS rules are available. These measurements provide reference to help operators to identify whether the steering mode in the ATSSS rules is implemented as expected.

The Access Availability report can help distribute the QoS flow over different accesses in several steering modes. The number of access state measurement will reflect if an access of MA PDU session is available.

End of change