3GPP TSG-SA5 Meeting #157	S5-245291

Hyderabad, India, 14 - 18 October 2024

# Source:	Huawei

# Title:	Update potential solution of EE KPIs evaluated from network availability performance dimension

# Document for:	Approval

Agenda Item:	6.19.20

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1]		3GPP TR 28.880 v1.0.1: Study on energy efficiency and energy saving aspects of 5G networks and services

# 3	Rationale

This contribution proposes to update potential solution of EE KPIs evaluated from network availability performance dimension in TR 28.880 [1].

# 4	Detailed proposal

This document proposes the following changes in TR 28.880 [1].

1st Change

#### 5.6.3.2	Potential solution #2: <EE KPIs evaluated for a network from availability performance dimension>

##### 5.6.3.2.1	Introduction

As defined in TS 28.310 [12], EE is the ratio between performance and energy consumption. In this potential solution, new EE KPIs are considered in terms of availability performance aspect for a network.

##### 5.6.3.2.2	Description

Availability KPIs exist as follows

- Cell Availability

- Radio access network availability

The proposed performance dimension of new EE KPIs, i.e. the numerator part of the formula of new EE KPIs, are evaluated from availability performance dimension for a network as reflected by some existing KPIs in TS 28.554 [2], see Table 5.6.3.1.2-1.

Table 5.6.3.2.2-1: Network availability performance dimension from which new EE KPIs are evaluated

| Performance dimension to evaluate EE KPIs | Performance part of new EE KPIs | Reference to the clause number in TS 28.554 [2] | Note |
| --- | --- | --- | --- |
| Availability | Cell Availability KPI | 6.10.1.1.1 | This availability KPI provides “average availability time duration per cell” in a gNB-CU. |
|  | Radio access network availability KPI | 6.10.1.1.2 | This availability KPI provides “average availability time duration per cell” of an entire RAN sub-network. |

The new EE KPIs, which are evaluated as the ratio between performance (see availability KPIs captured in Table 5.6.3.2.2-1) and energy consumption, are captured in Table 5.6.3.2.2-2.

Table 5.6.3.2.2-2: New EE KPIs evaluated from network availability performance dimension

| Performance dimension to evaluate EE KPIs |  | New EE KPIs | Note |
| --- | --- | --- | --- |
| Network availability | Availability as the network availability performance indicator | Cell AvailabilityECcell | This new EE KPI provides the energy efficiency of a cell. This new EE KPI is obtained by Cell Availability KPI divided by the energy consumption of the cell over the same observation period. |
|  |  | Radio access network availabilityECNG-RAN | This new EE KPI provides the energy efficiency of an entire RAN sub-network. This new EE KPI is obtained by Radio access network availability KPI divided by the energy consumption of the entire RAN sub-network over the same observation period. |

End of change