3GPP TSG SA5 Meeting #157	S5-245376

Hyderabad, India, 14 - 18 October 2024

Source:	SA3

Title:	LS on Masking of cryptographic keys in UE trace file from 3GPP core network entities

Document For:	Information

Agenda Item:	6.1

3GPP TSG-SA3 Meeting #117	S3-243696

Maastricht, Netherlands  19 - 23 August 2024

Title:	LS on Masking of cryptographic keys in UE trace file from 3GPP core network entities

Source:	SA3

To:	SA5

CC:	-

Contact person:	Niraj Rathod

Niraj.rathod@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# 1	Overall description

Several cryptographic keys are used in the 3GPP 5GC and EPC for security purposes. These cryptographic keys could be generated, derived, persistent, ephemeral, symmetric or asymmetric. These keys are to be protected at rest, in transit, and while in use. For example, KSEAF, KAMF, K/Ki etc. Inadvertent disclosure of these keys poses a security risk.

UE trace function is standardized in 3GPP SA WG5 specifications. The specifications cover security requirements to mask the security keys from the radio access nodes in the trace file. However, UE trace specifications do not cover security requirements to mask the security keys from EPC and 5GC in trace file.

According to clause 5 of TS 32.421:

[SET-SEC-1] Keys stored inside eNBs shall never leave a secure environment within the eNB. When security key(s) transported on control signalling messages are included in the trace file, the key value(s) shall be removed and replaced with the value “Unavailable”.

[SET-SEC-2] Keys stored inside NG-RAN node shall never leave a secure environment within the NG-RAN node. When security key(s) transported on control signalling messages are included in the trace file, the key value(s) shall be removed and replaced with the value "Unavailable".

According to a note in clause 4.13 of TS 32.423:

NOTE:	For the security keys in IEs or part of IEs that are containing security keys used by the eNB (e.g. KeNB), the value 0 shall be written in the trace file.

TS 33.401 and TS 33.501 emphasize on protection of cryptographic keys. For example;

According to clause 6.2.2.1 of TS 33.501:

The SEAF shall never transfer KSEAF to an entity outside the SEAF

According to clause 5.8.1 of TS 33.501:

The long-term key(s) used for authentication and security association setup purposes shall be protected from physical attacks and shall never leave the secure environment of the UDM/ARPF unprotected.

According to clause 6.4 of TS 33.401:

KASME shall never be transferred from the EPC to an entity outside the EPC , with the exception of the following scenario(s):

-	interworking from EPS to 5G as described in clause 8.2 and 8.4 of TS 33.501 [43].

To mitigate security risks related to inadvertent leakage of security key(s) from the core network nodes, 3GPP SA WG3 proposes to include security requirements for masking security keys specified in clause 6.2 of TS 33.401 and in clause 6.2.2.1 of TS 33.501 in UE trace file.

# 2	Actions

To 3GPP SA WG5

ACTION: 	3GPP SA WG3 kindly asks SA WG5 to review proposal from the above and liaise with SA WG3 as required.

# 3	Dates of next TSG SA WG 3 meetings

SA3#118	14 - 18 October 2024		Hyderabad (India)

SA3#119	11 - 15 November 2024		Orlando (US)