3GPP TSG-SA5 Meeting #157	S5-245464

Hyderabad, India, 14 - 18 October 2024

# Source:	ZTE Corporation

# Title:	Rel-19 pCR TR 28.858 Add New Requirement Management of Federated Learning Use Case

# Document for:	Approval

Agenda Item:	6.19.1

1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

2	References

[1]	3GPP TR 28.858: " Study on Artificial Intelligence / Machine Learning (AI/ML) management Phase 2"

3	Rationale

In last SA5 meeting, the use case of Management of Federated Learning is agreed, which is a generic use case for FL, i.e., both HFL and VFL are needed to be supported. But in the existing description, description of training process for VFL is missing.

Besides, Maintaining Federated Learning Processes has been already defined in SA2 in Rel-18, see clause 6.2C.2.3 of TS 23.288, which specifies how to maintain a Federation Learning process in FL execution phase, including FL Server NWDAF triggers reselection, addition, or removal of FL Client NWDAF(s). Maintaining Federated Learning Processes also raises new management requirements

This contribution proposes to update the description and add new requirement to Management of Federated Learning use case.

4	Detailed proposal

Start of First change

5.1.10	Management of Federated Learning

5.1.10.1	Description

Federated Learning (FL) is a distributed machine learning approach that allows multiple FL clients to collaboratively train an ML model on local datasets contained in each FL client without explicitly exchanging data samples.

FL is supported by a group of FL clients and FL server wherein FL client keeps the data localized and private, and trains the ML model directly on the local nodes (client) where the data is generated or stored.

Federated learning can be categorized into two main types: Horizontal federated learning (HFL) and Vertical federated learning (VFL), based on the nature of the data distribution and the way the model training is orchestrated among participants. The training process for HFL and VFL are also different, which is summarized as follows:

- HFL Training: HFL Clients train models on local datasets and then the HFL Server aggregates the intermediate results from all HFL Clients to update the global model. The HFL Server sends the aggregated global model or updates back to all HFL Clients. Communication mainly involves the exchange of model parameters or gradients since all participants work on the same feature space.

- VFL Training: VFL Clients align shared sample IDs. Then, VFL Clients transform their local data into intermediate feature representations for joint training and exchange them with each other or with the VFL Server. VFL Clients update their local models or model parameters based on the joint training results. In VFL, the information of the interaction includes not only model parameters or gradients, but also intermediate feature representations and possible encrypted information.

For HFL, the process typically includes FL Client discovery and selection, and ML model distribution and aggregation.

Figure 5.1.10.1-1: ML model distribution and aggregation for HFL

NOTE: 	A prior agreement needs to exist between the FL server and the FL clients to exchange ML models.

For VFL, a sample alignment procedure is needed to ensure that different VFL participators share the same sample space before initiating VFL process. Based on the VFL training procedure, the functionalities for VFL Server are as follows:

- Initiate the VFL process and perform sample alignment with the selected VFL Clients

- Send the initial ML model to VFL Clients and collect the intermediate results from the VFL Clients

- Store the local ML model for future VFL inference

The functionalities for VFL Client are as follows:

- Perform the sample alignment based on the requirements from the VFL Server

- Train the received part of initial ML model with local data and exchange intermediate results with the VFL Server and other VFL Clients

- Store the local ML model for future VFL inference

5.1.10.2	Use cases

5.1.10.2.1	Management of different roles in Federated Learning

For FL, an ML model is collaboratively trained by a group of FL clients (e.g., MTLF in NWDAFs) including one acting as FL server and the others acting as FL clients. Federated Learning training allows multiple FL clients to collaboratively train an ML model on local datasets, it means that the local training of each FL client needs to start and complete almost at the same time to ensure the performance of ML model aggregation performing in FL server.

For managing the FL, the ML training MnS consumer needs to know the FL clients and FL server involved in the FL, so that the consumer understands the impact of each one of them and can manage it correspondingly.

When receiving an ML Training request, the ML Training MnS Producer should evaluate whether FL process needs to be started according to the training requirements (e.g., minimal number of FL Clients, minimal number of total iterations, minimal number of data samples and available time of the FL Clients) provided by the ML training consumer. Based on the received requirements, the FL server may select appropriate FL Clients.

To evaluate the performance of FL, the consumer needs to know the performance of the final global ML model running on the local training data set of participating FL clients. For instance, if an FL server cannot generate a global ML model with satisfied performance for the FL clients, the consumer may interact with the MnS ML training producer to optimize the FL for future training, e.g., updating the criteria for selecting FL clients.

In addition, the consumer needs to get the information about the contribution of each FL client to the FL process, for instance, number of iterations the FL client participated in the FL, number of data examples the FL client used, training duration the FL Client performed.

Maintaining a Federation Learning process in FL execution phase is helpful to make the most of resource and improve the FL training efficiency, which may include reselection, addition, or removal of FL Client(s). The consumer needs to get the information of candidate FL clients and contribution of each FL client to maintain the FL process.

5.1.10.3	Potential requirements

REQ-FL_MGMT-01: The ML training MnS producer should have a capability allowing an authorized consumer to discover the FL roles (FL server or FL client) in Federated Learning.

REQ-FL_MGMT-02: The ML training MnS producer should have a capability allowing an authorized consumer to provide FL training requirements to the MnS Producer.

REQ-FL_MGMT-03: The ML training MnS producer should have a capability allowing an authorized consumer to provide requirements for selecting FL clients in Federated Learning to the MnS Producer.

REQ-FL_MGMT-04: The ML training MnS producer should have a capability allowing an authorized consumer to get the performance of ML model on each participating FL client.

REQ-FL_MGMT-05: The ML training MnS producer should have a capability to report the information about the contribution of each FL client to the FL process to MnS consumer.

REQ-FL_MGMT-06: The ML training MnS producer should have a capability allowing an authorized consumer to maintain Federated Learning Processes.

End of First change