3GPP TSG-SA5 Meeting #157	S5-245499

Hyderabad, India, 14 - 18 October 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.158 | CR | 0144 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 32.158 Add design pattern for requests of data fractions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Increasing amount of data represented by one resource needs to be transferred. If a connection is broken, the total amount of data needs to be retransmitted. Request of data fractions increase efficiency. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce HTTP Range Requests to allow requests of data fractions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not possible to request for a fraction of data. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	IETF RFC 7231: "Hypertext Transfer Protocol (HTTP/1.1): Semantics and Content".

[3]	3GPP TS 32.300: "Telecommunication management; Configuration Management (CM); Name convention for Managed Objects".

[4]	IETF RFC 3986: "Uniform Resource Identifier (URI): Generic Syntax".

[5]	IETF RFC 7230: "Hypertext Transfer Protocol (HTTP/1.1): Message Syntax and Routing".

[6]	IETF RFC 7159: " The JavaScript Object Notation (JSON) Data Interchange Format".

[7]	draft-bhutton-json-schema-01 (June 2022): "JSON Schema: A Media Type for Describing JSON Documents".

Note: The above document is an individual draft from IETF. It cannot be formally referenced until it is published as an RFC. It is available from the following link: https://datatracker.ietf.org/doc/html/draft-bhutton-json-schema-01.

[8]	draft-bhutton-json-schema-validation-01 (June 2022): "JSON Schema Validation: A Vocabulary for Structural Validation of JSON".

Note: The above document is an individual draft from IETF. It cannot be formally referenced until it is published as an RFC. It is available from the following link: https://datatracker.ietf.org/doc/html/draft-bhutton-json-schema-validation-01.

[9]	draft-handrews-json-schema-hyperschema-02 (September 2019): "JSON Hyper-Schema: A Vocabulary for Hypermedia Annotation of JSON.

Note: The above document is an individual draft from IETF. It cannot be formally referenced until it is published as an RFC. It is available from the following link: https://datatracker.ietf.org/doc/html/draft-handrews-json-schema-hyperschema-02.

[10]	OpenAPI Specification (https://github.com/OAI/OpenAPI-Specification)

[11]	IETF RFC 5789: "PATCH Method for HTTP".

[12]	IETF RFC 7396: "JSON Merge Patch".

[13]	IETF RFC 6902: "JavaScript Object Notation (JSON) Patch".

[14]	IETF RFC 6901: "JavaScript Object Notation (JSON) Pointer".

[15]	XML Path Language (XPath) Version 1.0, W3C Recommendation 16 November 1999 (https://www.w3.org/TR/xpath-10/)

[16]	3GPP TS 32.160: "Management and orchestration; Management service template".

[17]	IETF RFC 4918: "HTTP Extensions for Web Distributed Authoring and Versioning (WebDAV)"

[18]	IETF RFC 6585: "Additional HTTP Status Codes"

[19]	IETF RFC 7807: "Problem Details for HTTP APIs"

[20]	IETF RFC 7725: "An HTTP Status Code to Report Legal Obstacles"

[21]	3GPP TS 32.161: "JSON expressions (Jex)"

[XX]	IETF RFC 7233: "Hypertext Transfer Protocol (HTTP/1.1): Range Requests".

Next change

## 6.7	Design pattern for conditional data node selection

Scoping with the query parameters "scopeType" and "scopeLevel", and filtering with the query parameter "filter" allows for conditional object selection. The query parameters "attributes" and "fields" allow for (unconditional) selection of attributes and attribute fields.

For multi-valued attributes, where the attribute elements itself are big complex data types, it may be desirable to select also attribute elements based on conditions. For example, assume an alarm list object that has a multi-valued attribute containing alarm records. For retrieving only alarm records with a certain perceived severity it needs to be possible to filter on the perceived severity and return only the alarm records matching that filter criteria. But also attributes or attribute fields may need to be selected based on conditions. For example, assume a managed object that can be locked and disabled. When locked or disabled some state attributes are not updated any more and do not reflect the current state. Reading of these state attributes is hence only meaningful when the object is neither locked nor disabled.

Conditional attribute data node selection is similar to conditional object selection with the "filter" query parameter. Instead of specifying a query parameters for conditional object selection and another parameter for conditional attribute data node selection, these selection mechanisms may be combined into a single query parameter.

The name of this query parameter is "dataNodeSelector". Jex [21] shall be used for specifying the selection expression.

## 6.X	Design pattern for requests of data fractions

Interrupted data transfer as a result of dropped connections appear. Especially in case of large amount of data, it is useful to have methods to request relevant extracts of the desired data. For example, when an MnS consumer has stored a part of the HTTP message body, it is desirable to request the remainder of this message body in a subsequent request rather than transfer the entire message body. Furthermore, range request can be used to speed up downloads by allowing parallel download of smaller fractions.

HTTP Range Requests, see IETF RFC 7233 [XX], should be used for request of data fractions. For indicating the fractions of the HTTP message body byte ranges should be used.

The "Accept-Ranges" header field inside an HTTP response allows to indicate the support of range requests. The "Range" header field in an HTTP GET request allows to carry the requested one or more fractions of the selected representation data. The "Content-Range" header field in an HTTP GET response allows to indicate the data fraction of the selected representation enclosed as the response message body.

End of changes