3GPP TSG-SA5 Meeting #157	S5-245501

Hyderabad, India, 14 - 18 October 2024

# Source:	Nokia

# Title:	Discussion on external management data

# Document for:	Endorsement

Agenda Item:	6.19.10 Data management phase 2

# 1	Decision/action requested

The group is asked to discuss and endorse detailed proposal.

# 2	References

[1]	3GPP TS 28.537 Management capabilities

[2]		3GPP TS 28.632 Inventory Management (IM) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)

[3]		3GPP TS 28.633 Inventory Management (IM) Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions

[4]	3GPP TS 28.622 Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)

[3]	3GPP 32.423 Subscriber and equipment trace; Trace data definition and management

[4]	3GPP 28.552 5G performance measurements

[5]	3GPP 28.550 Performance assurance

[6]	3GPP 28.532 Generic management services

[7]	3GPP 28.533 Architecture framework

# 3	Rationale

External management data has been discussed already in Release 17. The conclusions are summarized in clause 6.4 of TS 28.537 [1] comprising the description and requirements of external management data.

So far there is no conclusion on how to include external management data in the 3GPP management system such that it can be used. This aspect is discussed in detail in the following chapter.

# 4	Discussion

According to TS 28.533 [7] management information MnS component type C, i.e. „internal 3GPP“ management data, is associated to managed entities that are described by MnS component type B, i.e. the network resource model, and are exchanged by using the operations described by MnS component type A.

In other words, „internal 3GPP“ management data is associated to object instances of information object classes (IOC) that are defined in the network resource model. E.g. each counter (even if vendor-specific) has an associated measured object instance, and each alarm has an associated alarming object instance where the IOC are known in the NRM.

In contrast, external management data does not have such well-defined association to object instances. Nevertheless, the external data obviously has some relationship to the network because otherwise it would not make sense to handle external data in the context of network management at all. Such relationship depends on the implementation and the details of the external data.

Potential data which could be treated as external management data according to the description in TS 28.537[1] are in our understanding:

- Pictures, or videos

- Measurements which are not specified by 3GPP or are not reported by 3GPP specified PM framework e.g. brightness/lumen of sun

- Analytics and/or predictions of various parameters e.g.

  - weather forecasts,

  - traffic forecasts,

  - amount of data to be transmitted in a certain area.

  - UE paths

- Schedule for planned events e.g. concerts, sports events

In the following, it is discussed one by one how above-mentioned examples of types of external management data could be included into the 3GPP management system.

E.g. pictures or videos might be taken by a camera located at the antenna mast. For some use cases the picture or video might be associated to the equipment (while SA5 has no model of inventory of 5G specified). The picture might show a crowded train station that is covered by many cells from different base stations of different RAT. In such case it might be useful to associate the picture to all cells that are covering the train station.

Observation 1: 	External management data produced by a data source located at or in a 3GPP specified equipment might be assigned to this 3GPP specified equipment.

However, it is worth to mention that in current 3GPP specifications for 5G or SBMA, the inventory is not modelled. For 3G and 4G there exists the specification "Inventory Management (IM) Network Resource Model (NRM) Integration Reference Point (IRP)", see TS 28.632 [2], and the corresponding solution set, see TS 28.633 [3].

Observation 2: 	There is no NRM specified for inventory or network hardware in 5G or SBMA.

Besides an IOC of an inventory management NRM, another option where to assign external management data might be ManagedElement, see TS 28.622 [4]. Having a closer look at the definition, you find the following:

"A ManagedElement IOC is used to represent a Network Element defined in TS 32.101[1] including virtualization or non-virtualization scenario. […] A ManagedElement contains equipment that may or may not be geographically distributed."

This means that the "ManagedElement" MOI is not necessarily related to a single location but might present several distributed locations, namely the location(s) of the equipment which may be geographically distributed due to virtuzalization etc.

Observation 3: 	"ManagedElement" does not necessarily represent a single location and is therefore not suitable to associate with external management data which is related to a single location.

Nevertheless, the data source producing the pictures or videos could be positioned at the cell edge or at a location where no 3GPP specified equipment is located and therefore, the location of the data source cannot be directly mapped to a 3GPP specified equipment.

Observation 4: 	There might be external management data which is not directly related to a network entity defined in current 3GPP management system and can therefore not be assigned to a managed object instance of the current 3GPP management system.

Considering other data in the list above of potential external management data, like weather forecast or traffic forecast or schedule for planned events, it is obvious to assume that these are more related to a geographic area than a specific network entity.

Observation 5: 	Some external management data, e.g. weather forecast or traffic forecast, might be related to a geographic area.

Observation 6: 	In current specified 3GPP management system, geographic areas are not modelled in the NRM.

If data is related to a geographic area, it is related to a set of managed objects, NFs, or NEs. The set managed objects, NFs, or NEs might change over time e.g. by adapting transmission power or beam direction of antenna (elements), assignment policy etc.

Observation 7:	A geographic area corresponds to a set of managed objects, NFs, or NEs. This set might change over time.

Furthermore, one managed object, NF, or NE might be associated to multiple forecasts, e.g. a weather forecast and a traffic forecast. Meaning one managed object, NF or NE might be part of multiple sets. In turn, this excludes "SubNetwork" IOC as means for grouping because the NRM shall be modelled such that each network entity has only one single representation. One "SubNetwork" MOI containing two objects of external management data would mean that they belong to the same group of geographic area.

Observation 8: 	Data related to a geographic area is related to a set of managed objects, NFs, or NEs. One managed object, NF, or NE might be associated to multiple forecasts, i.e. part of multiple sets. This excludes "SubNetwork" IOC as means for grouping.

Additionally, it's worth noting that 	weather might impact managed objects, NFs or NEs, even if the corresponding network entity as such is not impacted by the weather physically. E.g. nice weather might cause many people to visit the beach, such that the serving UPF gets overloaded – independent of the weather conditions at the location of the UPF.

Observation 9: 	Weather might impact managed objects, NFs or NEs, even if the corresponding network entity as such is not impacted by the weather physically.

As discussed above external management data does not fit without adaptations or enhancements in the existing NRM. Therefore, we propose the following:

Proposal: 	Where to include the information about supported external management data in the NRM needs further study.

# 5	Detailed proposal

Observation 1: 	External management data produced by a data source located at or in a 3GPP specified equipment might be assigned to this 3GPP specified equipment.

Observation 2: 	There is no NRM specified for inventory or network hardware in 5G or SBMA.

Observation 3: 	"ManagedElement" does not necessarily represent a single location and is therefore not suitable to associate with external management data which is related to a single location.

Observation 4: 	There might be external management data which is not directly related to a network entity defined in current 3GPP management system and can therefore not be assigned to a managed object instance of the current 3GPP management system.

Observation 5: 	Some external management data, e.g. weather forecast or traffic forecast, might be related to a geographic area.

Observation 6: 	In current specified 3GPP management system, geographic areas are not modelled in the NRM.

Observation 7:	A geographic area corresponds to a set of managed objects, NFs, or NEs. This set might change over time.

Observation 8: 	Data related to a geographic area is related to a set of managed objects, NFs, or NEs. One managed object, NF, or NE might be associated to multiple forecasts, i.e. part of multiple sets. This excludes "SubNetwork" IOC as means for grouping.

Observation 9: 	Weather might impact managed objects, NFs or NEs, even if the corresponding network entity as such is not impacted by the weather physically.

Proposal: 	Where to include the information about supported external management data in the NRM needs further study.