3GPP TSG-SA5 Meeting #157	S5-245504

Hyderabad, India, 14 - 18 October 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.537 | CR | 0025 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-17 CR 28.537 Correction of requirement labels |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2024-10-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In TS 32.160, the management service template is specified. The format of the requirement label is given as follows: The format of the requirement label is REQ-xx-yy-zz, where xx is a unique abbreviation of the service/function, yy is the management capability name, and zz is the serial number under the corresponding management capability category. It is no longer differentiated between conceptual (CON), functional (FUN) etc. requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct and adapt requirement labels according to the specified format. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Several requirement labels are not according to the format specified in TS 32.160. Some references are not aligned with the defined requirement labels (REQ-DMS-CON-x. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.1.1, 4.2.1.2, 4.2.1.3, 4.2.2.1, 4.2.2.2, 5.2.1.1, 5.2.1.2, 5.2.1.3, 6.1.3, 6.2.3, 6.3.3, 6.4.3, 6.5.3, 7.1.2, 7.1.3, 7.2.3, 7.3.3, 7.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

#### 4.2.1.1	Configuring heartbeat notification periodicity

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To configure the periodicity at which the management service producer shall emit heartbeat notifications to its authorized management service consumer. |  |
| Actors and Roles | An authorized consumer of the management service. |  |
| Telecom resources | The management service producer. |  |
| Assumptions | N/A |  |
| Pre-conditions | The periodicity requested by the management service consumer has a valid value. |  |
| Begins when | The management service consumer sends a request to the management service producer to set the periodicity at which it shall emit heartbeat notifications. |  |
| Step 1 | The management service producer receives the request and sets its internal countdown timer to a value (which can be zero) equal to the periodicity requested by the management service consumer. |  |
| Step 2 | The management service producer sends a heartbeat notification to all authorized management service consumer(s), provided they previously subscribed to heartbeat notifications. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The notification periodicity has been configured according to the management service consumer request. A heartbeat notification is sent out to all authorized management service consumer(s). |  |
| Traceability | REQ-HB-CTRL-2. |  |

#### 4.2.1.2	Requesting immediate heartbeat notification

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To trigger the emission of an immediate heartbeat notification by the management service producer. |  |
| Actors and Roles | An authorized consumer of the management service. |  |
| Telecom resources | The management service producer. |  |
| Assumptions | N/A |  |
| Pre-conditions | N/A |  |
| Begins when | The soliciting management service consumer sends a request to the management service producer to emit immediately a heartbeat notification. |  |
| Step 1 | The management service producer receives the request and sends immediately a heartbeat notification to all authorized management service consumer(s) who had previously subscribed to heartbeat notifications. The management service producer countdown timer is not impacted. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The immediate heartbeat notification has been emitted according to the soliciting management service consumer request. |  |
| Traceability | REQ-HB-CTRL-3, REQ-HB-NOTIF-2. |  |

#### 4.2.1.3	Emitting periodic heartbeat notifications

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To send periodic heartbeat notifications at the periodicity requested by the management service consumer. |  |
| Actors and Roles | An authorized producer of the management service. |  |
| Telecom resources | The management service consumer. |  |
| Assumptions | The heartbeat notification periodicity has been configured according to the management service consumer request. | Configuring heartbeat notification periodicity |
| Pre-conditions | N/A |  |
| Begins when | The internal countdown timer managed by the management service producer has reached the value 0. |  |
| Step 1 | The management service producer sends a heartbeat notification to all authorized management service consumer(s), provided they previously subscribed to heartbeat notifications. |  |
| Step 2 | The management service producer resets its internal countdown timer to the value of the heartbeat notification periodicity. | Configuring heartbeat notification periodicity |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The periodic heartbeat notification has been emitted to all authorized management service consumer(s) at the requested periodicity. |  |
| Traceability | REQ-HB-NOTIF-1. |  |

### 4.2.2	Requirements

#### 4.2.2.1	Requirements for controlling heartbeat

REQ-HB-CTRL-1: The management service provider shall have the capability to allow its authorized consumer to read the heartbeat period.

REQ-HB-CTRL-2: The management service provider shall have the capability to allow its authorized consumer to set the heartbeat period.

REQ-HB-CTRL-3: The management service provider shall have the capability to allow its authorized consumer to request the emission of an immediate heartbeat notification.

#### 4.2.2.2	Requirements for notifying heartbeat

REQ-HB-NOTIF-1: The management service provider shall have the capability to send periodic heartbeat notifications to its authorized consumer at the frequency specified by the management service consumer.

REQ-HB-NOTIF-2: The management service provider shall have the capability to send immediate heartbeat notifications to its authorized consumer, upon request from the management service consumer.

Next change

#### 5.2.1.1	Adding a new management service producer to MnS registry

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | Add a MnS producer to a 3GPP management system. |  |
| Actors and Roles | MnS Producer, MnS Registry |  |
| Telecom resources | MnS producer. MnS discovery service producer. |  |
| Assumptions | MnS producer is ready to be added to MnS registry. |  |
| Pre-conditions | The MnS Producer is available. |  |
| Begins when | There is a need for a MnS producer to be exposed via MnS registry. |  |
| Step 1 (M) | The MnS producer is added to the MnS registry. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the mandatory steps fails. |  |
| Post-conditions | MnS discovery service producer has stored the MnS information. |  |
| Traceability | REQ-DMS-1 |  |

#### 5.2.1.2	Removing a management service producer from MnS registry

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | Remove a MnS producerfrom MnS registry.. |  |
| Actors and Roles | Network operator. |  |
| Telecom resources | MnS producer. |  |
| Assumptions | - |  |
| Pre-conditions | The management service producer is no longer required in the MnS registry. |  |
| Begins when | The management service is ready to be removed from MnS Registry. |  |
| Step 1 (M) | The management service producer is removed from the MnS Registry. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the mandatory steps fails. |  |
| Post-conditions | MnS discovery service producer has removed the MnS information related to the MnS Producer. |  |
| Traceability | REQ-DMS-1 |  |

#### 5.2.1.3	MnS Consumer retrieves management service information from MnS registry

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | MnS consumer retrieves information from MnS registry. |  |
| Actors and Roles | MnS Consumer |  |
| Telecom resources | MnS registry |  |
| Assumptions | MnS consumer is authorized to obtain the MnS information for the available management service(s) from MnS discovery service producer. |  |
| Pre-conditions | MnS information exists in MnS registry. |  |
| Begins when | MnS Consumer needs to access a specific MnS Producer(s). |  |
| Step 1 (M) | MnS Consumer queries MnS Registry with filter criteria based on the management service(s) of interest. |  |
| Step 2 (M) | MnS Consumer receives response with MnS Info for the management service(s) which match the criteria. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the mandatory steps fails. |  |
| Post-conditions | MnS Consumer has basic information about the management service(s). |  |
| Traceability | REQ-DMS-2, REQ-DMS-3, REQ-DMS-4 |  |

NOTE: MnS information refer to the information used by the consumer to discover the producers of specific Management Services and to derive the addresses of the Management Service.

Next change

### 6.1.3	Requirements

REQ-MDM-PR-1: The 3GPP management system shall enable an authorized data consumer to request management data (specified by 3GPP) to be produced.

REQ-MDM-PR-2: The 3GPP management system shall enable an authorized data consumer to request management data specified by 3GPP to be produced by certain managed object instance(s) only. The selection criteria to determine the managed object instance(s) shall be deterministic in such a way that the target node(s) can be selected unambiguously. The managed object instances can be targeted based on:

-	Area of interest (e.g. list of cells, list of tracking areas or geographical area).

-	Domain (CN or RAN).

-	User plane or control plane.

-	Slice type (e.g. eMBB, URLLC, mIoT, V2X, HMTC).

The MnS to request management data specified by 3GPP in a simple way shall not be exposed at any network function.

The mapping of geographical area to corresponding managed object instances reflects the cell coverage status at the time of the request.

REQ-MDM-PR-3: The 3GPP management system shall enable an authorized data consumer to request management data specified by 3GPP to be produced according to a certain time scheduler.

REQ-MDM-PR-4: The 3GPP management system shall enable an authorized data consumer to request management data (specified by 3GPP) to be reported to the requesting or another authorized data consumer.

REQ-MDM-PR-5: The 3GPP management system shall enable an authorized data consumer to obtain context data for management data. Access to management data does not imply access to context data. Different data privacy considerations may apply.

NOTE: The term "management data specified by 3GPP" relates to

-	5G performance measurements as defined by TS 28.552 [4]

-	5G end to end key performance indicators as defined by TS 28.554 [5], and

-	Trace/MDT data as defined by TS 32.422 [6].

Next change

### 6.2.3	Requirements

REQ-MDM-C-1: The 3GPP management system shall coordinate requests from several data consumers to avoid producing multiple times the same data at a certain point of time.

Editor's note: It is tbc what exactly is "same data".

Next change

### 6.3.3	Requirements

REQ-MDM-S-1: The 3GPP management system shall support the storing of produced management data.

REQ-MDM-S-2: The 3GPP management system shall enable an authorized data consumer to discover stored management data.

REQ-MDM-S-3: The 3GPP management system shall enable an authorized data consumer to retrieve stored management data.

Next change

### 6.4.3	Requirements

REQ-MDM-ED-1: The 3GPP management system shall enable an authorized data consumer to request external management data to be produced.

REQ-MDM-ED-2: The 3GPP management system shall enable an authorized data consumer to request external management data to be reported to the requesting or another authorized data consumer.

REQ-MDM-ED-3: The 3GPP management system shall support the storing of produced external management data.

REQ-MDM-ED-4: The 3GPP management system shall enable an authorized data consumer to discover stored external management data.

REQ-MDM-ED-5: The 3GPP management system shall enable an authorized data consumer to retrieve stored external management data.

NOTE: The term "external management data" relates to data not specified by 3GPP.

Next change

### 6.5.3	Requirements

REQ-MDM-DIS-1: The 3GPP management system shall enable an authorized data consumer to discover what management data can be produced by 3GPP management system.

Next change

### 7.1.2	Void

### 7.1.3	Requirements

REQ-FM-G-1: The file transfer protocol shall preserve the formatting of the file during exchange.

REQ-FM-G-2: The file transfer protocol shall preserve the encoding of the file during exchange.

REQ-FM-G-3: The MnS producer shall support at least one of the following file transfer protocols: SFTP, FTPES, HTTPS.

REQ-FM-G-4: The MnS consumer shall use the file transfer protocol supported by the MnS producer.

Next change

### 7.2.3	Requirements

REQ-FM-R-1: The MnS producer shall support the capability allowing a MnS consumer to retrieve (get) a file from the MnS producer.

REQ-FM-R-2: The MnS producer shall support the capability allowing a MnS consumer to retrieve the list of files available for transfer from the MnS producer.

REQ-FM-R-3: The MnS producer shall support the capability to inform a MnS consumer about files that are available for retrieval.

REQ-FM-R-4: The MnS producer shall support the capability to inform a MnS consumer about errors that occurred during the preparation of a file.

REQ-FM-R-5: The information transferred to a MnS consumer about an available file shall allow associating the file to the process on the MnS producer that generated the file, if any such process exists and has an identifier.

REQ-FM-R-6: The MnS producer shall support the capability allowing a MnS consumer to indicate to the MnS producer, that the MnS consumer does not need a file anymore, such that the MnS producer hides the file in responses to subsequent read requests or decide to delete it altogether.

Next change

### 7.3.3	Requirements

REQ-FM-P-1: void

REQ-FM-P-2: void

REQ-FM-P-3: void

REQ-FM-P-4: The MnS producer shall support the capability to push a file to the MnS consumer or a designated file server when configured by a MnS consumer.

REQ-FM-P-5: The MnS producer shall support the capability for a MnS consumer to configure the MnS producer to push a file to the MnS consumer or a designated file server based on an event occurring on the MnS producer.

REQ-FM-P-6: The MnS producer shall support the capability to inform the MnS consumer, that has configured a file push, or any other MnS consumer about the completion of that file push.

Next change

### 7.4.3	Requirements

REQ-FM-D-1: The MnS producer shall support the capability to download a file from a MnS consumer or a designated file server when triggered by a MnS consumer.

REQ-FM-D-2: The MnS producer shall support the capability allowing a MnS consumer to trigger the MnS producer to download a file from the MnS consumer or a designated file server.

REQ-FM-D-3: The MnS producer shall support the capability to inform the MnS consumer that has triggered a file download, or any other consumer about the progress of that file download.

End of changes