3GPP TSG-SA5 Meeting #157	S5-245561

Hyderabad, India, 14 - 18 October 2024

# Source:	Amdocs

# Title:	Reference Points N107 and N108

# Document for:	Approval

Agenda Item:	7.3

# 1	Decision/action requested

The group is asked to discuss and approve the proposal.

# 2	References

[1]	3GPP TS 32.255 Charging management; 5G data connectivity domain charging; Stage 2

[2]	3GPP TS 32.240 Charging management; Charging architecture and principles

[3]	3GPP TS 23.501 System architecture for the 5G System (5GS); Stage 2

[4]	3GPP TR 21.905 Vocabulary for 3GPP specifications

# 3	Rationale

The definitions and use of reference points N107 and N108 is inconsistent and overlapping in charging specifications. The discussion paper proposes an enhancement to streamline the use of these reference points to help avoid confusion.

## 3.1 Background

The TS 23.501 clause 4.2.1 defines a reference point representation as:

“A reference point representation, shows the interaction exist between the NF services in the network functions described by point-to-point reference point (e.g. N11) between any two network functions (e.g. AMF and SMF)

Further TS 23.501 clause 4.2.7 states:

“…reference points are realized by corresponding NF service-based interfaces and by specifying the identified consumer and producer NF service as well as their interaction in order to realize a particular system procedure”

The reference points relevant to Nchf Converged Charging service are listed in TS 32.240 clause 4.4.3

N40:	Reference point between SMF and the CHF in the same PLMN defined in clause 4.2 of TS 32.255 [15].

N41:	Reference point between AMF and CHF in different PLMNs defined in clause 4.2 of TS 32.256 [16].

N42:	Reference point between AMF and CHF in the same PLMN defined in clause 4.2 of TS 32.256 [16].

….

….

N47: 	Reference point between SMF and the CHF in different PLMNs defined in clause 4.2 of TS 32.255 [15].

….

N107:	Reference point between two CHFs (V-CHF and H-CHF) defined in clause 4.2 of TS 32.255 [15] and clause 4.2 of TS 32.256 [16].

N108:	Reference point between two CHFs (C-CHF to B-CHF) defined in clause 4.3 of TS 32.255 [15] and clause 4.3 of TS 32.256 [16].

- While N40, N41, N42, N47 are clearly defined with respect to PLMN of the consumer and producer NFs, the definition for N107 and N108 is not so clear:

  - TS 32.255 clause 4.2 specifies N107:

    - For Roaming, in terms of PLMN i.e. V-CHF v/s H-CHF

    - For MVNO, in terms of ownership of CHF function

The N108 is defined in terms of – functional role of CHF e.g. Business Charging v/s retail charging, regardless of the PLMN or the ownership of the CHF

### 3.1.1 Definition and Use of Reference Points N47 and  N107

As per TS 32.255 clause 4.2:

The N40 reference point is defined for the interactions between V-SMF and V-CHF, the N47 reference point is defined for the interactions between V-SMF and H-CHF

For scenarios with MVNO (owning a CHF referred to as A-CHF) non-roaming: the N40 reference point is defined for the interactions between SMF and CHF owned by MNO, the N47 reference point is used for the interactions between SMF owned by the MNO and A-CHF owned by the MVNO.

The above clause defines N47 for non-roaming scenario i.e. same PLMN, which contradicts the definition of N47 in TS 32.240 clause 4.4.3 stated above.

Furthermore, An MVNO may or may not have same PLMN Id as the MNO.

In case the MVNO has different PLMN than MNO, then the use N47 matches its definition in TS 32.240 (i.e. in different PLMN). But, if an MVNO has same PLMN as MNO, then the use of N47 contradicts the definition of N47 in TS 32.240

TS 32.255 clause 4.2 also states:

N47 used by A-CHF owned by an additional actor (i.e. MVNO) to perform retail charging for its own subscribers is operator specific.

- This implies that N47 is defined in specifications for roaming scenario, but not for MVNO.

TS 32.255 clause 4.2 describes two possible architectures for MVNO Charging:

For scenarios with MVNO (owning a CHF referred to as A-CHF) non-roaming, the N40 reference point is defined for the interactions between SMF and CHF owned by MNO, the N47 reference point is used for the interactions between SMF owned by the MNO and A-CHF owned by the MVNO.

…

For scenarios with MVNO (owning a CHF referred to as A-CHF) non-roaming, the N40 reference point is defined for the interactions between SMF and CHF owned by MNO, the N107 reference point is used for the interactions between CHF owned by the MNO and A-CHF owned by the MVNO.

These can be shown as below (These may be added to TS 32.255 clause 4.2 for clarity on MVNO charging)

As per TS 32.255 clause 4.2, one or both of the above architectures can be supported. In case both architectures are supported, the SMF determines, which one to use for an inbound roaming based on operator agreement.

- However, the option using N47 is applicable only if MVNO has a separate PLMN than MNO

### 3.1.2 Definition and Use of Reference Point N108:

TS 32.255 clause 4.3 defines N108 for interactions between C-CHF and B-CHF as shown below:

Since, N40 is defined in same PLMN, it is understood from above architecture that C-CHF (or consumer CHF) is in the same PLMN as SMF. Also, the N47 (i.e. SMF -> Producer CHF), which is defined across PLMNs, is not defined for business charging.

- This implies that business charging is defined for same PLMN scenario only i.e. when C-CHF and B-CHF belong to same PLMN.

Furthermore, the TS 32.240 Informative Annex G describes different scenarios and architectures for business charging

Clause G.2.2 describes charging architecture for business charging with B-CHF only e.g. Inter-CHF communication is not used

Clause G.2.3 describes business charging using N108 between C-CHF and B-CHF

Clause G.2.4 describes business charging using N107 between B-CHF and C-CHF in case of local breakout roaming.

In the B2B charging architecture where the NF (CTF) have direct B-CHF interaction and where the B-CHF interact with the C-CHF using N107 reference point, the charging information collected by the NF (CTF) is per consumer and used for both B2C and B2B charging.

The related NF(s) and the corresponding references for the B2B charging with B-CHF, where the B-CHF interacts with the C-CHF include, but not limited to:

-	5G data connectivity charging in local breakout roaming scenario, as specified in Figure 4.2.6a of TS 32.255 [15];

- 	5G connection and mobility charging, in local breakout roaming scenario, as specified in Figure 4.2.2.3 of TS 32.256 [16];

Though both G.2.3 and G.2.4 describe business charging, one uses N107 and the other one uses N108

Specifically, clause TS 32.240 Annex G.2.4 refers to fig 4.2.6a, which if modelled for business charging, would be shown as below:

It is clear from the above analysis that the Annex G shows reference point N108 used for business charging in same PLMN, whereas N107 is used for business charging across different PLMNs

## 3.1 Ambiguity between N107 and N108

The definitions and use of N107 and N108 reference points is ambiguous in charging specifications, as described above e.g. for data connectivity charging in local breakout roaming scenario, the reference point between V-CHF and H-CHF could be:

- N107 as described in TS 32.255 fig 4.2.6a

- OR N108, as described in TS 32.240 Annex G.2.4

This contradicts the definition of a reference point in TR 21.905, which says:

Reference point: A conceptual point at the conjunction of two non-overlapping functional groups (source: ITU-T I.112).

# 4	Detailed proposal

The proposal is to define the reference point between two CHF in terms of PLMNs e.g.

N107 for CHF in different PLMN regardless of whether it is used for roaming or MVNO charging

N108 for CHF in same PLMN regardless of whether it is used for business charging or MVNO

This means that if MVNO use the same PLMN as MNOs, then the reference point  between MNO CHF and A-CHF would use N108

And if MVNO use a different PLMN than MNO, then the  reference point between the two CHFs would use N107.

If an MVNO uses the same PLMN as MNO, then the charging architecture between visited MNO, home MNO and MVNO/Additional Actor uses N107 for inter-PLMN and N108 for intra-PLMN interface between two CHF

If an MVNO uses a different PLMN as MNO, then the charging architecture between visited MNO, home MNO and MVNO/Additional Actor uses N107 for inter-PLMN interface between two CHF

If agreed, TS 32.240, TS 32.255 and TS 32.256 would need updates as per above proposal.