3GPP TSG-SA5 Meeting #157	S5-245590

Hyderabad, India, 14 - 18 October 2024

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.290 | CR | 0234 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 32.290 Correction on CHF as NF consumer detected failure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The CHF as NF consumer detected failure handling contains two aspects. First, CHF as NF consumer need to informing CTF about the failure to assist it handling the service after failure (e.g. application level failure handling defined in clause 5.5.1.1). Second, the CHF as NF consumer need to handle the charging session between CTF and consumer CHF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the CHF as NF consumer detected failure handling mechanism. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The CHF as NF consumer detected failure handling mechanism is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of S5-244003 (SA5#156). |  |  |  |  |  |  |  |  |

First change

5.5.1.3	CHF as NF Consumer detected failure

When a Charging Data Request [Initial/Update/Termination] received by the consumer CHF for inter CHFs communication, in the case of the consumer CHF towards producer CHF request times out and without alternative producer CHF, consumer CHF uses application level failure handlingwhich may be received from the producer CHF previously or may be locally configured based on operator agreement, as following:

-	Terminate: the service in NF Consumer(CTF) should not be granted by consumer CHF;

-	Continue: the service in NF Consumer(CTF) SHOULD be granted by consumer CHF, and charging information may be stored in the consumer CHF. Once the connection to a producer CHF is established, the producer CHF may receive the charging information that were previously stored by consumer CHF.

-	Retry_and_terminate: the consumer CHF should re-send the request to an alternative producer CHF. Otherwise the service in NF Consumer(CTF) should not be granted by consumer CHF when the charging data request can't be delivered.

End of change