3GPP TSG SA WG5 Meeting #157	S5-245592

Hyderabad, India, 14 - 18 October 2024

# Source:	Huawei

# Title:	Introduce the solution for satellite roaming charging

# Document for:	Approval

Agenda Item:	7.5.1

1 	Discussion

This is a pCR to provide the solutions for satellite roaming charging in TR 28.846.

2	References

[1]	3GPP TR 28.846: "Study on charging aspects of satellite access Phase 3".

3	Rationale

This pCR proposes to introduce the solutions for satellite roaming charging.

4	Detailed proposal

Propose to incorporate the following change into the TR 28.846 [1].

First change

6.2.4	Possible solutions

6.2.4.1	Solution #2.1: Roaming architecture for satellite roaming case

6.2.4.1.1	General description

This solution #2.1 which relying on CHF/5G Converged Charging System for satellite roaming cases, including the Home Routed and Local breakout roaming scenarios, addresses the Key Issue #2.1.

6.2.4.1.2	Architecture description

The 5G System high level charging architecture for SMF charging in roaming cases to support the satellite backhaul roaming.

6.2.4.1.3	Procedures description

According to the figure 4.3.2.2.2 in TS 23.502[X], for Home-routed Roaming case, the Nsmf_PDUSession_Create Request or Nsmf_PDUSession_Update Request is transferred from V-SMF to H-SMF in step6, including the Satellite backhaul category.

In the Home Routed charging, the Satellite backhaul category or the satellite access type to V-CHF and H-CHF.

According to the figure 4.3.2.2.1 in TS 23.502[X], for Local breakout Roaming, the Nsmf_PDUSession_CreateSMContext Request and Nsmf_PDUSession_UpdateSMContext Request is transferred from AMF to SMF in step3, including the Satellite backhaul category, GEO Satellite ID

In the LBO charging, the Satellite backhaul category and GEO Satellite ID or the satellite access type is reported to V-CHF and H-CHF.

End of change