3GPP TSG SA WG5 Meeting #157	S5-245601

Hyderabad, India, 14 - 18 October 2024

# Source:	Huawei

# Title:	Introduce the Aircraft-to-Everything for UAS Charging

# Document for:	Approval

Agenda Item:	7.5.4

1 	Discussion

This is a pCR to provide the Aircraft-to-Everything for UAS charging in TR 28.853.

2	References

[1]	3GPP TR 28.853: "Study on charging aspects of uncrewed aerial systems".

3	Rationale

This pCR proposes to introduce the Aircraft-to-Everything service for UAS Charging.

4	Detailed proposal

Propose to incorporate the following change into the TR 28.853 [1].

First change

5.4	Topic 4: Aircraft-to-Everything (A2X) services for UAS Charging

5.4.1	Use cases

5.4.1.1	Use Case #4.1: AF-based service parameter provisioning for A2X communication

The UAS-SP (A2X Application Server as AF) and the UAS-SC (A2X Application in UE as the UAS) has the service agreement about the A2X service subscription. The USA-SP and UAS-MNO have the service agreement and subscription for 5GS network access.

The A2X Application Server performs the AF-based service parameter provisioning for A2X communications via NEF. The NEF stores the A2X service parameters in the UDR.

The charging party: UAS-MNO

The charged party: UAS-SP.

The potential charging requirements for this Use Case is: REQ-3GPPCH-A2X-01.

Next change

5.4.1.2	Use Case #4.2: Non-roaming 5G System for A2X communication

The UAS-SP (A2X Application Server) and the UAS-SC (A2X Application in UE as the UAS) has the service agreement about the A2X service subscription.

The UAS-SC and UAS-MNO have the service agreement and subscription for 5GS network access. The subscription to A2X services based on the user's profile stored in the UDM.

- 	The A2X communication is supported between UAS-SC (UAS UEs).

- 	The A2X communication is supported between USA-SC (UAS UE) and UAS-SP (A2X Application Server).

The charging party: UAS-MNO

The charged party: UAS-SC

The potential charging requirements for this Use Case is: REQ-3GPPCH-A2X-02.

The UAS-SC and UAS-MNO have no the service agreement and subscription for the A2X communication. The UAS-SC (UAV UEs) can only support the A2X communication can be supported when authorized for "not served by E-UTRA" and "not served by NR", which charging relationship is outside the scope of 3GPP.

Next change

5.4.1.3	Use Case #4.3: Roaming 5G System architecture for A2X communication

The UAS-SP (A2X Application Server) and the UAS-SC (A2X Application in UE as the UAS) has the service agreement about the A2X service subscription. The UAS-SC and Home UAS-MNO have the service agreement and subscription for 5GS network access.

- 	In the local breakout roaming scenarios, the UAS-SC (UAS UEs) are roamed into Visited UAS-MNO, the A2X Application Server is located in the Visited UAS-MNO.

- 	In the home routed roaming scenarios, the UAS-SC (UAS UEs) are roamed into Visited UAS-MNO, the A2X Application Server is located in the Home UAS-MNO.

The A2X services for UAS charging including the following aspects:

- 	The A2X communication is supported between UAS-SC (UAS UEs).

- 	The A2X communication is supported between USA-SC (UAS UE) and UAS-SP (A2X Application Server).

The Retail charging party: Home UAS-MNO

The Retail charged party: UAS-SC

The wholesale charging is between the Home UAS-MNO and Visited UAS-MNO, which is outside the scope of 3GPP.

The potential charging requirements for this Use Case is: REQ-3GPPCH-A2X-03.

Next change

5.4.2	Potential charging requirements

The following are potential high-level charging requirements for A2X service for USA Charging:

REQ-3GPPCH-A2X-01: The 5G System should support converged charging for service provision invocation per UAS-SP basis.

REQ-3GPPCH-A2X-02: The 5G System should support converged charging for A2X communication per UAS-SC basis.

REQ-3GPPCH-A2X-01: The 5G System should support converged charging for LBO and HR roaming case A2X communication per UAS-SC basis.

Next change

5.4.3	Key issues

5.4.3.1	Key issue #4.1: Charging information required

This key issue is for investigating how to support the A2X services for UAS charging considering REQ- 3GPPCH-A2X-01, REQ- 3GPPCH-A2X-02 and REQ- 3GPPCH-A2X-03. This investigation covers the following:

-	identification of the charging architecture for the A2X service for UAS Charging;

-	identification of triggers and charging information required to A2X service for UAS Charging.

End of change