3GPP TSG SA WG5 Meeting #157	S5-245602

Hyderabad, India, 14 - 18 October 2024

# Source:	Huawei

# Title:	New charging solution for provisioning for A2X communication

# Document for:	Approval

Agenda Item:	7.5.4

1 	Discussion

This is a pCR to provide the Aircraft-to-Everything for UAS charging in TR 28.853.

2	References

[1]	3GPP TR 28.853: "Study on charging aspects of uncrewed aerial systems".

3	Rationale

This pCR proposes a new charging solution for Aircraft-to-Everything service.

4	Detailed proposal

Propose to incorporate the following change into the TR 28.853 [1].

First change

5.4.4.1	Solution #4.1: Provisioning for A2X communication via UAS NF/NEF

5.4.4.1.1	General description

This solution#4.1 which relying on CHF/5G Converged Charging System for UAS NF/NEF Charging, addresses the Key Issue#4.1.

5.4.4.1.2	Architecture description

The 5G System high level charging architecture for UAS NF/NEF Charging to support the AF-based service parameter provisioning for A2X communication is the same with figure 5.3.4.1.2-1 in clause 5.3.4.1.

5.4.4.1.3	Procedures description

The high-level charging procedure for UAS NF/NEF charging for AF-based service parameter provisioning for A2X communication is same with figure 5.3.4.1.3-1, with the difference in steps:

- 	Step1: AF (A2X Application Server) sends the service parameter provisioning for A2X communication request to NEF.

- 	Step 3: Not required

End of change