3GPP TSG-SA5 Meeting #157	S5-245610

Hyderabad, India, 14 - 18 October 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.104 | CR | 0137 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.104 Correct the allowed value of issue type |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In clause 7.2.2.1 of TS 28.104, service experience analysis is required to identify the source of service experience issue, e.g. RAN issue, CN issue, TN issue, UE issue, service provider issue. However, in the analytics output for service experience analysis, the allowed value of serviceExperienceIssueType only include RAN issue and CN issue, and not consider other issue such as TN issue, UE issue and service provider issue . |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add “Other issue” in the allowed value of serviceExperienceIssueType to align with the use case and requirement in clause 7.2.2.1 of TS 28.104. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It will lead to misunderstanding and wrong implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

8.4.2.1	Service experience analysis

8.4.2.1.1	MDA type

The MDA type for Capability-Service experience analysis is: SLSAnalysis.ServiceExperienceAnalysis.

8.4.2.1.2	Enabling data

The enabling data for SLSAnalysis.ServiceExperienceAnalysis MDA type are provided in table 8.4.2.1.2-1.

Table 8.4.2.1.2-1: Enabling data for service experience analysis

| Data category | Description | References |
| --- | --- | --- |
| Performance measurements | Average e2e uplink/downlink delay for a network slice | Average e2e uplink/downlink delay for a network slice (in clause 6.3.1.8 in TS 28.554 [5]). |
|  | Integrated uplink/downlink delay in RAN | Integrated downlink delay in RAN (clause 6.3.1.2 in TS 28.554 [5]); Integrated uplink delay in RAN (clause 6.3.1.7 in TS 28.554 [5]). |
|  | Round-trip packet delay | Round-trip packet delay between PSA UPF and NG-RAN (clause 5.4.8 in TS 28.552 [4]). |
|  | UL/DL throughput for network and Network Slice Instance | Upstream throughput for network and Network Slice Instance (clause 6.3.2 in TS 28.554 [5]); Downstream throughput for Single Network Slice Instance (clause 6.3.3 in TS 28.554 [5]). |
|  | RAN UE Throughput | RAN UE Throughput (clause 6.3.6 in TS 28.554 [5]) |
|  | Throughput at N3 interface | Upstream Throughput at N3 interface (clause 6.3.4 in TS28.554 [5]); Downstream Throughput at N3 interface (clause 6.3.5 in TS28.554 [5]). |
| QoE Data | The QoE data of the different services | QoE data (TS 26.247 [22] and TS 26.114 [23] can be acquired through the procedures defined in TS 28.405 [8]). |

8.4.2.1.3	Analytics output

The specific information elements of the analytics output for service experience analysis, in addition to the common information elements of the analytics outputs (see clause 8.3), are provided in table 8.4.2.1.3-1.

Table 8.4.2.1.3-1: Analytics output for Service experience analysis

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| serviceExperienceId | The identifier indicates the analytics report is related with service experience analysis. | M | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| serviceInformation | This field include the service information related to this analysis such as service name. See NOTE 1. | O | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| serviceExperienceIssueType | Indication of the service experience issue type. allowedValues: - RAN issue; - CN issue; - Other issue | M | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| affectedObjects | The managed object instances where the service experience is applicable, e.g. SubNetwork Instance, NetworkSlice Instance, NetworkSlice subnetwork Instance. The subset values of this field may be different due to cross domain management and domain management. | O | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| serviceExperienceStatistics | The statistics of the level of service experience for a service in a certain time period, e.g. there are five levels which are represented by 1, 2, 3, 4, 5 where level 1 represents the users are enduring bad experience while level 5 represents the users' requirements are perfectly satisfied. | O | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| serviceExperiencePredictions | The predictions of the level of service experience for a service in a certain time period. | O | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NOTE 1: This field of serviceInformation is used for MDA MnS producer to include the names of e2e services (e.g., browsring, video streaming etc.) and detail information (specific information of an e2e service). |  |  |  |

End of Changes