3GPP TSG-SA5 Meeting #157	S5-245618

Hyderabad, India, 14 - 18 October 2024

# Source:	Nokia

# Title:	Rel-19 pCR TR 28.880 Potential solution for use case Handling of power shortages

# Document for:	Approval

Agenda Item:	6.19.20

# 1	Decision/action requested

The group is requested to discuss and approve the pCR below.

# 2	References

[1]          3GPP TR 28.880: "Study on energy efficiency and energy saving aspects of 5G networks and services".

# 3	Rationale

This pCR proposes a potential solution for Use case #11: Handling of power shortages, of TR 28.880 [1].

# 4	Detailed proposal

The following changes are proposed for TR 28.880[1].

1st Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[3]	ETSI GR NFV-IFA 029 (V3.3.1): "Network Functions Virtualisation (NFV) Release 3; Architecture; Report on the Enhancements of the NFV architecture towards "Cloud-native" and "PaaS"".

[4]	ETSI GS NFV-IFA 040 (V4.3.1): "Network Functions Virtualisation (NFV) Release 4; Management and Orchestration; Requirements for service interfaces and object model for OS container management and orchestration specification".

[5]	ETSI GS NFV-IFA 027 (V4.3.1): "Network Functions Virtualisation (NFV) Release 4; Management and Orchestration; Performance Measurements Specification".

[6]	ETSI GS NFV-IFA 027 (V5.1.1): "Network Functions Virtualisation (NFV) Release 5; Management and Orchestration; Performance Measurements Specification".

[7]	3GPP TS 22.261: "Service requirements for the 5G system".

[8]	ETSI GS OEU 020: "Operational energy Efficiency for Users (OEU); Carbon equivalent Intensity measurement; Operational infrastructures; Global KPIs; Global KPIs for ICT Sites".

[9]	ETSI EN 303 472: "Environmental Engineering (EE); Energy Efficiency measurement methodology and metrics for RAN equipment".

[10]	ISO/IEC 30134-3:2016: "Information technology -- Data centres -- Key performance indicators -- Part 3: Renewable energy factor (REF)".

[11]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[12]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[13]	3GPP TS 28.313: "Management and orchestration; Self-Organizing Networks (SON) for 5G networks".

[14]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[15]	Energy Benchmark 2023: Telco Insights and Industry Health Check.

[16]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

Editor's Note: reference [6] is not published yet, will be published soon, the latest draft of DGS/NFV-IFA027 is available in the following location: https://docbox.etsi.org/ISG/NFV/Open/Drafts/IFA027ed451

[17]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[18]	3GPP TS 22.104: "Service requirements for cyber-physical control applications in vertical domains".

[X]	ITU-T Recommendation L.1333: "Carbon data intensity for network energy performance monitoring".

[Y] 	3GPP TS 28.104: "Management and orchestration; Management Data Analytics (MDA)"

Next Change

## 5.11	Use case #11: Handling of power shortages

### 5.11.1	Description

In this use case, the 3GPP network management system becomes aware of current or upcoming power shortages. The network operator in turn takes actions (e.g. at service, network and/or resource level) to ensure the delivery of communication services such as emergency calls, first responder communication, voice calls etc. while reducing energy use.

### 5.11.2	Potential requirements

REQ-Energy_Shortage_Handling-01: The 3GPP management system, in case it is made aware of current energy shortages or upcoming energy shortages, should be able to take appropriate measures to conserve energy use in the network while ensuring delivery of important communication services.

NOTE:	Examples for communication services that need to be preserved include emergency calls, first responder communication, voice calls etc.

### 5.11.3	Potential solutions

#### 5.11.3.y	Potential solution #Y: Service adjustment based on available energy information

##### 5.11.3.y.1	Introduction

In this potential solution, it is proposed to enhance the MDA assisted Energy Saving use case (see clause 7.2.4 of TS 28.104 [Y]) to consider the power shortage information and re-use the existing inputs such as traffic trends and provides service adjustment recommendations for different periods of time.

##### 5.11.3.y.2	Description

MDA assisted Energy Saving use case is defined in clause 7.2.4 of TS 28.104 [Y], and the data definitions for the same are defined in clause 8.4.4 of TS 28.104 [Y]. This potential solution proposes enhancement to this MDA use case.

It is proposed to enhance the enabling data for this MDA use case with information on power shortage and outage information and enhance the analytics output from this MDA use case with service adjustment recommendations for different periods of time.

The network elements and network functions can be powered with multiple sources of energy at different time periods. These power sources include power grids, batteries, UPS (Uninterrupted Power Supplies), generators (e.g., diesel back-up generator), on-site generated power (e.g., solar power, wind power). These sources of energy are generally considered as Grid electricity, Backup energy and locally generated energy (see clause 6 of ITU-T Recommendation L.1333 [X]). The shortage and availability from each source could be different. Such information can be configured by the operator to provide this information to 3GPP system and used by the MDA assisted Energy Saving use case.

The operator can configure the power supply information using a new PowerSupplyInfo IOC as proposed below.

PowerSupplyInfo IOC can be name-contained by ManagedEntity proxy class representing SubNetwork or ManagedElement. Hence, the power supply information can be associated with the network elements and network functions that are name-contained under the same SubNetwork or ManagedElement.

The operator can configure the power supply information using PowerSupplyInfo IOC as described below.

PowerSupplyInfo IOC is proposed to be inherited from Top IOC.

Figure 5.3.3.X.2-1: Inheritance for NRM fragment for PowerSupplyInfo

PowerSupplyInfo IOC is proposed to be name-contained by ManagedEntity proxy class representing SubNetwork or ManagedElement.

Figure 5.3.3.X.2-2: Relationship for NRM fragment for PowerSupplyInfo

The following attributes are proposed for the PowerSupplyInfo IOC:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| powerSource | M | T | T | F | T |
| usageTimeScheduleList | M | T | T | F | T |
| energyOutages | M | T | T | F | T |

The following table provides the attribute definitions:

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| powerSource | It defines the source of power used. The power source can be one of the following, GRID_ELECTRICITY: Most of the energy consumed comes from local grid electricity. (See Grid electricity in clause 6 of ITU-T Recommendation L.1333 [X]) BACKUP_ENERGY: A small proportion of energy comes from backup diesel or other types, or from generators such as solar panels. (See Backup energy in clause 6 of ITU-T Recommendation L.1333 [X]) LOCALLY_GENERATED_ENERGY: energy provided by local generators not used for backup functionality, e.g., solar, wind generator. (See Locally generated energy in clause 6 of ITU-T Recommendation L.1333 [X]) allowedValues: GRID_ELECTRICITY, BACKUP_ENERGY, LOCALLY_GENERATED_ENERGY | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| usageTimeScheduleList | It defines the scheduling times for usage of the power source. This list can consist of one or more scheduling times for the power source. If it is configured to null, then the power source is used always. allowedValues: NA | Type: SchedulingTime multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| energyOutages | It defines the scheduling times for the outages for the power source. This list can consist of one or more scheduling times for the power source. If it is configured to null, then the power source is always available. allowedValues: NA | Type: SchedulingTime multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |

ServiceProfile (see clause 6.3.3 of TS 28.541 [16]) data type includes attributes that can represent service requirements. The service adjustment recommendations can be represented using the ServiceProfile.

The enhancements to the enabling data and analytics output for the MDA assisted Energy Saving use case (see clause 8.4.4 of TS 28.104 [Y]) are described below.

Extensions to the enabling data for this MDA use case is described below:

| Data category | Description | References |
| --- | --- | --- |
| Configuration data | PowerSupplyInfo associated with MOIs of the cells, UPFs and SMFs. | TS 28.622 [14]. |

Extensions to the analytics output from this MDA use case is described below:

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| networkServiceOperabilityRecommendations | It contains the recommendations of network service operability levels based on energy availability. This can be expressed as ServiceProfile. The recommended values in ServiceProfile could indicate different levels of SLAs (i.e., the recommended QoS) for different time periods (expressed using attribute sliceAvailability) and applicable for entire network or parts of the network (expressed using coverageArea) | M | type: ServiceProfile (see TS 28.541 [16]) multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |

### 5.11.4	Evaluation of potential solutions

The potential solution #Y proposes to introduce a new IOC to enable the operator to configure the power supply information, including source of power, its usage schedules and outage information for the network elements and network functions. The MDA assisted Energy Saving use case can be extended to utilize this information along with network analytics to provide recommendations on the network service operability levels for different time periods. The network service operability levels can then be applied to the network to save energy.

This solution is feasible by providing attributes for the operator to configure the power shortage information and using MDA for providing the recommendations.

End of change