3GPP TSG-SA5 Meeting #157	S5-245635

Hyderabad, India, 14 - 18 October 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0611 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 28.552 Correction of performance monitoring for UDM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The naming of the operations for UDM is not in line with the manes in TS 23.502, and the references to the TS 23.502 and TS 29.503 are inconsistent. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct service operation naming to get, retrieval, and subscribe. The references to TS 23.502 and TS 29.503 are corrected with the clause, to make it clearer what is defined. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistency in the specifications can lead to different interpretations and by that also interoperability issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.8.1, 5.6.8.1.1, 5.6.8.1.2, 5.6.8.1.3, 5.6.8.2.1, 5.6.8.2.2, 5.6.8.2.3, and 5.6.8.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

#### 5.6.8.1	Subscription data get

##### 5.6.8.1.1	Number of subscription data get requests

a)	This measurement provides the number of subscription data get requests received by the UDM.

b)	CC

c)	Receipt of an Nudm_SDM_Get service operation request by the UDM from a consumer NF (e.g., AMF). Each message increments the relevant sub-counter, per subscriber data type, by 1 (see 3GPP TS 23.502 [7] clause 5.2.3.3).

d)	An integer value

e)	SDM.GetReq.Type,
Where Type indicates the subscription data type (see 3GPP TS 29.503 [51]) clause 5.2.2.2).

f)	UDMFunction

g)	Valid for packet switched traffic

h)	5GS

Second change

##### 5.6.8.1.2	Number of successful subscription data retrieval

a)	This measurement provides the number of successful subscription data retrieval at UDM.

b)	CC

c)	Transmission of an Nudm_SDM_Get service operation response by the UDM to a consumer NF (e.g., AMF) indicating a successful subscription data retrieval. Each message increments the relevant sub-counter, per subscriber data type, by 1 (see 3GPP TS 23.502 [7] clause 5.2.3.3).

d)	An integer value

e)	SDM.GetSucc.Type,
Where Type indicates the subscription data type (see 3GPP TS 29.503 [51]) clause 5.2.2.2).

f)	UDMFunction

g)	Valid for packet switched traffic

h)	5GS

Third change

##### 5.6.8.1.3	Number of failed subscription data retrieval

a)	This measurement provides the number of failed subscription data retrieval at UDM.

b)	CC

c)	Transmission of an Nudm_SDM_Get service operation response by the UDM to a consumer NF (e.g., AMF) indicating a failed subscription data retrieval. Each message increments the relevant sub-counter, per failure case, by 1 (see 3GPP TS 23.502 [7] clause 5.2.3.3).

d)	An integer value

e)	SDM.GetFail.Cause,
Where Cause indicates the failure cause of the subscription data retrieval.

f)	UDMFunction

g)	Valid for packet switched traffic

h)	5GS

Fourth change

##### 5.6.8.2.1	Number of SDM subscribe requests

a)	This measurement provides the number of SDM subscribing requests received by the UDM.

b)	CC

c)	Receipt of an Nudm_SDM_Subscribe service operation request by the UDM from a consumer NF (e.g., AMF). Each message increments the relevant sub-counter, per subscriber data type, by 1 (see 3GPP TS 23.502 [7] clause 5.2.3.3).

d)	An integer value

e)	SDM.SubscribeReq.Type,
Where Type indicates the subscription data type (see 3GPP TS 29.503 [51]) clause 5.2.2.2).

f)	UDMFunction

g)	Valid for packet switched traffic

h)	5GS

Fifth change

##### 5.6.8.2.2	Number of successful SDM subscriptions

a)	This measurement provides the number of successful SDM subscriptions at UDM.

b)	CC

c)	Transmission of an Nudm_SDM_Subscribe service operation response by the UDM to a consumer NF (e.g., AMF) indicating a successful SDM subscription. Each message increments the relevant sub-counter, per subscriber data type, by 1 (see 3GPP TS 23.502 [7] clause 5.2.3.3).

d)	An integer value

e)	SDM.SubscribeSucc.Type,
Where Type indicates the subscription data type (see 3GPP TS 29.503 [51]) clause 5.2.2.2).

f)	UDMFunction

g)	Valid for packet switched traffic

h)	5GS

Sixth change

##### 5.6.8.2.3	Number of failed SDM subscriptions

a)	This measurement provides the number of failed SDM subscriptions at UDM.

b)	CC

c)	Transmission of an Nudm_SDM_Subscribe service operation response by the UDM to a consumer NF (e.g., AMF) indicating a failed SDM subscriptions. Each message increments the relevant sub-counter, per failure case, by 1 (see 3GPP TS 23.502 [7] clause 5.2.3.3).

d)	An integer value

e)	SDM.SubscribeFail.Cause,
Where Cause indicates the failure cause of the SDM subscribing.

f)	UDMFunction

g)	Valid for packet switched traffic

h)	5GS

Seventh change

##### 5.6.8.3.1	Number of subscription data notifications

a)	This measurement provides the number of subscription data notifications sent by the UDM.

b)	CC

c)	Transmission of an Nudm_SDM_Notification service operation by the UDM to a consumer NF (e.g., AMF). Each message increments the relevant sub-counter, per subscriber data type, by 1 (see TS 23.502 [7] clause 5.2.3.3).

d)	An integer value

e)	SDM.SubDataNotif.Type,
Where Type indicates the subscription data type (see 3GPP TS 29.503 [51]) clause 5.2.2.2).

f)	UDMFunction

g)	Valid for packet switched traffic

h)	5GS

End of changes