3GPP TSG-SA5 Meeting #157	S5-245636

Hyderabad, India, 14 - 18 October 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.554 | CR | 0207 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 28.554 Energy consumption per node type for NwSlice |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph3 |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The network slice energy comsumption is not clearly defined for 5GC NFs shared between network slices. The measurements decribed cannot be reused in other energy efficiency calculations since they are only described as part of the network slice energy consumpotion. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of separate defintions in regards to 5GC NFs gNB, AMF, SMF, UPF, and PSA UPF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The defintions of 5GC NFs shared between network slices will be unclear, and not usable in energy efficiency calculations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.3.3, 6.7.3.3.x1 (new) , 6.7.3.3.x2 (new) , 6.7.3.3.x3 (new) , 6.7.3.3.x4 (new) , 6.7.3.3.x5 (new) , and 6.7.3.3.x6 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

#### 6.7.3.3	Network Slice Energy Consumption (EC)

a) ECns

b) This KPI describes the Energy Consumption (EC) of the network slice. It is obtained by summing up the Energy Consumption of all the Network Functions (ECNF) that compose the network slice. The unit of this KPI is J.

c)

ECns= NFECNF+ECns,gNB+ECns,AMF+ECns,SMF+ECns,UPF+ECns,PsaUpf

Where ECNF are 5GC NFs dedicated to the network slice, other ECs represent 5GC NFs (i.e., gNB, AMF, SMF, UPF, and PSA UPF) shared between network slices. The case of other 5GC NFs shared between network slices is not addressed in the present document. Add some text around adding the assumed EC for other NF.

As a network slice may be composed of a RAN network slice subnet, a Transport Network (TN) network slice subnet and a 5GC network slice subnet, they all participate to the energy consumption of the network slice. However, the definition and way to measure the energy consumption of the TN segment is not in the scope of the present document.

d) NetworkSlice

Second change

##### 6.7.3.3.x1	Network Slice gNB Energy Consumption (EC)

a)	ECns,gNB

b)	This KPI describes the Energy Consumption (EC) of gNBs in the network slice. It is obtained by summing up the Energy Consumption of all the gNB (ECgNB), that compose the network slice. The calculation of ECgNB is defined in clause 6.7.3.4.2, For non-split gNB clause 5.1.1.19.3 (PNF Energy consumption) of TS 28.552 [6] applies, and the measurement is obtained according to the method defined in ETSI ES 202 336-12 [10] – clauses 4.4.3.1, 4.4.3.4, Annex A. The energy consumption attributable to each network slice is estimated as a proportion of the total gNB energy consumption, where the proportion is calculated as the data volume of the network slice relatively to the total data volume carried by the gNB. The unit of this KPI is J.

c)

ECns,gNB=gNBDRB.PdcpSduVolumeUl,SNSSAI+DRB.PdcpSduVolumeDl.SNSSAIDRB.PdcpSduVolumeUl+DRB.PdcpSduVolumeDl ×ECgNB

-	The case of split gNB shared between network slices is not addressed in the present document.

d) NetworkSlice

Third change

##### 6.7.3.3.x2	Network Slice AMF Energy Consumption (EC)

a) ECns,AMF

b) This KPI describes the Energy Consumption (EC) of AMFs in the network slice. It is obtained by summing up the Energy Consumption of all the AMF (ECAMF), that compose the network slice. The ECAMF can be calculated based on the ECNF in clause 6.7.3.1 for the NF of type AMF. The energy consumption attributable to each network slice is estimated as a proportion of the total estimated AMF energy consumption, where the proportion is calculated as the mean number of registered subscribers of the network slice relatively to the overall mean number of registered subscribers of the AMF during the same time period (see TS 28.552 [6] clause 5.2.1.1 for the definition of the mean number of registered subscribers). The unit of this KPI is J.

c)

ECns,AMF=AMFRM.RegisteredSubNbrMean.SNSSAIRM.RegisteredSubNbrMean ×ECAMF

d) NetworkSlice

Fourth change

##### 6.7.3.3.x3	Network Slice SMF Energy Consumption (EC)

a) ECns,SMF

b) This KPI describes the Energy Consumption (EC) of SMFs in the network slice. It is obtained by summing up the Energy Consumption of all the SMF (ECSMF), that compose the network slice. The ECSMF can be calculated based on the ECNF in clause 6.7.3.1 for the NF of type SMF. The energy consumption attributable to each network slice is estimated as a proportion of the total estimated SMF energy consumption, where the proportion is calculated as the mean number of PDU sessions of the network slice relatively to the overall mean number of PDU sessions of the SMF during the same time period (see TS 28.552 [6] clause 5.3.1.1 for the definition of the mean number of PDU sessions). The unit of this KPI is J.

c)

ECns,SMF=SMFSM.SessionNbrMean.SNSSAISM.SessionNbrMean×ECSMF

d) NetworkSlice

Fifth change

##### 6.7.3.3.x4	Network Slice UPF Energy Consumption (EC)

a) ECns,UPF

b) This KPI describes the Energy Consumption (EC) of one UPFs in the network slice. It is obtained by summing up the Energy Consumption of all the UPF (ECUPF), that compose the network slice. The ECUPF can be calculated based on the ECNF in clause 6.7.3.1 for the NF of type UPF. The energy consumption attributable to each network slice is estimated as a proportion of the total estimated UPF energy consumption, where the proportion is calculated as the data volume of the network slice relatively to the overall data volume of the UPF during the same time period. The data volume of the UPF is obtained by summing up, for all N3 interface(s), the number of octets of incoming GTP data packets on the N3 interface, from (R)AN to UPF (see TS 28.552 [6] clause 5.4.1.3) and the number of octets of outgoing GTP data packets on the N3 interface, from UPF to (R)AN (see TS 28.552 [6] clause 5.4.1.4). The unit of this KPI is J.

c)

ECns,UPF=UPFGTP.InDataOctN3UPF.SNSSAI+GTP.OutDataOctN3UPF.SNSSAIGTP.InDataOctN3UPF+GTP.OutDataOctN3UPF×ECUPF

d) NetworkSlice

Sixth change

##### 6.7.3.3.x5	Network Slice PSA UPF Energy Consumption (EC)

a) ECns,PsaUpf

b) This KPI describes the Energy Consumption (EC) of PSA UPFs in the network slice. It is obtained by summing up the Energy Consumption of all the UPF (ECUPF), that compose the network slice. The ECPsaUpf can be calculated based on the ECNF in clause 6.7.3.1 for the NF of type PSA UPF. The energy consumption attributable to each network slice is estimated as a proportion of the total estimated PSA UPF energy consumption, where the proportion is calculated as the data volume of the network slice relatively to the overall data volume of the PSA UPF during the same time period. The data volume of the PSA UPF is obtained by summing up, for all N9 interface(s), the number of octets of incoming GTP data packets on the N9 interface for PSA UPF (see TS 28.552 [6] clause 5.4.4.2.3) and the number of octets of outgoing GTP data packets on the N9 interface for PSA UPF (see TS 28.552 [6] clause 5.4.4.2.4). The unit of this KPI is J.

c)

ECns,PsaUpf=PsaUpfGTP.InDataOctN9PsaUpf.SNSSAI+GTP.OutDataOctN9PsaUpf.SNSSAIGTP.InDataOctN9PsaUpf+GTP.OutDataOctN9PsaUpf×ECPsaUpf

d) NetworkSlice

End of changes