3GPP TSG-SA5 Meeting #156	S5-245640

Hyderabad, India, 14 - 18 October 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.274 | CR | 0089 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR 32.274 Correction of references for SMS information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18, 5GS_Ph1-SMSCH |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Faulty references for SMS Charging Information, RAT Type, and SMSC Address. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correcting references for SMS Charging Information, RAT Type, and SMSC Address. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect references may lead to different interpretations and by that interoperability issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2a.3.2 and 6.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

#### 6.2a.3.2	SMS charging CHF CDR data

If enabled, CHF CDRs for SMS charging shall be produced for SMS chargeable events.

The fields of SMS charging CHF CDR are specified in table 6.2a.3.2.1.

Table 6.2a.3.2.1: SMS charging CHF record data

| Field | Category | Description |
| --- | --- | --- |
| Record Type | M | Described in TS 32.298 [3] |
| Recording Network Function ID | OM | Described in TS 32.298 [3] |
| Subscriber Identifier | M | This field holds the 5G Subscription Permanent Identifier (SUPI) of the served party. |
| NF Consumer Information | M | This field holds the information of the SMSF that used the charging service. |
| NF Functionality | M | This field contains the function of the node (i.e. SMSF) |
| NF Name | OC | This field holds the name of the SMSF used. |
| NF Address | OC | This fields holds the IP Address of the SMSF used. |
| NF PLMN ID | Oc | This field holds the PLMN identifier (MCC MNC) of the SMSF. |
| Triggers | - | This field is not applicable. |
| List of Multiple Unit Usage | OM | This field is present when the number of units is beyond one (i.e. more than one SMS) |
| Record Opening Time | OC | Described in TS 32.298 [3] |
| Duration | M | Described in TS 32.298 [3] |
| Record Sequence Number | C | Described in TS 32.298 [3] |
| Cause for Record Closing | M | Described in TS 32.298 [3] |
| Local Record Sequence Number | OM | Described in TS 32.298 [3] |
| Record Extensions | OC | Described in TS 32.298 [3] |
| SMS Charging Information | OM | This field holds the SMS specific information defined in clause 6.5.2 |

Second changes

### 6.5.2	Definition of SMS charging information

SMS specific charging information used for SMS converged charging is provided within the SMS charging Information.

# Table 6.5.2.1: Structure of SMS Charging information

| Information Element |  | Category |  | Description |  |
| --- | --- | --- | --- | --- | --- |
| Originator Info |  | OM |  | This field is a grouped field and holds information on originator of the SMS |  |
| Originator SUPI |  | OM |  | This field holds the SUPI of the originator of the SMS, if available. This field is present if different from subscriber identifier field. |  |
| Originator GPSI |  | OC |  | This field holds the GPSI of the originator of the SMS, if available. |  |
| Originator Other Address |  | OM |  | This field holds the address of the originator of the SMS, when different from SUPI and GPSI, if available: e.g. email, short code. This field may have multiple occurrences. |  |
| Originator Received Address |  | OC |  | Described in table 6.3.1.2.1 |  |
| Originator SCCP Address |  | OC |  | Described in table 6.3.1.2.1 |  |
| SM Originator Interface |  | OM |  | Described in table 6.3.1.2.1 |  |
| SM Originator Protocol Id |  | OC |  | Described in table 6.3.1.2.1: SM Protocol Id information element |  |
| Recipient Info |  | OC |  | Described in table 6.3.1.2.1 |  |
| Recipient SUPI |  | OM |  | This field holds the SUPI of the recipient of the SMS, if available. This field is present if different from subscriber identifier field. |  |
| Recipient GPSI |  | OC |  | This field holds the GPSI of the recipient of the SMS, if available. |  |
| Recipient Other Address |  | OC |  | This field holds the address of the recipient of the SMS, when different from SUPI and GPSI, if available: e.g. email, short code. This field may have multiple occurrences |  |
| Recipient Received Address |  | OC |  | Described in table 6.3.1.2.1 |  |
| Recipient SCCP Address |  | OC |  | Described in table 6.3.1.2.1 |  |
| SM Destination Interface |  | OM |  | Described in table 6.3.1.2.1 |  |
| SM Recipient Protocol Id |  | OC |  | Described in table 6.3.1.2.1: SM Protocol Id information element. |  |
| User Equipment Info |  | OC |  | This field holds the identification of the terminal (i.e. PEI, MAC Address) used by the UE the SMS transaction, if available. |  |
| Roamer In Out |  | OC |  | This field holds an indication of the UE is an in-bound roamer. This field is present only if UE is identified as a roamer. |  |
| User Location Info |  | OC |  | Described in table 6.3.1.1.1 |  |
| UE Time Zone |  | OC |  | Described in table 6.3.1.1.1 |  |
| RAT Type |  | OC |  | Described in table 6.3.1.1.1 |  |
| SMSC Address |  | OM |  | Described in table 6.3.1.2.1 |  |
| SM Data Coding Scheme |  | OM |  | Described in table 6.3.1.2.1 |  |
| SM Message Type |  | OM |  | Described in table 6.3.1.2.1 |  |
| SM Reply Path Requested |  | OC |  | Described in table 6.3.1.2.1 |  |
| SM User Data Header |  | OC |  | Described in table 6.3.1.2.1 |  |
| SM Status |  | OC |  | Described in table 6.3.1.2.1 |  |
| SM Discharge Time |  | OC |  | Described in table 6.3.1.2.1 |  |
| Number of Messages Sent |  | OC |  | Described in table 6.3.1.2.1 |  |
| SM Service Type |  | OC |  | Described in table 6.3.1.2.1 |  |
| SM Sequence Number |  | OC |  | Described in table 6.3.1.2.1 |  |
| SMS result |  | C |  | Described in table 6.3.1.2.1 |  |
| Submission Time |  | OC |  | Described in table 6.3.1.1.1 |  |
| SM Priority |  | OC |  | Described in table 6.3.1.1.1 |  |
| Message Reference |  | OM |  | Described in table 6.3.1.1.1 |  |
| Message Size |  | OM |  | Described in table 6.3.1.1.1 |  |
| Message Class |  | OM |  | Described in table 6.3.1.1.1 |  |
| Delivery Report Requested |  | OC |  | Described in table 6.3.1.1.1 |  |

End of changes