3GPP TSG-SA5 Meeting #157	S5-245707

Hyderabad, India, 14 - 18 October 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.422 | CR | 0498 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 32.422 Corrections to RRC reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TraceQoE_OAM |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The VisitedCellList is part of the Mobility History Information (MHI) report, and does not to be handled explicitly using the UE History Information (UHI) report. Incorrect figure 4.11.1.1 clarified. References to 38.331 are unclear or missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove references to VisitedCellList and UHI. Figure 4.11.1.1 clarified to include configuratoin for the logging of SHR and SPR. References to TS 38.331 improved. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecessary function clutters specification and leads to unnecessary implementattion. Unclear specification leading to interoperability problems. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 4.11.1, 5.14, 10.6, Annex C.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS 28.622 CR 0477 |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [4], TS 32.101 [1] , TS 23.501 [40], TS 38.300 [42] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [4].

AS	Application Server

BGCF	Breakout Gateway Control Function

CAG	Closed Access Group

CSCF	Call Session Control Function

I-CSCF	Interrogating-CSCF

IM CN SS	IP Multimedia Core Network Subsystem

IMEI-TAC	IMEI Type Allocation Code

MHI	Mobility History Information

MN	Main Node

NID	Network ID

P-CSCF	Proxy – CSCF

PNI-NPN	Public Network Integrated Non-Public Network

RCEF	RRC Connection Establishment Failure

RLF	Radio Link Failure

S-CSCF	Serving-CSCF

SHR	Successful Handover Report

SN	Secondary Node

SNPN	Stand-alone Non-Public Network

SPR	Successful PSCell Addition/Change Report

TAU	Tracking Area Update

TRSR	Trace Recording Session Reference

TR	Trace Reference

Next change

### 4.11.1	Trace session activation for RRC reporting in NG-RAN

RRC reporting is activated to the gNB as a special Trace Session where the Job Type parameter indicates RRC report, and the RRC Report Type parameter indicates which RRC reports should be traced. The detailed procedure is shown in figure 4.11.1.1 where one UE experiences an event which results in a RRC report being generated and traced.

Figure 4.11.1.1 Example scenario for RRC reporting when UE generates a RRC report.

Upon Trace Session activation indicating RRC reporting, the gNB shall start a Trace Session. This Trace Session shall collect one or more RRC reports received from the UE.

Next change

## 5.14	RRC Report Type (CM)

The RRC Report Type parameter is a conditionally mandatory parameter. It shall be present when RRC Reporting is supported.

The RRC Report Type defines which of the possible RRC reports that are requested. These reports are based on IEs specified in 3GPP TS 38.331 [43]:

•	RLF report, provided by the IE rlf-Report-r16 in message UEInformationResponse.

•	RCEF report, provided by the IE connEstFailReportList-r17 in message UEInformationResponse.

•	SHR, provided by the IE successHO-Report-r17 in message UEInformationResponse.

•	SPR, provided by the IE successPSCell-Report-r18 in message UEInformationResponse.

•	MHI, provided by the IE mobilityHistoryReport-r16 in message UEInformationResponse.

•	RA Report, provided by the IE ra-ReportList-r16 in message UEInformationResponse.

Next change

## 10.6	RRC report

The measurement names for the RRC report consists of the following names:

- The first item identifies the RRC job type, i.e. "rrcReport".

- The second item identifies the kind of RRC Report i.e. one of "rlfReport", "rcefReport", "shr", "spr", "mhi", or "raReport".

Possible examples:

- rrcReport.rlfReport

- rrcReport.rcefReport

- rrcReport.shr

- rrcReport.spr

- rrcReport.mhi

- rrcReport.raReport

Next change

Annex C (informative):	Plant UML code

# Annex C.1	Code for figure 4.11.1.1

@startuml

hide footbox

participant "Management System:\nProvisioning MnS consumer"  as prov #white

participant "gNB:\nProvisioning MnS producer"  as gnb #white

participant "UE" #white

participant "Management System:\nTCE" as tce #white

participant "Management\nsystem:\nMF" as mf #white

participant "Management System:\nStreaming data reporting\nMnS consumer" as data #white

prov -> gnb :createMOI(Tracejob)

gnb -> gnb : Start trace job

alt Configure UE to generate specific reports

gnb -> UE: RRCReconfiguration(flags)

end

note right: Flags needed for UE to generate SHR and/or SPR.

'note over gnb,UE: Active connection

'note over UE: RRC Report generated

ref over gnb,UE: Active connection

ref over UE: Report generated

UE -> gnb :RRCSetupComplete or\nRRCReconfigurationComplete or\nRRCReestablishmentComplete or\nRRCResumeComplete\n ...with report availability indication

gnb -> UE: UEInformationRequest

UE -> gnb: UEInformationResponse

gnb -> gnb: Store RRC report in Trace record

alt File base reporting

gnb -> tce: Trace data file

else Streaming recording

alt Direct streaming

gnb -> data: Streaming trace record

else Relayed streaming

gnb -> mf: Streaming trace record

mf -> data: Streaming trace record

end

end

@enduml

End of changes