3GPP TSG-SA5 Meeting #157	S5-245732

Hyderabad, India, 14 - 18 October 2024

# Source:                         NTT DOCOMO, China Mobile

# Title:	pCR 28.869 VNF traffic mgmt solution and evaluation

# Document for:	Approval

Agenda Item:	6.19.6

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

- 3GPP TR 28.869 v1.0.1 Study on cloud aspects of management and orchestration.

# 3	Rationale

The contribution proposes to add evaluation of solutions for Cloud-native VNF traffic management.

# 4	Detailed proposal

It proposes to make the following changes to TR 28.869 [1].

1st Change

# 5	Use cases, potential requirements, and potential solutions

## 5.1	Use of VNF generic OAM functions

Editor's Note: This clause describes the use cases, issues, requirements, and solutions related to WT-1.

### 5.1.3	Use case #3: Cloud-native VNF Traffic management

#### 5.1.3.1	Description

Effective traffic management for cloud-native functions is essential to ensure high quality of service levels. Nevertheless, especially in containerized deployments additional challenges need to be considered since many OS containers realizing the cloud-native VNF instances are usually deployed per host, while OS containers are often created and destroyed rapidly, requiring the network to adapt quickly to topological changes. In the context of a cloud-native VNF, traffic management includes controlling the inbound/outbound traffic to, from and within the cloud-native VNF instance.

Traffic management actions to be considered in a 3GPP management system context are, for example, controlling the rate of incoming requests to prevent overloading services and directing traffic to different VNFCs realizing a cloud-native VNF. These actions can surge in the context of diverse OAM procedures of maintenance, re-configuration, and upgrade of NFs, etc. managed through the 3GPP management system.

The 3GPP management system needs to be able to support an operator to manage and orchestrate the traffic management actions for cloud-native VNF instances.

#### 5.1.3.2 	Potential requirements

REQ-CVNF_TM-1 The 3GPP management system should have the capability to support traffic management of cloud-native VNF instances.

REQ-CVNF_TM-2 The reference point between 3GPP management system and external OAM entity should have the capability enabling the 3GPP management system to interact with external (non-3GPP) traffic management entities for the purpose of performing traffic management for cloud-native VNF instances.

#### 5.1.3.3	Potential solutions

##### 5.1.3.3.1	Traffic Enforcer function

As shown in figure 5.1.3.3.1-1, this solution introduces a platform entity that interacts with 3GPP management system for traffic management of cloud-native VNFs via a new PaaS reference point.

Figure 5.1.3.3.1-1: Traffic management of cloud native VNF

The solution proposes using Traffic Enforcer function defined in ETSI GS ISG NFV-IFA 049 [2], which is one of the VNF generic OAM functions. Some key functionalities supported by the Traffic Enforcer function are the capability to perform the required traffic blocking and rerouting operations on the VNFC instances.

According to ETSI GS ISG NFV-IFA 049 [2] Traffic Enforcer functionality can be called by functions residing inside the 3GPP management system or other VNF generic OAM functions (e.g. the Upgrade VNF function) or other PaaS Services (e.g. the Policy Agent).

The present solution addresses the potential requirement REQ-CVNF_TM-2.

##### 5.1.3.3.2	3GPP Management System to support traffic management of cloud-native VNF instances

In TS 28.622, it is specified that the 3GPP management system can set a managed object’s operational state as ENABLED which means the managed object is operable, and DISABLED which means the managed object is inoperable. As NFs can be managed objects of 3GPP management systems, their operational state can be configured by the 3GPP system as part of management workflow. If the operational state of an NF (which in turn can be realized as a cloud-native VNF) is DISABLED, for example during fault management, then 3GPP management system can determine the need to block any traffic to/from the NF. Similarly, if the operational state of an NF is ENABLED, then 3GPP management system can decide to enable traffic to/from the NF.

Furthermore, in TS 28.622, the EP_RP  IOC is specified in order to represent an end point of a communication link or a reference between two network entities, while in TS 28.541, the end points between specific entities are specified with dedicated EP_<rp>  IOCs where <rp> is a string that represents the name of the reference point.  (i.e. EP_F1C represents control plane interface between gNB-DU and gNB-CU or gNB-CU-CP ). If an MOI of an EP_<rp> associated to the link between two network functions is removed, this can also imply that there is no more connectivity/link between these network functions so that 3GPP management system can decide to block the traffic over this link.

Therefore, current referenced 3GPP specifications already satisfy REQ-CVNF_TM-1.

2nd Change

#### 5.1.3.4	Evaluation of solutions

The potential solution described in clause 5.1.3.3 supports the capability to perform traffic management for cloud-native VNFs. It introduces a PaaS reference point and a related platform entity. The advantages of this solution are:

-	Provides the capabilities to perform traffic management beyond the cloud-native VNF, i.e., traffic management can be performed before reaching the cloud-native VNF or after leaving the cloud-native VNF alleviating the cloud-native VNF itself to process such traffic rules.

-	Provides the capability to isolate, block or reroute traffic in cases where actual cloud-native VNF might be unresponsive or be under maintenance.

The new platform entity resides outside the 3GPP management system, and the potential solution is compatible, without introducing any major impact, to the 3GPP management system framework.

In addition, in clause 5.1.3.3.2, it is described how 3GPP management system supports capabilities, leveraging the existing 3GPP specifications, that can have a direct relationship to traffic management actions to be realized on cloud-native VNF instances.

Considering the potential solutions provided in clauses 5.1.3.3.1 and 5.1.3.3.2, both solutions together satisfy all potential requirements of this use-case and are feasible solutions.

End of Changes