3GPP TSG-SA5 Meeting #157	S5-245773

Hyderabad, India, 14 - 18 October 2024

# Source:                         China Mobile

# Title:	pCR Add Potential solution for WT-3 Placement of cloud native NFs

# Document for:	Approval

Agenda Item:	6.19.6

1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

2	References

- 3GPP TR 28.869 v1.0.1 Study on cloud aspects of management and orchestration.

3	Rationale

The contribution proposes to add Potential solution for WT-3 Placement of cloud native NFs.

4	Detailed proposal

It proposes to make the following changes to TR 28.869 [1].

1st Change

5.3	Support of different cloud deployment scenarios

Editor's Note: This clause describes the use cases, issues, requirements, and solutions related to WT-3.

5.3.1	Use case #1: Placement of cloud native NFs

5.3.3.1	Description

The distributed cloud deployment types enable the network to be deployed across different geographical locations. In this use case CSPs use the information about the available placement locations and resources in each, if available, to select the optimal location(s) for an NF. This provides the NOPs/CSPs with the choice of where a particular NF can be placed at the time of deployment. The parameters that can impact the choice of placement include geographical service area, performance in terms of latency and available bandwidth, as well as level of deployment complexity. In order to support distributed cloud deployments, the 3GPP management system needs to provide the capability for NOPs/CSPs to learn the available deployment locations.

NOTE	:The mechanism to indicate the preferred placement of a particular NF is already supported by use of attribute 'locality' in the ManagedNfProfile datatype.

5.3.3.2	Potential requirements

REQ-1 The 3GPP management system should be able to collect information about the available deployment locations.

NOTE:	The granularity of the location information is FFS.

5.3.3.3	Potential solutions

As shown in figure 5.3.3.3-1, in this solution the 3GPP management system interacts with an orchestration and management entity using the xyz reference point for placement of cloud native NFs.

Figure 5.3.3.3-1: High level procedure for placement of cloud native NFs

The 3GPP management requests placement of cloud native NFs from an orchestration and management entity over the xyz reference point. The 3GPP management system can collect information about the available deployment locations from different clouds, and can specify a deployment location for the cloud native NFs.

If the relevant placement request is sented to MANO through the xyz interface, MANO already has this capability of support different cloud deployment.

5.1.2.4	Evaluation of solutions

The solution is a feasible high level solution to be developed further in the normative specifications.

End of Changes