3GPP TSG-SA5 Meeting #157	S5-245818

Hyderabad, India, 14 - 18 October 2024

# Source:	Nokia

# Title:	Rel-19 pCR TR 28.879 Clean up to clarify the roles of the MnS producer vs the MnF and differentiates between discovery info configuration and authorization

# Document for:	Approval

Agenda Item:	6.19.21

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

[1] 3GPP TR 28.879, " Study on OAM for service management and exposure to external consumers".

# 3	Rationale

This pCR proposes a clean-up in TR 28.879[1] to differentiate between what functionalities are provided by the MnS producer and which are provided by the management function when exposing management services to an external MnS consumers. In addition, this pCR make modifications to differentiate between the configuration of the discovery information at the CCF and the authorization to consume service APIs or management services at the AEF.

# 4	Detailed proposal

It is proposed that the following changes be made to clause 5.1 of TR 28.879 [1].

Begin Change

## 5.1	Exposure of management services

### 5.1.1	Use case #1:  MnS producer registration into CAPIF

#### 5.1.1.1	Description

For a MnS to be made available for consumption through an exposure framework, it is needed that the MnS producer is registered into the exposure framework. Once registered, the MnF hosting the MnS producer is able to communicate with the CCF for the subsequent procedures needed to configure accessibility to MnS from external MnS consumers. These procedures include publication, discovery and access control, among others.

In the CAPIF framework, the scope of registration is limited to API provider domain functions, which include API Exposing Function (AEF), API Publishing Function (APF) and API Management Function (AMF). The registration is a procedure whereby these functions become recognized users of the CCF. This procedure is described in clause 8.28 of TS 23.222 [5], with stage 3 solution set detailed in clause 8.9 of TS 29.222 [13].

To register (and/or update the registration information of) an API provider domain on the CCF, the AMF from this API provider domain communicates with the CCF over CAPIF-5 interface. This means that:

-	This API provider domain shall have an AMF.

-	The AMF of this API provider domain shall be an authorized user of CCF.

-	The AMF security information for CCF to validate the registration request is provisioned by the CAPIF administrator.

To register a MnS producer on the CCF, it is needed for the MnF hosting the MnS producer to support the API provider domain functionality, as detailed below:

-	AEF. If supported, this means that the MnF hosting the MnS producer will need to support:

-	CAPIF-2/2e interface, so that the API invokers acting as external MnS consumers can access service APIs, when required. The API operations that will be implemented in this interface (see TS 29.222 [13], clause 9.1) can be configured later, depending on the intended usage of MnS producer.

NOTE: The service APIs can carry different MnS information for different API invokers. This issue is discussed in use case #3 (clause 5.1.3).

-	CAPIF-3 interface, so that the MnF hosting the MnS producer can communicate with CCF to exercise access and policy related control for service API invocations initiated by the API invoker. The API operations that will be implemented on this interface (see TS 29.222 [13], clauses 8.3, 8.5, 8.7, 8.6) can be configured later, depending on the intended usage of the MnS.

-	APF. If supported, this means that the MnF hosting the MnS producer will need to support the CAPIF-4 interface, so that it can communicate with the CCF to publish (and manage the published) MnS information. The API operations that will be implemented in this interface (see TS 29.222 [13], clause 8.2) can be configured later, depending on the intended usage of MnS.

-	AMF. If supported, this means that the MnF hosting the MnS producer will need to support CAPIF-5 interface. The API operations that will be implemented in this interface (see TS 29.222 [13], clause 8.3 and 8.9) can be configured later, depending on the intended usage of MnS.

The MnF hosting the MnS producer shall support all the API provider domain functions.

The MnF hosting the MnS producer playing the role of the AMF registers itself to the CCF using CAPIF_API_Provider_Management_API (see TS 29.222 [13], clause 8.9). Upon reception of these details, the CCF registers the MnF hosting the MnS producer with the supported API provider domain function(s).

#### 5.1.1.2	Potential requirements

PREQ-FS_MExpo-Reg-01: The 3GPP management system shall provide the capability to register the MnF hosting the MnS producer on the CCF.

PREQ-FS_MExpo-Reg-02: The 3GPP management system shall provide the capability to deregister the MnF hosting the MnS producer on the CCF.

PREQ-FS_MExpo-Reg-03: The 3GPP management system shall provide the capability to update the registration details of the MnF hosting the MnS producer on the CCF.

NOTE: The specification of the MnF hosting the MnS producer is out of scope of the present document.

#### 5.1.1.3	Potential solutions

##### 5.1.1.3.1	Potential solution #1: Capturing the MnF hosting the MnS Producer registration information with APIProviderEnrolmentDetails

###### 5.1.1.3.1.1	Introduction

This solution focuses on meeting the requirements PREQ-FS_MExpo-Reg-01, PREQ-FS_MExpo-Reg-02 and PREQ-FS_MExpo-Reg-03.

In the CAPIF, the registration procedure is executed with the CAPIF_API_Provider_Management API (see TS 29.222 [13], clause 8.3), initiated by the AMF over the CAPIF-5 interface. The AMF sends a HTTP POST message to the CCF with a request body containing the following dataType: “APIProviderEnrolmentDetails” (see TS 29.222 [13], clause 8.9.4.2.2).

The solution proposes to capture the MnF hosting the MnS producer information with “APIProviderEnrolmentDetails” and reuse the CAPIF_API_Provider_Management API to register, de-register and update registration information related to the MnF hosting the MnS producer.

###### 5.1.1.3.1.2	Description

How the MnF hosting the MnS producer information relevant for registration can be captured with “APIProviderEnrolmentDetails” is described in tables 5.1.1.3.1.2-1, 5.1.1.3.1.2-2, 5.1.1.3.1.2-3, and 5.1.1.3.1.2-4.

Table 5.1.1.3.1.2-1: Representing the MnF registration information with APIProviderEnrolmentDetails attributes (extract Table 8.9.4.2.2-1 in TS 29.222 [13]).

| Attribute name | Data type | P | Cardinality | Description | Comments |
| --- | --- | --- | --- | --- | --- |
| apiProvDomId | string | O | 0..1 | API provider domain ID assigned by the CAPIF core function to the API management function while registering the API provider domain. Shall not be present in the HTTP POST request from the API management function to the CAPIF core function, to on-board itself. Shall be present in all other HTTP requests and responses. |  |
| regSec | string | M | 1 | Security information necessary for the CAPIF core function to validate the registration of the API provider domain. Shall be present in HTTP request from API management function to CAPIF core function for API provider domain registration. | It can be used to store the credentials of the MnF. |
| apiProvFuncs | array(APIProviderFunctionDetails) | O | 1..N | A list of individual API provider domain functions details. When included by the API management function in the HTTP request message, it lists the API provider domain functions that the API management function intends to register/update in registration or update registration procedure. When included by the CAPIF core function in the HTTP response message, it lists the API domain functions details that are registered or updated successfully. | See Table 5.1.1.3.1.2-2 |
| apiProvDomInfo | string | O | 0..1 | Generic information related to the API provider domain such as details of the API provider applications. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Used to negotiate the supported optional features of the API as described in clause 7.8. This attribute shall be provided in the HTTP POST request and in the response of successful resource creation. | Only applicable when API provider is SCEF/NEF, and thus not applicable when API provider is the MnF. |
| failReason | string | C | 0..1 | Registration or update specific failure information of failed API provider domain function registrations. Shall be present in the HTTP response body if atleast one of the API provider domain function registration or update registration fails. |  |
| apiProvName | string | O | 0..1 | Represents the API provider name. | RNAA |

Table 5.1.1.3.1.2-2: Representing MnS producer registration information with APIProvFuncs attributes (extract Table 8.9.4.2.3-1 in TS 29.222 [13]).

| Attribute name | Data type | P | Cardinality | Description | Comments |
| --- | --- | --- | --- | --- | --- |
| apiProvFuncId | string | C | 0..1 | API provider domain functionID assigned by the CAPIF core function to the API provider domain function while registering/updating the API provider domain. Shall not be present in the HTTP POST request from the API management function to the CAPIF core function, to register itself. Shall be present in all other HTTP requests and responses. |  |
| regInfo | RegistrationInformation | M | 1 | Information necessary for the CAPIF core function to register the API provider domain function. This information shall be present in HTTP POST/PUT request from API management function to CAPIF core function for API provider domain registration. In the HTTP response message from CAPIF core function, shall include the updated registration information for API provider domain function. | See Table 5.1.1.3.1.2-3 |
| apiProvFuncRole | ApiProviderFuncRole | M | 1 | Role of API provider domain function. The role shall be present in the HTTP POST/PUT request that the API management function intends to register/update the API provider domain function as. CAPIF core function shall register the role of API provider domain function as per the request. | See Table 5.1.1.3.1.2-4. This data type serves to specify, for the MnF to be registered, which CAPIF API provider domain function(s) will be supported. |
| apiProvFuncInfo | string | O | 0..1 | Generic information related to the API provider domain function such as details of the API provider applications. |  |

Table 5.1.1.3.1.2-3: Representing MnS producer registration information with RegistrationInformation attributes (extract Table 8.9.4.2.4-1 in TS 29.222 [13]).

| Attribute name | Data type | P | Cardinality | Description | Comments |
| --- | --- | --- | --- | --- | --- |
| apiProvPubKey | string | M | 1 | Public Key of API Provider domain function. | It can be used to store the public key of the MnF hosting the MnS producer. |
| apiProvCert | string | O | 0..1 | API provider domain function's generic client certificate | It can be used to store the client certificate of the MnF hosting the MnS producer, if existing. |

Table 5.1.1.3.1.2-4: Representing MnS producer registration information with ApiProviderFuncRole attributes (extract Table 8.9.4.3.3-1 in TS 29.222 [13]).

| Enumeration value | Description | Comments |
| --- | --- | --- |
| AEF | API provider function is API Exposing Function. |  |
| APF | API provider function is API Publishing Function. |  |
| AMF | API provider function is API Management Function. |  |

#### 5.1.1.4	Evaluation of potential solutions

Editor's Note:	This clause provides the evaluation of potential solutions.

##### 5.1.1.4.1	Evaluation of potential solution #1: Capturing MnS Producer registration information with APIProviderEnrolmentDetails

To manage the registration of the MnF hosting the MnS producer on the CCF using CAPIF_API_Provider_Management API, it is needed to populate the “APIProviderEnrolmentDetails” datatype with the registration information of the MnF hosting the MnS producer. Detailed in clause 5.1.1.3.1.2, the mapping solution is summarized below:

- The credentials of the MnF hosting the MnS producer can be mapped to “APIProviderEnrolmentDetails /regSec”.

- The  role played for the MnF hosting the MnS producer can be mapped to “APIProviderEnrolmentDetails /apiProvFuncs”. Note that the MnS producer will provide some of the functionality provided by the  AEF, i.e., exposing the management services through the CAPIF-2 and CAPIF-2e interfaces.

- The public key of the MnF hosting the MnS producer can be mapped to “APIProviderEnrolmentDetails/ apiProvFuncs/regInfo/apiProvPubKey”.

The solution is feasible.

### 5.1.2	Use case #2: Publishing of management services to the CCF

#### 5.1.2.1	Description

Upon registration of the MnF hosting the MnS producer into the CAPIF, then the MnF hosting the MnS producer can be enabled to decide what management service information will be made available for external consumption (i.e., the entire MnS, or a subset of it).

Once this decision is made, then this information needs to be published into CCF, so that it can later be used by the external MnS consumers to discover and subsequently invoke the MnS. To this end, the following should occur:

First, the management services are mapped into service APIs that can be published to the CCF. This is due to the fact that the information published into CAPIF is information related to service APIs. This information is represented with “ServiceAPIDescription” (see clause 8.2.4.2.2 in TS 29.222 [13]). However, in the 3GPP management system, the IOC that describes the information related to an MnS’s is MnSInfo IOC (see clause 4.3.42 in TS 28.622 [3]). The MnS producer posts and manages the MnSInfo instances into MnSRegistry IOC (see clause 4.3.41 in TS 28.622 [3]). In this regard, there is a need to map MnSInfo elements into the serviceAPIDescription information element and to determine who in the 3GPP management system does the mapping.

Secondly, the mapped management services (now service APIs) will be published in the CCF. In CAPIF, the API Publishing Function (APF) is the API provider domain’s entity responsible for publishing and managing service API information (see clause 6.3.5 in TS 23.222 [5]). The APF supports the CAPIF-4 interface (see clause 6.4.7 in TS 23.222 [5]). However, in the 3GPP management system, there is a need to determine who will play the APF role, for example:

-	MnS registry (represented by the  MnSRegistry IOC): The issue is that MnS registry is a managed object and therefore as a managed object, it is not capable of either the mapping of management service information into service API information or the actual publishing of the mapped management service information to the CCF. Accordingly, the MnSRegistry IOC cannot play the role of the CAPIF APF. There is an option of defining a new management function (MnF) that consumes the MnSInfo IOCs data from MnSRegistry IOC and, in turn, takes on the role of APF and publishes the management services to the CCF via the CAPIF-4 interface. This option has the advantage of simultaneously publishing all the management services to the CCF. However, considering SBMA principles, the definition of a new MnF would mean going back to IRP definitions.

-	MnS producer: the MnS producers are not capable of providing the functionalities offered by the CAPIF APFs.

-  MnF hosting the MnS producer: The MnF can be enhanced to play the role of the APF and support the publishing of the MnS(s) through the CAPIF-4 interface.

NOTE: The specification of the MnF is out of scope of the present document.

#### 5.1.2.2	Potential requirements

PREQ-FS_MExpo-Pub-01: The 3GPP management system shall have the capability to map management services information (described by the MnSInfo IOC) into service API information (described by the CAPIF ServiceAPIDescription information element).

PREQ-FS_MExpo-Pub-02: The 3GPP management system shall have the capability to publish the mapped management service information to the CCF.

#### 5.1.2.3	Potential solutions

##### 5.1.2.3.1	Potential solution #1: Publishing of management services into the CCF

###### 5.1.2.3.1.1	Introduction

This potential solution describes how to publish the management services to the CCF. This solution assumes that the MnF hosting the MnS producer supports the CAPIF-4 interface and is able to publish management services to the CCF.

###### 5.1.2.3.1.2	Description

To publish a management service to the CCF, there is a need to map the MnsInfo IOC attributes into the ServiceAPIDescription data type. The potential solution for this mapping is given in Table 5.1.2.3.1.2-1.

Table 5.1.2.3.1.2-1: Mapping of MnSInfo information attributes to the serviceAPIDescription information elements (extract Table 8.2.4.2.2-1 in TS 29.222 [13])

| Attribute name | Data type | P | Cardinality | Description | Equivalent MnS Info IOC attribute/comments |
| --- | --- | --- | --- | --- | --- |
| apiName | string | M | 1 | API name, it is set as {apiName} part of the URI structure as defined in clause 5.2.4 of 3GPP TS 29.122 [14]. | mnsLabel |
| apiId | string | O | 0..1 | API identifier assigned by the CAPIF core function to the published service API. Shall not be present in the HTTP POST request from the API publishing function to the CAPIF core function. Shall be present in the HTTP POST response from the CAPIF core function to the API publishing function and in the HTTP GET response from the CAPIF core function to the API invoker (discovery API). | Shall be stored by the MnF hosting the MnS producer and will provide the mapping between the published MnS and its corresponding service API identification information. |
| aefProfiles | array(AefProfile) | C | 1..N | AEF profile information, which includes the exposed API details (e.g. protocol). For CAPIF-4/4e interface, API publishing function shall provide this attribute to the CAPIF core function in service API publishing. For CAPIF-1/1e interface, the CAPIF core function shall provide this attribute to the API Invoker during service API discovery. (NOTE 2) | See Table 5.1.2.3.1.2-2 |
| description | string | O | 0..1 | Text description of the API |  |
| supportedFeatures | SupportedFeatures | O | 0..1 | The supported optional features of the CAPIF API. (NOTE 1) |  |
| shareableInfo | ShareableInformation | O | 0..1 | Represents whether the service API and/or the service API category can be published to other CCFs. |  |
| serviceAPICategory | string | C | 0..1 | The service API category to which the service API belongs to. This attribute is only applicable for CAPIF-6/6e interface. (NOTE 2) | Not a significant concern for publishing management services |
| ccfId | string | C | 0..1 | CAPIF core function identifier which can be contacted further for discovering the details of service API information. This attribute is only applicable for CAPIF-6/6e interface and shall be provided with serviceAPICategory. (NOTE 2) | Not a significant concern for publishing management services |
| apiSuppFeats | SupportedFeatures | O | 0..1 | Provided by the consumer to indicate the features supported by the service API. |  |
| pubApiPath | PublishedApiPath | C | 0..1 | It contains the published API path within the same CAPIF provider domain. it shall be provided by the CCF when publishing the service API to other CCF via the CAPIF-6 reference point. | Not a significant concern for publishing management services |
| NOTE 1: For CAPIF_Publish_Service_API, the supported features attribute shall be provided in the HTTP POST request and in the response of successful resource creation. In addition, the supportedFeatures attribute may include one or more the supported features as defined in clause 8.2.6. NOTE 2: For CAPIF-6/6e interface, at least one of aefProfiles or serviceAPICategory and the corresponding ccfId shall be provided. |  |  |  |  |  |

Table 5.1.2.3.1.2-2 Transformation of MnSInfo IOC attributes to AefProfile data type attributes (extract of Table 8.2.4.2.4-1 in TS 29.222 [13])

| Attribute name | Data type | P | Cardinality | Description | Equivalent MnS Info IOC attribute/comments |
| --- | --- | --- | --- | --- | --- |
| aefId | string | M | 1 | AEF identifier | Provided to the MnF hosting the MnS producer by the CCF during the registration of the API provider domain functions. |
| versions | array(Version) | M | 1..N | API version | See Table 5.1.2.3.1.2-3 for more information |
| protocol | Protocol | O | 0..1 | Protocol used by the API. |  |
| dataFormat | DataFormat | O | 0..1 | Data format used by the API |  |
| securityMethods | array(SecurityMethod) | O | 1..N | Security methods supported by the AEF for all interfaces. Certain interfaces may have different security methods supported in the attribute interfaceDescriptions. | Will most likely always be OAUTH 2.0 for the 3GPP management system |
| domainName | string | O | 0..1 | Domain to which API belongs to (NOTE 1) |  |
| interfaceDescriptions | array(InterfaceDescription) | O | 1..N | Interface details (NOTE 1) | See Table 5.1.2.3.1.2-5 for more information |
| aefLocation | AefLocation | O | 0..1 | The location information (e.g. civic address, GPS coordinates, data center ID) where the AEF providing the service API is located. |  |
| NOTE 1: Only one of the attributes "domainName" or "interfaceDescriptions" shall be included. NOTE 2: Notification or callback type of resource is not included. |  |  |  |  |  |

Table 5.1.2.3.1.2-3 Mapping of MnSInfo IOC attributes to Version datatype (extract of Table 8.2.4.2.5-1 in TS 29.222 [13])

| Attribute name | Data type | P | Cardinality | Description | Equivalent MnS Info IOC attribute/comments |
| --- | --- | --- | --- | --- | --- |
| apiVersion | string | M | 1 | API major version in URI (e.g. v1) | mnsVersion |
| expiry | DateTime | O | 0..1 | Expiry date and time of the AEF service. This represents the planned retirement date as specified in clause 4.3.1.5 of 3GPP TS 29.501 [18]. |  |
| resources | array(Resource) | O | 1..N | Resources supported by the API. It may include the custom operations with resource association. | mnsScope (each MOI under the mnsScope will be mapped to the Resource datatype (See Table 5.1.2.3.1.2-4)) |
| custOperations | array(CustomOperation) | O | 1..N | Custom operations without resource association. |  |

Table 5.1.2.3.1.2-4: Mapping of MnSInfo IOC attributes to Resource data type (extract of Table 8.2.4.2.6-1 in TS 29.222 [13])

| Attribute name | Data type | P | Cardinality | Description | Equivalent MnS Info IOC attribute/comments |
| --- | --- | --- | --- | --- | --- |
| resourceName | string | M | 1 | Resource name. | No possible mapping to MnSInfo attribute |
| commType | CommunicationType | M | 1 | Communication type used by the API resource. (NOTE 1) | Shall be either REQUEST_RESPONSE or SUBSCRIBE_NOTIFY |
| uri | string | M | 1 | Relative URI of the API resource, it is set as {apiSpecificSuffixes} part of the URI structure as defined in clause 5.2.4 of 3GPP TS 29.122 [14]. | Shall be the DN that uniquely identifies the MOI. The DN-to-URI mapping mechanism is defined in clause 4.2.3 TS 32.158 [30]. |
| custOpName | string | O | 0..1 | it is set as {custOpName} part of the URI structure for the case where there is only a single custom operation associated with this resource as defined in clause 5.2.4 of 3GPP TS 29.122 [14]. (NOTE 2) |  |
| custOperations | array(CustomOperation) | O | 1..N | List of custom operations associated to this resource. (NOTE 2) | MultipleCustomOperations |
| operations | array(Operation) | C | 1..N | Supported HTTP methods for the API resource. Only applicable when the protocol in AefProfile indicates HTTP. | HTTP methods invocable on the MOI |
| description | string | O | 0..1 | Text description of the API resource. |  |
| NOTE 1: The communication type refers to the semantics of the resource or custom operation and is independent of the HTTP methods that are supported (e.g. if a resource is used for subscriptions then its CommunicationType shall be SUBSCRIBE_NOTIFY even if it supports also the GET method for retrieving the subscriptions). NOTE 2: The attributes "custOpName" and "custOperations" are mutually exclusive. |  |  |  |  |  |

Table 5.1.2.3.1.2-5 Mapping of MnSInfo IOC attributes to InterfaceDescription datatype (extract of Table 8.2.4.2.3-1 in TS 29.222 [13])

| Attribute name | Data type | P | Cardinality | Description | Equivalent MnS Info IOC attribute/comments |
| --- | --- | --- | --- | --- | --- |
| ipv4Addr | Ipv4Addr | C | 0..1 | String identifying an IPv4 address (NOTE 1) | Only fqdn attribute applies incase of MnS |
| ipv6Addr | Ipv6Addr | C | 0..1 | String identifying an IPv6 address (NOTE 1) | Only fqdn attribute applies incase of MnS |
| fqdn | Fqdn | C | 0..1 | String containing a Fully Qualified Domain Name. (NOTE 1) | mnsAddress |
| port | Port | O | 0..1 | Port |  |
| apiPrefix | string | O | 0..1 | A string representing an optional deployment-specific string (API prefix) in the form of a sequence of path segments that starts with a "/" character. |  |
| securityMethods | array(SecurityMethod) | M | 1..N | Security methods supported by the interface. It takes precedence over the security methods provided in AefProfile, for this specific interface | It will most likely always be OAUTH 2.0 for SA5 |
| NOTE 1: Exactly one of the attributes "ipv4Addr", "ipv6Addr" and "fqdn" shall be included. NOTE 2: When the contents of this data type are used to construct the apiRoot of an API, they are used as described in clause 4.4.1 of 3GPP TS 29.501 [18]. |  |  |  |  |  |

With this mapping, the management service information can be represented by the service API description information element. Accordingly, the MnF hosting the MnS producer acting as the APF can send an HTTP POST via the CAPIF-4 interface to publish the management service to the CCF (as defined in clause 8.2 in TS 29.222 [13]).

#### 5.1.2.4	Evaluation of potential solutions

Editor's Note:	This clause provides the evaluation of potential solutions.

### 5.1.3	Use case #3: Configuring discovery information of an external MnS consumer.

#### 5.1.3.1		Description

The operator may not want that all service APIs published into CAPIF are discoverable to all API invokers; in fact, the operator may want to limit the visibility that certain API invokers have over published API information, according to the business agreements settled with the stakeholder owning the API invoker. To that end, the CCF shall be able to be configured with discovery information on a per API invoker basis. The discovery information , allows to specify the visibility for an API invoker, and therefore filter which service API information this API invoker can:

-	Subscribe to, during the API invoker onboarding.

-	Discover and subsequently access, once API invoker gets onboarded. Upon receiving a discovery request from the API invoker with certain query information (i.e., criteria for discovery matching service APIs), the CCF will apply the configured discovery information on the search results matching the query criteria and filter them accordingly.

When using the CAPIF as the framework to expose MnSs, the external MnS consumer plays the role of API invoker, and the published API information corresponds to the MnS information published into CCF compliant with ServiceAPIDescription (clause 8.2.4.2.2 from [5]). Likewise, for the cases where CAPIF and 3GPP management system belong both to the same administrative domain, the 3GPP management system administrator [29] also acts as CAPIF administrator.

However, there are two issues that deserve further analysis.

On the one hand, it is needed to discuss how 3GPP management system can help define discovery information for each external MnS consumer. The aim of the configured discovered information is to limit their visibility over the published MnS information, either at onboarding time (when the external MnS consumer becomes an onboarded API invoker) or at operation time (when the external MnS consumer issues discovery requests). The configured discovered information can define filters at two levels:

-	First, the discovery information allows configuring which resources are made discoverable to the external MnS consumer. For each MOI which is under the management scope of the published MnS, the discovery information specifies whether the MOI is visible or not.

-	Secondly, for each visible MOI, the discovery information allows limiting what the external MnS consumer can see regarding:

-	CRUD operations and/or notifications associated to the MOI.

-	Management data, e.g. performance and fault information, associated to the MOI.

On the other hand, it is needed to discuss how a defined discovery information can be configured on the CCF. This association allows setting the visibility that this API invoker will have over its lifetime, from onboarding to offboarding, including all stages in between. If needed, the discovery information can be updated over the API invoker’s lifetime.

#### 5.1.3.2	Potential requirements

PREQ-FS_MExpo-Disc-01: The 3GPP management system shall provide the capability to configure which MOIs, among those ones that are within the scope of a published MnSs, are visible to an external MnS consumer. The filtered set of MOIs is referred to as discoverable MOIs.

PREQ-FS_MExpo-Disc-02: The 3GPP management system shall provide the capability to configure, for a discoverable MOI, which attributes supported by this MOI are visible to an external MnS consumer.

PREQ-FS_MExpo-Disc-03: The 3GPP management system shall provide the capability to configure, for a discoverable MOI, which CRUD operations associated to this MOI are visible to an external MnS consumer.

PREQ-FS_MExpo-Disc-04: The 3GPP management system shall provide the capability to configure, for a discoverable MOI, which notifications associated to this MOI are visible to an external MnS consumer.

PREQ-FS_MExpo-Disc-05: The 3GPP management system shall provide the capability to configure, for a discoverable MOI, which management data associated to this MOI are visible to an external MnS consumer.

NOTE 1: Management data includes for example performance measurements, KPIs and alarm information.

NOTE 2: The above listed requirements are based on those originally defined in 3GPP TS 28.319 [29], but now applicable for external MnS consumers.

#### 5.1.3.3	Potential solutions

##### 5.1.3.3.1	Potential solution #1: Using AccessRule class to support discovery information definition

###### 5.1.3.3.1.1	Introduction

AccessRule class (TS 28.319 [29], clause 7.3.3) enables providing granular control on what actions are visible over which resources, when certain conditions (filter criteria) are met. The table below provides a summary of the class attributes.

| Attribute Name | S |
| --- | --- |
| ruleName | M |
| dataNodeSelector | M |
| operations | M |
| actions | O |
| componentCData | O |

The different AccessRule instances are defined at design time and stored in a database accessible by 3GPP management system. An example of this database can be the operator’s Authentication, Authorization and Accounting (AAA) server.

This solution proposes using the existing AccessRule class to define discovery information for external MnS consumer. With this solution, it is proposed to fulfil the requirements PREQ-FS_MExpo-Disc-01, PREQ-FS_MExpo-Disc-02, PREQ-FS_MExpo-Disc-03, PREQ-FS_MExpo-Disc-04 and PREQ-FS_MExpo-Disc-05.

###### 5.1.3.3.1.2	Description

It is proposed to use AccessRule to generate discovery information as follows:

-	 “dataNodeSelector”: it can be used not only to configure which MOI are visible (first level of filter), but also the visibility over the attributes of these MOIs. This allows fulfilling PREQ-FS_MExpo-Disc-01 and PREQ-FS_MExpo-Disc-02.

-	 “operations”: it can be used to specify which CRUD operations are visible for each MOI listed in the dataNodeSelector. This allows fulfilling PREQ-FS_MExpo-Disc-03.

-	 “componentCData”: it can be used to specify which management data (e.g, performance measurements, KPIs and alarm information) are visible for each MOI listed in the dataNodeSelector. This allows fulfilling PREQ-FS_MExpo-Disc-05.

-	 “actions”: The usage of this attribute is to grant authorization to the external MnS consumer when accessing service APIs over CAPIF-2/2e interface. However, this is a separate use case, which is described in clause 5.1.4.

##### 5.1.3.3.2	Potential solution #1b: Using Role class for discovery information definition

###### 5.1.3.3.2.1	Introduction

Role class (TS 28.319 [29], clause 7.3.2) enables capturing multiple access rules. The table below provides a summary of the class attributes.

| Attribute Name | S |
| --- | --- |
| roleName | M |
| Attribute related to role |  |
| accessRulesList | M |

The different Role instances are defined at design time and stored in a database accessible by 3GPP management system. Examples of this database can be the operator’s Authentication, Authorization and Accounting (AAA) server.

This solution proposes using the existing Role class to define discovery information for external MnS consumer. With this solution, it is proposed to fulfil the requirements PREQ-FS_MExpo-Disc-01, PREQ-FS_MExpo-Disc-02, PREQ-FS_MExpo-Disc-03, PREQ-FS_MExpo-Disc-04 and PREQ-FS_MExpo-Disc-05.

###### 5.1.3.3.2.2	Description

This solution builds on top of solution #1, by associating roles instead of individual access rules. Solution #1b requires the underlying implementation of solution #1.

#### 5.1.3.4	Evaluation of potential solutions

##### 5.1.3.4.1	Evaluation of potential solutions #1 and #1b

Solutions #1 and #1b allow fulfilling the requirements PREQ-FS_MExpo-Disc-01, PREQ-FS_MExpo-Disc-02, PREQ-FS_MExpo-Disc-03 and PREQ-FS_MExpo-Disc-05, providing controllable granularity on which published MnS information can be consumed by an external MnS consumer.

However, there are two gaps with the proposed solutions.

Firstly, it is noteworthy that PREQ-FS_MExpo_Disc-04 cannot be fulfilled with the current MSAC solution. The “operations” attribute only supports CRUD operations, but no notifications. Before working on fulfilling PREQ-FS_MExpo_Disc-04, there are open questions that need to be answered first:

-    Notification types: it should be clarified whether the intention is to set the visibility over certain notification types, and if so, which notification types are eligible. Are Configuration Management related notifications (e.g., notifyMOICreation, notifyMOIAttributeValueChange, notifyFileReady) included? Are Fault Management related notifications (e.g., notifyNewAlarm, notifyAckStateChanged, notifyClearedAlarm) included? Are Threshold Crossing notifications included?

-    Notification content: it should be clarified whether it is the intention to set the visibility over notification content; accessing to notification content can reveal secret.

-    “dataNodeSelector” attribute in AccessRule class: it should be clarified the role of this attribute with notification support: In every notification, there is parameter “objectInstance” in the notification header. Is “dataNodeSelector” attribute evaluated against this parameter “objectInstance”?

Secondly, it is not clear what is the default system behavior when the operator does not configure any rule, and therefore any policy. For example, what happens when a node (specified by “dataNodeSelector”) is not covered by any rule?. Should the system allow or deny visibility over such a node? The default system behavior for this situation is not described in this TR nor in the TS 28.319 [29].

### 5.1.4	Use case #4: Authorization of the external MnS consumer to access the management service API

#### 5.1.4.1	Description

After the external MnS consumer has discovered the available management services (now service APIs) at the CCF, the external MnS consumer proceeds to request the CCF for the authorization information to access the service API (see clause 8.11 of TS 23.222 [5] and clause 6.5.2.3 of TS 33.122 [14]) via the CAPIF-1e interface. This authorization information contains the actions (e.g., ALLOW, DENY) the external MnS consumer can perform on the discovered MnS producer. These actions might differ from what the external MnS consumer is authorized to discover from the CCF (depending on the configured discovery policy). However, currently, the CCF does not have this authorization information in order to generate the access token to grant external MnS consumers accessto the discovered service APIs (i.e., the management services).

Furthermore, after receiving the authorization information from the CCF, the external MnS consumer will proceed to access the management service at the MnS producer (the MnS producer role is provided by the MnF acting as  the CAPIF API exposing function (AEF)) via the CAPIF-2e interface. This implies that the MnF hosting the MnS producer should be able to support the CAPIF-2e interface

#### 5.1.4.2	Potential requirements

PREQ-FS_MExpo-Auth-01: The 3GPP management system shall provide the CCF with the authorization information for an external MnS consumer.

#### 5.1.4.3	Potential solutions

#### 5.1.4.3.1	Potential solution #<i>: <Potential Solution i Title>

###### 5.1.4.3.1.1	Introduction

This potential solution describes how the CCF can authorize the external MnS consumer to access the management service API.

###### 5.1.4.3.1.2	Description

To authorize an external MnS consumer to consume management service(s) at the MnS producer, the CCF can use the AccessRule class (defined in clause 7.3.3 in TS 28.319 [29]) to define the access rule for authorization. This access rule is identified by the ruleName attribute. The dataNodeSelector attribute of the AccessRule class can be used to select the resources to which the access rule will apply. These resources are either the managed objects (IOCs), instances of managed objects (MOIs), or their corresponding attribute(s).

For the selected resources under the dataNodeSelector attribute, the operations attribute is used to specify the operations (as specified in clause 11.1.1 in TS 28.532[17]) that can be applied to these resources.

For each resource and operation combination, the actions attribute permits what operation(s) the external MnS consumer can perform on the resource.

To request for authorization to consume a specific service API, the external MnS consumer sends a "service API authorization request" (see clause 8.11 of TS 23.222[5]) to the CCF. The request should include the information provided in Table 5.1.4.3.1.2-1.

Table 5.1.4.3.1.2-1: Definition of type AccessTokenReq (Table extract from Table 8.5.4.2.6-1in TS 29.222 [13])

| Attribute name | Data type | P | Cardinality | Description | Equivalent MnS Info IOC attribute/comments |
| --- | --- | --- | --- | --- | --- |
| grant_type | string | M | 1 | This attribute shall contain the grant type as "client_credentials", or when the "RNAA" feature is supported, either "client_credentials" or "authorization_code". (NOTE 3, NOTE 4) | Should be set to "client_credentials" |
| client_id | string | M | 1 | This attribute shall contain the API invoker Identifier. (NOTE 3) | This is the same as the API invoker Id. |
| resOwnerId | ResOwnerId | O | 0..1 | Contains the identifier of the resource owner. This attribute shall be present only when the access token request is used for RNAA. |  |
| client_secret | string | O | 0..1 | This attribute when present shall contain the onboarding secret which is got during API invoker onboarding. (NOTE 3) |  |
| scope | string | O | 0..1 | This attribute when present shall contain a list of AEF identifiers and its associated API names for which the access_token is authorized for use. It takes the format of 3gpp#aefId1:apiName1,apiName2,…apiNameX;aefId2:apiName1,apiName2,…apiNameY;…aefIdN:apiName1,apiName2,…apiNameZ Using delimiter "#" after the discriminator "3gpp", ":" after AEF identifier, "," between API names and ";" between the last API name of the previous AEF identifier and the next AEF identifier. (NOTE 2) Example: '3gpp#aef-jiangsu-nanjing:3gpp-monitoring-event,3gpp-as-session-with-qos;aef-zhejiang-hangzhou:3gpp-cp-parameter-provisioning,3gpp-pfd-management' |  |
| authCode | string | C | 0..1 | Contains the authorization code. This attribute shall be included only when the access token request is used for RNAA and the OAuth "authorization code" grant type is used. | RNAA |
| redirect_uri | string | O | 0..1 | Contains the redirection URI that was used to obtain the authorization code provided within the "authCode" attribute. This attribute may be included only when the access token request is used for RNAA and the OAuth "authorization code" grant type is used. (NOTE 3) | RNAA |
| NOTE 1: This data structure shall not be treated as a JSON object. It shall be treated as a key, value pair data structure to be encoded using x-www-urlencoded format as specified in clause 17.13.4.1 of W3C HTML 4.01 Specification [22]. NOTE 2: The scope may contain more space-delimited strings which further add additional access ranges to the scope, the definition of those additional strings is out of the scope of the present document. NOTE 3: The "grant_type", "client_id", "client_secret" and "redirect_uri" attributes do not follow the related naming convention defined in clause 7.2.1. These attributes are however kept as currently defined in this specification in order to keep them aligned with corresponding claims defined in IETF RFC 6749 [23] and for backward compatibility considerations. NOTE 4: The enumeration value "client_credentials" or "authorization_code" of the "grant_type" attribute does not follow the related naming convention defined in clause 7.2.1. This enumeration is however kept as currently defined in this specification for backward compatibility considerations. |  |  |  |  |  |

When the CCF receives the "service API authorization request" (see clause 8.11 of TS 23.222[5]) from the external MnS consumer, the CCF first validates the request and generates the access token. Upon generating the access token, the CCF sends the "service API authorization response" with the access token information to the external MnS consumer. The generated access token is structured according to the information provided in Table 5.1.4.3.1.2-2.

Table 5.1.4.3.1.2-2: Access token response message parameters (Table extract from Table 8.5.4.2.7-1in TS 29.222 [13])

| Attribute name | Data type | P | Cardinality | Description | Equivalent MnS Info IOC attribute/comments |
| --- | --- | --- | --- | --- | --- |
| access_token | string | M | 1 | This IE shall contain JWS Compact Serialized representation of the JWS signed JSON object containing AccessTokenClaims (see clause 8.5.4.2.8). (NOTE 2) |  |
| token_type | string | M | 1 | This IE shall contain the token type (i.e. "Bearer"). (NOTE 2, NOTE 3) |  |
| expires_in | DurationSec | M | 1 | This IE when present shall contain the number of seconds after which the access_token is considered to be expired. (NOTE 2) |  |
| scope | string | O | 0..1 | This IE when present shall contain a list of AEF identifiers and its associated API names for which the access_token is authorized for use. It takes the format of 3gpp#aefId1:apiName1,apiName2,…apiNameX;aefId2:apiName1,apiName2,…apiNameY;…aefIdN:apiName1,apiName2,…apiNameZ Using delimeter "#" after the discriminator "3gpp", ":" after AEF identifier, "," between API names and ";" between the last API name of the previous AEF identifier and the next AEF identifier. (NOTE 1) Example: '3gpp#aef-jiangsu-nanjing:3gpp-monitoring-event,3gpp-as-session-with-qos;aef-zhejiang-hangzhou:3gpp-cp-parameter-provisioning,3gpp-pfd-management' | This scope contains information that will be used by the management system to enforce authorization to consume management services. |
| NOTE 1: The scope may contain more space-delimited strings which further add additional access ranges to the scope, the definition of those additional strings is out of the scope of the present document. NOTE 2: The "access_token", "token_type" and "expires_in" attributes do not follow the related naming convention defined in clause 7.2.1. These attributes are however kept as currently defined in this specification for backward compatibility considerations. NOTE 3: The enumeration value "Bearer" of the "token_type" attribute does not follow the related naming convention defined in clause 7.2.1. This enumeration is however kept as currently defined in this specification for backward compatibility considerations. |  |  |  |  |  |

To consume the management service, the external MnS consumer provides the generated access token to the MnS producer on the MnF via the CAPIF-2 interface (following the procedures described in clause 8.16 of TS 23.222[5]).

#### 5.1.4.4	Evaluation of potential solutions

The proposed solution covers the requirement PREQ-FS_MExpo-Auth-01 of the use case. However, the proposed solution has the following limitations when it comes OAuth2.0 scopes:

1)   Given that the access token is carried in the HTTP authorization header (see RFC 6750[31]), size limitations are introduced on how big the access token could be. In the 3GPP management system, the access control for each MnS consumer is at the granularity of the resources (i.e., MOIs, IOCs, or their corresponding attributes), the operations that can be performed on these resources, and the actions that are allowed on the permitted operations. This implies that as the number of resources the external MnS consumer is authorized to access increases, so does the size of the access token since the access rule(s) are stored in the "scope" attribute of the access token.

2)   At the MnS producer, as the enforcer, the MnS producer is incharge of validating the access token provided by the external MnS consumer (see clause C.7 TS 33.122[14]). The validation process of the access token involves ensuring that the access token is valid (see RFC 7519[32]). Upon validation, the MnS producer needs to map the scope value of the access token to the access rules (see clause C.5 TS 33.122[14]) and ensure that the access rules are valid for the external MnS consumer. To do this, the MnS producer can contact the authorization server. These checks (i.e., the access token validation, the mapping and the validation of the access rules applicable to the external MnS consumer) contribute to the processing time of the access token on the MnS producer.

### 5.1.5	Use case #5: Logging the management service API invocations to the CCF

#### 5.1.5.1	Description

When exposing management services for consumption by the external MnS consumers (referred to as the API invoker in CAPIF terms), it is crucial to monitor, for example, what management service API was invoked (i.e., the MOIs, attributes or operations), who invoked the API (i.e., the ID of the external MnS consumer), the result of the invocation (e.g., success, or failure) and at what time it was invoked.

Accordingly, the MnF hosting the MnS producer (which acts as the CAPIF AEF) should be able to create the management service API (now service API) invocation log(s) (see clause 8.7 of TS 29.222 [13]) with the desired information. Subsequently, the MnF hosting the MnS producer should be able to send the invocation log(s) to the CCF via the CAPIF-3 interface. The stored logs of the service API invocations can be consumed by authorized consumers (e.g., the AMF entity for auditing purposes and the charging functions). However, currently, the MnFs hosting the MnS producers do not support these functionalities.

#### 5.1.5.2	Potential requirements

PREQ-FS_MExpo-Log-01: The 3GPP management system should support the capability to create logs based on the management service API invocations by external MnS consumers.

PREQ-FS_MExpo-Log-02: The 3GPP management system should support the capability to log the management service API invocations to the CCF.

#### 5.1.5.3	Potential solutions

##### 5.1.5.3.1	Potential solution #1: Creation and logging of the management service API invocations

###### 5.1.5.3.1.1	Introduction

The potential solution assumes that the MnF hosting the MnS producer is the AEF and interacts with the CCF via the CAPIF-3 interface. Accordingly, this potential solution describes how the MnF hosting the MnS producer can create the management service API invocation log. Secondly, the solution describes how the MnF hosting the MnS producer can send these logging data to the CCF for authorized consumers to consume (e.g., AMF and charging functions).

###### 5.1.5.3.1.2	Description

To log the management service API invocations to the CCF, the MnF hosting the MnS producer should create an invocation log (see clause 8.7 in TS 29.222 [13]). Table 5.1.5.3.1.2-1 maps the CAPIF InvocationLog data type and the MnSInfo IOC (see clause 4.3.42 of TS 28.622 [3]) attributes.

Table 5.1.5.3.1.2-1: Mapping between CAPIF InvocationLog data type and the MnSInfo IOC attributes (Table extract from Table 8.7.4.2.2-1 in clause 8.7.4.2.2 in TS 29.222 [13])

| Attribute name | Data type | P | Cardinality | Description | Mapping to MnSInfo IOC attributes/Comments |
| --- | --- | --- | --- | --- | --- |
| aefId | string | M | 1 | Identity information of the API exposing function requesting logging of service API invocations | The MnF hosting the MnS producer obtains this information from the CCF after registration (see clause 5.1.1) |
| apiInvokerId | string | M | 1 | Identity of the API invoker which invoked the service API | Provided by the external MnS consumer when invoking the management service API |
| logs | array(Log) | M | 1..N | Service API invocation log | See Table 5.1.5.3.1.2-2 |
| supportedFeatures | SupportedFeatures | O | 0..1 | Used to negotiate the supported optional features of the API as described in clause 7.8. This attribute shall be provided in the HTTP POST request and in the response of successful resource creation. |  |

Table 5.1.5.3.1.2-2: Mapping between CAPIF Log data type and the MnSInfo IOC attributes (Table extract from Table 8.7.4.2.3-1 in clause 8.7.4.2.3 in TS 29.222 [13]

| Attribute name | Data type | P | Cardinality | Description | Mapping to MnSInfo IOC attributes (see clause 4.3.42 of TS 28.622 [3])/Comments |
| --- | --- | --- | --- | --- | --- |
| apiId | string | M | 1 | String identifying the API invoked. | The MnF hosting the MnS producer receives this attribute from the CCF after publishing the MnS information (see clause 5.1.2). |
| apiName | string | M | 1 | Name of the API which was invoked, it is set as {apiName} part of the URI structure as defined in clause 5.2.4 of 3GPP TS 29.122 [14]. | Same as mnsLabel |
| apiVersion | string | M | 1 | Version of the API which was invoked | Same as mnsVersion |
| resourceName | String | M | 1 | Name of the specific resource invoked | No possible mapping to MnSInfo attributes (could be the IOC name of the MOI) |
| uri | Uri | 0 | 0..1 | Full URI of the API resource as defined in clause 5.2.4 of 3GPP TS 29.122 [14]. | Shall be the DN that uniquely identifies the MOI. The DN-to-URI mapping mechanism is defined in clause 4.2.3 TS 32.158 [30]. |
| protocol | Protocol | M | 1 | Protocol invoked. | HTTPS?? |
| operation | Operation | C | 0..1 | Operation that was invoked on the API, only applicable for HTTP protocol. | E.g., PUT, POST, GET, DELETE, PATCH etc |
| result | string | M | 1 | For HTTP protocol, it contains HTTP status code of the invocation | HTTP status codes |
| invocationTime | DateTime | O | 0..1 | Date on which it was invoked | Is this time when the MnS producer receives the service API invocation request from the external MnS consumer? Needs further clarification |
| invocationLatency | DurationMs | O | 0..1 | Latency for the API invocation. | Is this the duration of the service API invocation (i.e., from the time the external MnS consumer sends the invocation request to the MnS producer to the time when the MnS producer sends the response back to the external MnS consumer)? Needs further clarification |
| inputParameters | ANY TYPE (NOTE) | O | 0..1 | List of input parameters |  |
| OutputParameters | ANY TYPE (NOTE) | O | 0..1 | List of output parameters |  |
| srcInterface | InterfaceDescription | O | 0..1 | Interface description of the API invoker. |  |
| destInterface | InterfaceDescription | O | 0..1 | Interface description of the API invoked. |  |
| fwdInterface | string | O | 0..1 | It includes the node identifier (as defined in IETF RFC 7239 [20] of all forwarding entities between the API invoker and the AEF, concatenated with comma and space, e.g. 192.0.2.43:80, unknown:_OBFport, 203.0.113.60 |  |
| NOTE: Any basic data type defined in OpenAPI Specification [3] may be used. |  |  |  |  |  |

After creating the invocation log, the MnF hosting the MnS producer can log these data to the CCF via the CAPIF-3 interface.

#### 5.1.5.4	Evaluation of potential solutions

The proposed solution satisfies the requirement PREQ-FS_MExpo-Log-01. To fulfill the use case requirement PREQ-FS_MExpo-Log-02, the proposed solution requires that the 3GPP management entity (i.e., the MnF) providing the AEF role implements the CAPIF-3 interface and the associated functionalities.

End Change