3GPP TSG-SA5 Meeting #157 	S5-245828

Hyderabad, India, 14 - 18 October 2024

# Source:	Ericsson España S.A.

# Title:	pCR TR 28.879 Publishing UC - updates on description and requirements

# Document for:	Approval

Agenda Item:	6.19.21

# 1	Decision/action requested

The group is asked to discuss and approve the proposal.

# 2	References

[1] 3GPP TR 28.879: " Study on OAM for service management and exposure to external consumers".

[2] S5-243252: “Rel-19 pCR TR 28.879 Add use case for publishing management services to the CCF”

# 3	Rationale

This pCR aims to update the description and requirements for the publishing use case (use case #2), by:

- Providing further details on the workflow of the use case, clarifying the decision variables on the operator side and the issues that deserve attention.

- Remove references to potential solutions on who is eligible to perform APF functionality; these solutions are to be discussed in the solutions clause, not in the description clause.

- Re-wording existing requirements and specifying two additional ones.

# 4	Detailed proposal

It is proposed to make the following changes in the latest version of TR 28.879 [1].

Begin Change

### 5.1.2	Use case #2: Publishing of management services to the CCF

#### Description

Upon registration of MnS producer into the CAPIF, then this MnS producer can be enabled to decide, for a given MnS:

- which endpoints of this MnS will be made available for external consumption. These endpoints are referred as to reachable MnS endpoints.

- whether reachable MnS endpoints will be offered through one or more service APIs. This decision is at operator discretion, depending on i) how large the scope of MnS endpoints are, ii) and how granular the operator want service APIs to be. It can occur that the operator decides to use one single service API to convey all the endpoints of a MnS; conversely, the operator might decide to split MnS endpoints into two or more service APIs

The information resulting from this two-fold decision needs to be published onto CCF, by making use of the “CAPIF_Publish_Service_API” (see clause 5.3 in 3GPP TS 29.222 [13]). This API allows the API Publishing Function (APF) to publish service API information onto the CCF  (see clause 6.3.5 in 3GPP TS 23.222 [5]) so that it can later be used by the external MnS consumers to discover and subsequently invoke the MnS. This information is represented with "ServiceAPIDescription"  data type (see clause 8.2.4.2.2 in 3GPP TS 29.222 [13]). There are two noteworthy issues here for 3GPP management system.

First, there is a need to ensure that “ServiceAPIDescription” data types can be built using MnS related information available in the 3GPP management system. The MnSInfo IOC (see clause 4.3.42 in 3GPP TS 28.622 [3]), which represents an available MnS, contains part of this information. The MnS producer posts and manages MnSInfo instances into MnSRegistry IOC (see clause 4.3.42 in 3GPP TS 28.622 [3]).

Secondly, there is a need to determine who in the 3GPP management system can perform the APF functionality, so it can invoke “CAPIF_Publish_Service_API”. Performing APF functionality means supporting the CAPIF-4 interface (see clause 6.4.7 in 3GPP TS 23.222 [5]).

#### 5.1.2.2	Potential requirements

PREQ-FS_MExpo-Pub-01: The 3GPP management system shall have the capability to configure, for a given MnS, which endpoints of this MnS will be made available for external consumption through CAPIF. These are referred as to reachable MnS endpoints.

PREQ-FS_MExpo-Pub-02: The 3GPP management system shall have the capability to map reachable MnS endpoints into the endpoints of one or more service APIs.

PREQ-FS_MExpo-Pub-03: The 3GPP management system shall have the capability to populate ServiceAPIDescription data type with information available in the MnSInfo IOC.

PREQ-FS_MExpo-Pub-04: The 3GPP management system shall have the capability to perform APF functionality.

End Change