3GPP TSG-SA5 Meeting #158	S5-246390

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.541 | CR | 1396 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.541 Correct isWritable property for Slicing NRM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2024-11-04 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - The isWritable property for attribute “dLThptPerSlice”,” dLThptPerUE”,” uLThptPerSlice” and “uLThptPerUE” which type is XLThpt<<dataType>> is “T”, however, the isWritable property for attribute all atttibute of XLThpt<<dataType>> are false, which is incorrect. Moreover, the vlaues of attribute “guaThpt” and “maxThpt” in ServiceProfile and SliceProfile are specified by the MnS consumer. - Same issue for attribute “3GPPNoOfPDUSessions” and “non3GPPnOofPDUSessions” in MaxNumberofPDUSessions <<dataType>> - Same issue for attribute “density” in TermDensity<<dataType>> - Same issue for attribute “3GPPNoOfUEs” and “non3GPPNoOfUEs” in MaxNumberofUEs <<dataType>> |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Change the isWritable property for attribute “guaThpt” and “maxThpt” in XLThpt<<dataType>> from “F” to “T”. - Change the isWritable property for attribute “3GPPNoOfPDUSessions” and “non3GPPnOofPDUSessions” in MaxNumberofPDUSessions <<dataType>> from “F” to “T”. - Change the isWritable property for attribute “density” in TermDensity<<dataType>> from “F” to “T”. - Change the isWritable property for attribute “3GPPNoOfUEs” and “non3GPPNoOfUEs” in MaxNumberofUEs <<dataType>> from “F” to “T”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | InCorrect isWritable property for attibute “guaThpt”, “maxThpt”, “3GPPNoOfPDUSessions”,“non3GPPnOofPDUSessions”, “3GPPNoOfUEs” and “non3GPPNoOfUEs”, “density”, “3GPPNoOfUEs” and “non3GPPNoOfUEs”. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.9.2, 6.3.12.2, 6.3.17.2, 6.3.38.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1445 at commit c6568eb46a544d755a86a0f165d0d781baba15eb |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

6.3.9	XLThpt<<dataType>>

6.3.9.1	Definition

This data type can be used to represent downlink or uplink throughput per network slice , per network slice subnet,or per UE in a network slice (see clause 3.4.5, 3.4.6, 3.4.31 and 3.4.32 of GSMA NG.116 [50]).

6.3.9.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| servAttrCom | CM | T | F | F | T |
| guaThpt | O | T | T | F | T |
| maxThpt | O | T | T | F | T |

6.3.9.3	Attribute constraints

| Name | Definition |
| --- | --- |
| servAttrCom S | Condition: This [attribute] is mandatory only when requirements are being defined on throughput per: network slice (GSMA attribute), per UE in a network slice (GSMA attribute) in ServiceProfile. |

6.3.9.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

2nd Change

6.3.12	MaxNumberofPDUSessions <<dataType>>

6.3.12.1	Definition

This data type represents the maximum number of concurrent PDU sessions supported by the network slice (see clause 3.4.16 of GSMA NG.116 [50]).

6.3.12.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| servAttrCom | CM | T | F | T | T |
| 3GPPNoOfPDUSessions | CM | T | T | F | T |
| non3GPPnOofPDUSessions | CM | T | T | F | T |

6.3.12.3	Attribute constraints

| Name | Definition |
| --- | --- |
| 3GPPNoOfPDUSessions S | Condition: This attribute is mandatory only when requirements are being defined for 3GPP access type. |
| non3GPPNoOfPDUSessions S | Condition: This attribute is mandatory only when requirements are being defined for non 3GPP access type. |
| servAttrCom S | Condition: This attribute is mandatory only when requirements are being defined on maximum number of PDU sessions per network slice (GSMA attribute) in ServiceProfile. |

6.3.12.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

3rd Change

6.3.17	TermDensity<<dataType>>

6.3.17.1	Definition

This data type represents Terminal density (See Clause 3.4.30 of GSMA NG.116 [50]).

6.3.17.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| servAttrCom | CM | T | F | T | T |
| density | M | T | T | F | T |

6.3.17.3	Attribute constraints

| Name | Definition |
| --- | --- |
| servAttrCom S | Condition: This attribute is mandatory only when requirements are being defined on terminal density (GSMA attribute) in ServiceProfile. |

6.3.17.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

4th Change

6.3.38	MaxNumberofUEs <<dataType>>

6.3.38.1	Definition

This data type represents the maximum number of concurrent UEs supported by the network slice (see clause 3.4.17 of GSMA NG.116 [50]).

6.3.38.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| servAttrCom | CM | T | F | T | T |
| 3GPPNoOfUEs | CM | T | T | F | T |
| non3GPPNoOfUEs | CM | T | T | F | T |

6.3.38.3	Attribute constraints

| Name | Definition |
| --- | --- |
| 3GPPNoOfUEs S | Condition: This attribute is mandatory only when requirements are being defined for 3GPP access type. |
| non3GPPNoOfUEs S | Condition: This attribute is mandatory only when requirements are being defined for non 3GPP access type. |
| servAttrCom S | Condition: This attribute is mandatory only when requirements are being defined on maximum number of concurrent UEs supported by the network slice (GSMA attribute) in ServiceProfile. |

5th Change

Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1445 at commit c6568eb46a544d755a86a0f165d0d781baba15eb

*** START OF CHANGE 1 ***

*** OpenAPI/TS28541_SliceNrm.yaml ***

<CODE BEGINS>

openapi: 3.0.1

info:

title: Slice NRM

version: 19.0.0

description: >-

OAS 3.0.1 specification of the Slice NRM

@ 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.541; 5G NRM, Slice NRM

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.541/

paths: {}

components:

schemas:

#------------ Type definitions ---------------------------------------------------

Float:

type: number

format: float

MobilityLevel:

type: string

enum:

- STATIONARY

- NOMADIC

- RESTRICTED_MOBILITY

- FULL_MOBILITY

SynAvailability:

type: string

enum:

- NOT_SUPPORTED

- BETWEEN_BS_AND_UE

- BETWEEN_BS_AND_UE_AND_UE_AND_UE

readOnly: true

PositioningAvailability:

type: array

items:

type: string

enum:

- CIDE_CID

- OTDOA

- RF_FINGERPRINTING

- AECID

- HYBRID_POSITIONING

- NET_RTK

readOnly: true

PredictionFrequency:

type: string

enum:

- PERSEC

- PERMIN

- PERHOUR

SharingLevel:

type: string

enum:

- SHARED

- NON_SHARED

NetworkSliceSharingIndicator:

type: string

enum:

- SHARED

- NON_SHARED

SliceSimultaneousUse:

type: integer

minimum: 0

maximum: 4

Category:

type: string

enum:

- CHARACTER

- SCALABILITY

readOnly: true

Tagging:

type: array

readOnly: true

items:

type: string

enum:

- PERFORMANCE

- FUNCTION

- OPERATION

readOnly: true

Exposure:

type: string

enum:

- API

- KPI

ServAttrCom:

type: object

properties:

category:

$ref: '#/components/schemas/Category'

tagging:

$ref: '#/components/schemas/Tagging'

exposure:

$ref: '#/components/schemas/Exposure'

Support:

type: string

enum:

- NOT_SUPPORTED

- SUPPORTED

readOnly: true

DelayTolerance:

type: object

properties:

servAttrCom:

$ref: '#/components/schemas/ServAttrCom'

support:

$ref: '#/components/schemas/Support'

NonIPSupport:

type: object

properties:

servAttrCom:

$ref: '#/components/schemas/ServAttrCom'

support:

$ref: '#/components/schemas/Support'

DeterministicComm:

type: object

properties:

servAttrCom:

$ref: '#/components/schemas/ServAttrCom'

availability:

$ref: '#/components/schemas/Support'

periodicityList:

type: array

items:

type: integer

XLThpt:

type: object

properties:

servAttrCom:

$ref: '#/components/schemas/ServAttrCom'

guaThpt:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Float'

maxThpt:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Float'

MaxPktSize:

type: object

properties:

servAttrCom:

$ref: '#/components/schemas/ServAttrCom'

maxsize:

type: integer

MaxNumberofPDUSessions:

type: object

properties:

servAttrCom:

$ref: '#/components/schemas/ServAttrCom'

nOofPDUSessions:

type: integer

KPIMonitoring:

type: object

properties:

servAttrCom:

$ref: '#/components/schemas/ServAttrCom'

kPIList:

type: array

items:

type: string

readOnly: true

NBIoT:

type: object

properties:

servAttrCom:

$ref: '#/components/schemas/ServAttrCom'

support:

$ref: '#/components/schemas/Support'

RadioSpectrum:

type: object

properties:

servAttrCom:

$ref: '#/components/schemas/ServAttrCom'

nROperatingBands:

type: array

items:

type: string

Synchronicity:

type: object

properties:

servAttrCom:

$ref: '#/components/schemas/ServAttrCom'

availability:

$ref: '#/components/schemas/SynAvailability'

accuracy:

$ref: '#/components/schemas/Float'

Positioning:

type: object

properties:

servAttrCom:

$ref: '#/components/schemas/ServAttrCom'

availability:

$ref: '#/components/schemas/PositioningAvailability'

predictionFrequency:

$ref: '#/components/schemas/PredictionFrequency'

accuracy:

$ref: '#/components/schemas/Float'

UserMgmtOpen:

type: object

properties:

servAttrCom:

$ref: '#/components/schemas/ServAttrCom'

support:

$ref: '#/components/schemas/Support'

V2XCommModels:

type: object

properties:

servAttrCom:

$ref: '#/components/schemas/ServAttrCom'

v2XMode:

$ref: '#/components/schemas/Support'

TermDensity:

type: object

properties:

servAttrCom:

$ref: '#/components/schemas/ServAttrCom'

density:

type: integer

NsInfo:

type: object

properties:

nsInstanceId:

type: string

readOnly: true

nsName:

type: string

readOnly: true

description:

type: string

EmbbEEPerfReq:

type: object

properties:

kpiType:

type: string

enum:

- NUMOFBITS

- NUMOFBITS_RANBASED

req:

type: number

UrllcEEPerfReq:

type: object

properties:

kpiType:

type: string

enum:

- INVOFLATENCY

- NUMOFBITS_MULTIPLIED_INVOFLATENCY

req:

type: number

MIoTEEPerfReq:

type: object

properties:

kpiType:

type: string

enum:

- MAXREGSUBS

- MEANACTIVEUES

req:

type: number

EEPerfReq:

oneOf:

- $ref: '#/components/schemas/EmbbEEPerfReq'

- $ref: '#/components/schemas/UrllcEEPerfReq'

- $ref: '#/components/schemas/MIoTEEPerfReq'

EnergyEfficiency:

type: object

properties:

servAttrCom:

$ref: '#/components/schemas/ServAttrCom'

performance:

$ref: '#/components/schemas/EEPerfReq'

NSSAASupport:

type: object

properties:

servAttrCom:

$ref: '#/components/schemas/ServAttrCom'

support:

$ref: '#/components/schemas/Support'

SecFunc:

type: object

properties:

secFunId:

type: string

readOnly: true

secFunType:

type: string

secRules:

type: array

items:

type: string

readOnly: true

N6Protection:

type: object

properties:

servAttrCom:

$ref: '#/components/schemas/ServAttrCom'

secFuncList:

type: array

items:

$ref: '#/components/schemas/SecFunc'

DataNetwork:

type: object

properties:

servAttrCom:

$ref: '#/components/schemas/ServAttrCom'

dnnList:

type: array

items:

type: string

DataAccess:

type: object

properties:

dataNetworkName:

type: string

dataAccessUsed:

type: string

enum:

- DIRECT_INTERNET_ACCESS

- TERM_PVT_NETWORK

- LOCAL_TRAFFIC

TunnellingMechanism:

type: object

properties:

dataNetworkName:

type: string

tunellingMechanismUsed:

type: string

enum:

- L2TP_TUNNEL

- GRE_TUNNEL

- VPN_TUNNEL

- LABEL_BASED_ROUTING

- 802.1Q_VLAN

- SRV6

- OTHER

LboAllowed:

type: object

properties:

dataNetworkName:

type: string

localBreakoutAllowed:

type: string

enum:

- YES

- NO

DataNetworkAccess:

type: object

properties:

servAttrCom:

$ref: '#/components/schemas/ServAttrCom'

dataAccessList:

type: array

items:

$ref: '#/components/schemas/DataAccess'

tunnellingMechanismList:

type: array

items:

$ref: '#/components/schemas/TunnellingMechanism'

localBreakoutAllowedList:

type: array

items:

$ref: '#/components/schemas/LboAllowed'

CNSliceSubnetProfile:

type: object

properties:

maxNumberofUEs:

type: integer

dLLatency:

type: number

uLLatency:

type: number

dLThptPerSliceSubnet:

$ref: '#/components/schemas/XLThpt'

dLThptPerUE:

$ref: '#/components/schemas/XLThpt'

uLThptPerSliceSubnet:

$ref: '#/components/schemas/XLThpt'

uLThptPerUE:

$ref: '#/components/schemas/XLThpt'

maxNumberOfPDUSessions:

type: integer

coverageAreaTAList:

$ref: 'TS28541_NrNrm.yaml#/components/schemas/TaiList'

resourceSharingLevel:

$ref: '#/components/schemas/SharingLevel'

dLMaxPktSize:

type: integer

uLMaxPktSize:

type: integer

delayTolerance:

$ref: '#/components/schemas/DelayTolerance'

sliceSimultaneousUse:

$ref: '#/components/schemas/SliceSimultaneousUse'

dLReliability:

type: number

uLReliability:

type: number

energyEfficiency:

type: number

dLDeterministicComm:

$ref: '#/components/schemas/DeterministicComm'

uLDeterministicComm:

$ref: '#/components/schemas/DeterministicComm'

survivalTime:

type: number

nssaaSupport:

$ref: '#/components/schemas/NSSAASupport'

n6Protection:

$ref: '#/components/schemas/N6Protection'

nonIPSupport:

$ref: '#/components/schemas/NonIPSupport'

availability:

type: number

maxDLDataVolume:

type: number

maxULDataVolume:

type: number

supportedDataNetworks:

$ref: '#/components/schemas/DataNetwork'

dataNetworkAccess:

$ref: '#/components/schemas/DataNetworkAccess'

dLPktDelayVariation:

$ref: '#/components/schemas/Float'

uLPktDelayVariation:

$ref: '#/components/schemas/Float'

sliceSubnetAvailability:

type: array

items:

$ref: 'TS28623_GenericNrm.yaml#/components/schemas/SchedulingTime'

RANSliceSubnetProfile:

type: object

properties:

coverageAreaTAList:

$ref: 'TS28541_NrNrm.yaml#/components/schemas/TaiList'

dLLatency:

type: number

uLLatency:

type: number

uEMobilityLevel:

$ref: '#/components/schemas/MobilityLevel'

resourceSharingLevel:

$ref: '#/components/schemas/SharingLevel'

maxNumberofUEs:

type: integer

activityFactor:

type: integer

dLThptPerSliceSubnet:

$ref: '#/components/schemas/XLThpt'

dLThptPerUE:

$ref: '#/components/schemas/XLThpt'

uLThptPerSliceSubnet:

$ref: '#/components/schemas/XLThpt'

uLThptPerUE:

$ref: '#/components/schemas/XLThpt'

uESpeed:

type: integer

dLReliability:

type: number

uLReliability:

type: number

dLMaxPktSize:

type: integer

uLMaxPktSize:

type: integer

nROperatingBands:

type: array

items:

type: string

delayTolerance:

$ref: '#/components/schemas/DelayTolerance'

positioning:

$ref: '#/components/schemas/Positioning'

sliceSimultaneousUse:

$ref: '#/components/schemas/SliceSimultaneousUse'

energyEfficiency:

type: number

termDensity:

$ref: '#/components/schemas/TermDensity'

survivalTime:

type: number

synchronicity:

$ref: '#/components/schemas/Synchronicity'

dLDeterministicComm:

$ref: '#/components/schemas/DeterministicComm'

uLDeterministicComm:

$ref: '#/components/schemas/DeterministicComm'

nonIPSupport:

$ref: '#/components/schemas/NonIPSupport'

availability:

type: number

maxDLDataVolume:

type: number

maxULDataVolume:

type: number

kPIMonitoring:

$ref: '#/components/schemas/KPIMonitoring'

dLPktDelayVariation:

$ref: '#/components/schemas/Float'

uLPktDelayVariation:

$ref: '#/components/schemas/Float'

TopSliceSubnetProfile:

type: object

properties:

dLLatency:

type: integer

uLLatency:

type: integer

maxNumberofUEs:

type: integer

dLThptPerSliceSubnet:

$ref: '#/components/schemas/XLThpt'

dLThptPerUE:

$ref: '#/components/schemas/XLThpt'

uLThptPerSliceSubnet:

$ref: '#/components/schemas/XLThpt'

uLThptPerUE:

$ref: '#/components/schemas/XLThpt'

dLMaxPktSize:

type: integer

uLMaxPktSize:

type: integer

maxNumberOfPDUSessions:

type: integer

nROperatingBands:

type: array

items:

type: string

sliceSimultaneousUse:

$ref: '#/components/schemas/SliceSimultaneousUse'

energyEfficiency:

$ref: '#/components/schemas/EnergyEfficiency'

synchronicity:

$ref: '#/components/schemas/Synchronicity'

delayTolerance:

$ref: '#/components/schemas/DelayTolerance'

positioning:

$ref: '#/components/schemas/Positioning'

termDensity:

$ref: '#/components/schemas/TermDensity'

activityFactor:

type: integer

coverageAreaTAList:

$ref: 'TS28541_NrNrm.yaml#/components/schemas/TaiList'

resourceSharingLevel:

$ref: '#/components/schemas/SharingLevel'

uEMobilityLevel:

$ref: '#/components/schemas/MobilityLevel'

uESpeed:

type: integer

dLReliability:

type: number

uLReliability:

type: number

dLDeterministicComm:

$ref: '#/components/schemas/DeterministicComm'

uLDeterministicComm:

$ref: '#/components/schemas/DeterministicComm'

survivalTime:

type: number

nssaaSupport:

$ref: '#/components/schemas/NSSAASupport'

n6Protection:

$ref: '#/components/schemas/N6Protection'

nonIPSupport:

$ref: '#/components/schemas/NonIPSupport'

availability:

type: number

maxDLDataVolume:

type: number

maxULDataVolume:

type: number

kPIMonitoring:

$ref: '#/components/schemas/KPIMonitoring'

supportedDataNetworks:

$ref: '#/components/schemas/DataNetwork'

dataNetworkAccess:

$ref: '#/components/schemas/DataNetworkAccess'

dLPktDelayVariation:

$ref: '#/components/schemas/Float'

uLPktDelayVariation:

$ref: '#/components/schemas/Float'

sliceSubnetAvailability:

type: array

items:

$ref: 'TS28623_GenericNrm.yaml#/components/schemas/SchedulingTime'

ServiceProfile:

type: object

properties:

serviceProfileId:

type: string

readOnly: true

plmnInfoList:

$ref: 'TS28541_NrNrm.yaml#/components/schemas/PlmnInfoList'

maxNumberofUEs:

type: number

dLLatency:

type: number

uLLatency:

type: number

uEMobilityLevel:

$ref: '#/components/schemas/MobilityLevel'

sst:

$ref: 'TS28541_NrNrm.yaml#/components/schemas/Sst'

networkSliceSharingIndicator:

$ref: '#/components/schemas/NetworkSliceSharingIndicator'

availability:

type: number

delayTolerance:

$ref: '#/components/schemas/DelayTolerance'

dLDeterministicComm:

$ref: '#/components/schemas/DeterministicComm'

uLDeterministicComm:

$ref: '#/components/schemas/DeterministicComm'

dLThptPerSlice:

$ref: '#/components/schemas/XLThpt'

dLThptPerUE:

$ref: '#/components/schemas/XLThpt'

uLThptPerSlice:

$ref: '#/components/schemas/XLThpt'

uLThptPerUE:

$ref: '#/components/schemas/XLThpt'

dLMaxPktSize:

$ref: '#/components/schemas/MaxPktSize'

uLMaxPktSize:

$ref: '#/components/schemas/MaxPktSize'

maxNumberofPDUSessions:

$ref: '#/components/schemas/MaxNumberofPDUSessions'

kPIMonitoring:

$ref: '#/components/schemas/KPIMonitoring'

nBIoT:

$ref: '#/components/schemas/NBIoT'

radioSpectrum:

$ref: '#/components/schemas/RadioSpectrum'

synchronicity:

$ref: '#/components/schemas/Synchronicity'

positioning:

$ref: '#/components/schemas/Positioning'

userMgmtOpen:

$ref: '#/components/schemas/UserMgmtOpen'

v2XCommModels:

$ref: '#/components/schemas/V2XCommModels'

coverageArea:

type: array

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/GeoArea'

termDensity:

$ref: '#/components/schemas/TermDensity'

activityFactor:

$ref: '#/components/schemas/Float'

uESpeed:

type: integer

survivalTime:

type: number

dLReliability:

type: number

uLReliability:

type: number

maxDLDataVolume:

type: number

maxULDataVolume:

type: number

sliceSimultaneousUse:

$ref: '#/components/schemas/SliceSimultaneousUse'

energyEfficiency:

$ref: '#/components/schemas/EnergyEfficiency'

nssaaSupport:

$ref: '#/components/schemas/NSSAASupport'

n6Protection:

$ref: '#/components/schemas/N6Protection'

nonIPSupport:

$ref: '#/components/schemas/NonIPSupport'

supportedDataNetworks:

$ref: '#/components/schemas/DataNetwork'

dataNetworkAccess:

$ref: '#/components/schemas/DataNetworkAccess'

dLPktDelayVariation:

$ref: '#/components/schemas/Float'

uLPktDelayVariation:

$ref: '#/components/schemas/Float'

sliceAvailability:

type: array

items:

$ref: 'TS28623_GenericNrm.yaml#/components/schemas/SchedulingTime'

SliceProfile:

type: object

properties:

sliceProfileId:

type: string

readOnly: true

plmnInfoList:

$ref: 'TS28541_NrNrm.yaml#/components/schemas/PlmnInfoList'

cNSliceSubnetProfile:

$ref: '#/components/schemas/CNSliceSubnetProfile'

rANSliceSubnetProfile:

$ref: '#/components/schemas/RANSliceSubnetProfile'

topSliceSubnetProfile:

$ref: '#/components/schemas/TopSliceSubnetProfile'

LogicalInterfaceInfo:

type: object

properties:

logicalInterfaceType:

type: string

enum:

- VLAN

- MPLS

- SEGMENT

logicalInterfaceId:

type: string

systemName:

type: string

portName:

type: string

routingProtocol:

type: string

enum:

- RIP

- IGMP

- OSPF

- EGP

- EIGRP

- BGP

- IS_IS

- STATIC

ConnectionPointInfo:

type: object

properties:

connectionPointId:

type: string

connectionPointIdType:

type: string

enum:

- VLAN

- MPLS

- SEGMENT

- IPV4

- IPV6

- ATTACHMENT_CIRCUIT

ServiceProfileList:

type: array

items:

$ref: '#/components/schemas/ServiceProfile'

SliceProfileList:

type: array

items:

$ref: '#/components/schemas/SliceProfile'

FeasibilityResult:

description: >-

An attribute which specifies the feasibility check result for the feasibility check and reservation job.

type: string

enum:

- FEASIBLE

- INFEASIBLE

readOnly: true

InFeasibleReason:

description: >-

An attribute that specifies the additional reason information if the feasibility check result is infeasible.The detailed ENUM value is FFS.

type: string

readOnly: true

RecommendationRequest:

description: >-

An attribute represents MnS consumer's request for recommended network slice related requirements.

type: boolean

default: false

RecommendedRequirements:

description: >-

An attribute that specifies the recommended network slicing related requirements (i.e. ServiceProfile and SliceProfile information) which can be supported by the MnS producer..

type: string

readOnly: true

ResourceReservation:

description: >-

An attribute represents MnS consumer's requirements for resource reservation.

type: boolean

default: false

RequestedReservationExpiration:

description: >-

An attribute which specifies MnS consuner's requirements for the validity period of the resource reservation.

$ref: 'TS28623_ComDefs.yaml#/components/schemas/TimeWindow'

ResourceReservationStatus:

description: >-

An attribute which specifies the resource reservation result for the feasibility check job.

type: string

enum:

- RESERVED

- UNRESERVED

- USED

readOnly: true

ReservationExpiration:

description: >-

An attribute which specifies the actual validity period of the resource reservation..

$ref: 'TS28623_ComDefs.yaml#/components/schemas/TimeWindow'

ReservationFailureReason:

description: >-

An attribute that specifies the additional reason information if the reservation is failed.

type: string

readOnly: true

ResourceIsolationRule:

type: object

properties:

resourceType:

type: string

enum:

- MANAGED_FUNCTION

- NETWORK_SERVICE

isolationRule:

type: string

enum:

- DEDICATED

- SHARED

NetworkSlicingApplicability:

type: string

enum:

- NETWORKSLICE

- NETWORKSLICESUBNET

#-------- Definition of types for name-containments ------

SubNetwork-ncO-SliceNrm:

type: object

properties:

NetworkSlice:

$ref: '#/components/schemas/NetworkSlice-Multiple'

NetworkSliceSubnet:

$ref: '#/components/schemas/NetworkSliceSubnet-Multiple'

EP_Transport:

$ref: '#/components/schemas/EP_Transport-Multiple'

NetworkSliceSubnetProviderCapabilities:

$ref: '#/components/schemas/NetworkSliceSubnetProviderCapabilities-Multiple'

FeasibilityCheckAndReservationJob:

$ref: '#/components/schemas/FeasibilityCheckAndReservationJob-Multiple'

NetworkSliceController:

$ref: '#/components/schemas/NetworkSliceController-Multiple'

NetworkSliceSubnetController:

$ref: '#/components/schemas/NetworkSliceSubnetController-Multiple'

IsolationProfile:

$ref: '#/components/schemas/IsolationProfile-Multiple'

#------------ Definition of concrete IOCs ----------------------------------------

NetworkSlice-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

networkSliceSubnetRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnRo'

operationalState:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/OperationalState'

administrativeState:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AdministrativeState'

serviceProfileList:

$ref: '#/components/schemas/ServiceProfileList'

networkSliceControllerRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnListRo'

isolationProfileRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

NetworkSliceSubnet-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

managedFunctionRefList:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnListRo'

networkSliceSubnetRefList:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnListRo'

operationalState:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/OperationalState'

administrativeState:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AdministrativeState'

nsInfo:

$ref: '#/components/schemas/NsInfo'

sliceProfileList:

$ref: '#/components/schemas/SliceProfileList'

epTransportRefList:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

priorityLabel:

type: integer

networkSliceSubnetType:

type: string

enum:

- TOP_SLICESUBNET

- RAN_SLICESUBNET

- CN_SLICESUBNET

networkSliceSubnetControllerRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnListRo'

isolationProfileRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

EP_Transport-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

ipAddress:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/IpAddrRo'

localLogicalInterfaceInfo:

$ref: '#/components/schemas/LogicalInterfaceInfo'

qosProfile:

type: string

epApplicationRefs:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

connectionPointRefList:

type: array

items:

$ref: '#/components/schemas/ConnectionPointInfo'

NetworkSliceSubnetProviderCapabilities-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

dLlatency:

type: integer

readOnly: true

uLlatency:

type: integer

readOnly: true

dLThptPerSliceSubnet:

$ref: '#/components/schemas/XLThpt'

uLThptPerSliceSubnet:

$ref: '#/components/schemas/XLThpt'

coverageAreaTAList:

$ref: 'TS28541_5GcNrm.yaml#/components/schemas/TaiList'

FeasibilityCheckAndReservationJob-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

profile:

oneOf:

- $ref: '#/components/schemas/SliceProfile'

- $ref: '#/components/schemas/ServiceProfile'

resourceReservation:

$ref: '#/components/schemas/ResourceReservation'

recommendationRequest:

$ref: '#/components/schemas/RecommendationRequest'

requestedReservationExpiration:

$ref: '#/components/schemas/RequestedReservationExpiration'

feasibilityTimeWindow:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/TimeWindow'

processMonitor:

$ref: 'TS28623_GenericNrm.yaml#/components/schemas/ProcessMonitor'

feasibilityResult:

$ref: '#/components/schemas/FeasibilityResult'

inFeasibleReason:

$ref: '#/components/schemas/InFeasibleReason'

resourceReservationStatus:

$ref: '#/components/schemas/ResourceReservationStatus'

reservationFailureReason:

$ref: '#/components/schemas/ReservationFailureReason'

reservationExpiration:

$ref: '#/components/schemas/ReservationExpiration'

recommendedRequirements:

$ref: '#/components/schemas/RecommendedRequirements'

NetworkSliceController-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

inputServiceProfile:

$ref: '#/components/schemas/ServiceProfile'

serviceProfileId:

type: string

readOnly: true

operationalState:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/OperationalState'

administrativeState:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AdministrativeState'

availabilityStatus:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AvailabilityStatus'

processMonitor:

$ref: 'TS28623_GenericNrm.yaml#/components/schemas/ProcessMonitor'

networkSliceRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnRo'

NetworkSliceSubnetController-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

inputSliceProfile:

$ref: '#/components/schemas/SliceProfile'

sliceProfileId:

type: string

operationalState:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/OperationalState'

administrativeState:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AdministrativeState'

availabilityStatus:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AvailabilityStatus'

processMonitor:

$ref: 'TS28623_GenericNrm.yaml#/components/schemas/ProcessMonitor'

networkSliceSubnetRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnRo'

IsolationProfile-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

networkSlicingApplicability:

$ref: '#/components/schemas/NetworkSlicingApplicability'

resourceIsolationRuleList:

type: array

items:

$ref: '#/components/schemas/ResourceIsolationRule'

networkSliceRefList:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnListRo'

networkSliceSubnetRefList:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnListRo'

#-------- Definition of JSON arrays for name-contained IOCs ----------------------

NetworkSlice-Multiple:

type: array

items:

$ref: '#/components/schemas/NetworkSlice-Single'

NetworkSliceSubnet-Multiple:

type: array

items:

$ref: '#/components/schemas/NetworkSliceSubnet-Single'

EP_Transport-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_Transport-Single'

NetworkSliceSubnetProviderCapabilities-Multiple:

type: array

items:

$ref: '#/components/schemas/NetworkSliceSubnetProviderCapabilities-Single'

FeasibilityCheckAndReservationJob-Multiple:

type: array

items:

$ref: '#/components/schemas/FeasibilityCheckAndReservationJob-Single'

NetworkSliceController-Multiple:

type: array

items:

$ref: '#/components/schemas/NetworkSliceController-Single'

NetworkSliceSubnetController-Multiple:

type: array

items:

$ref: '#/components/schemas/NetworkSliceSubnetController-Single'

IsolationProfile-Multiple:

type: array

items:

$ref: '#/components/schemas/IsolationProfile-Single'

#------------ Definitions in TS 28.541 for TS 28.532 -----------------------------

resources-sliceNrm:

oneOf:

- $ref: '#/components/schemas/NetworkSlice-Single'

- $ref: '#/components/schemas/NetworkSliceSubnet-Single'

- $ref: '#/components/schemas/EP_Transport-Single'

- $ref: '#/components/schemas/NetworkSliceSubnetProviderCapabilities-Single'

- $ref: '#/components/schemas/FeasibilityCheckAndReservationJob-Single'

- $ref: '#/components/schemas/NetworkSliceController-Single'

- $ref: '#/components/schemas/NetworkSliceSubnetController-Single'

- $ref: '#/components/schemas/IsolationProfile-Single'

<CODE ENDS>

*** END OF CHANGE 1 ***

End of Changes