# 3GPP TSG-SA5 Meeting #157	S5-246440

# 14 - 18 October 2024, Hyderabad, India

# Source:	Nokia, Nokia Shanghai Bell

# Title:	Rel-19 pCR TR28.867 General CCL model and use for FM CLLs

# Document for:	Approval

# Agenda Item:	6.19.4

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1] 	3GPP TR 28.867-041 “Closed control loop management”.

# 3	Rationale

The CCL study 28.867 includes a use case on for fault management . The solution shows the required information but does not show how the CCL may be modelled to contain this information besides other information on other purposes including network problem resolution or service assurance.. This pCR adds the general model for a CCL including how it supports the difeent specific purposes.

# 4	Detailed proposal

Start of modification

## 5.5	Use case 5: CCL for fault management

### 5.5.3	Potential Solutions

#### 5.5.3.1	Required Information on Closed Control Loop for Fault Management

The solution requires the following information to be maintained as part of CCL NRM:

- An attribute for clearUserId which indicates the user is allowed to clear the alarms as per specifications in TS 28.111

- A <<dataType>> for FaultManagementTargetList which can include a list of fault management targets including alarmId, of which alarms will be handled by the FaultManagementClosedControlLoop. An alarmId identifies an AlarmRecord in the Alarm List as specified in TS 28.111, which can include information such as alarmRaisedTime, eventType, probableCause, monitoredAttributes, rootCauseIndicator and correlatedNotifications etc. This information can be leveraged by the Closed Control Loop to further enhance the correlation of alarms, for example correlation of alarms with change in PM/KPIs and/or fault supervision events, and to identify the root causes in order to find solutions to mitigate or resolve them in order to clear the alarms that otherwise have to be manually cleared by the MnS consumer according to TS 28.111.

- A <<dataType>> for FaultManagementPolicies which can include required policies and preparation actions  in order to mitigate and resolve the root cause and clear the alarms by closed control loop, for example,

  - Fault management deadline that indicates when the time the identified root causes are resolved or mitigated to clear the alarms that are associated with the fault.

  - A list of thresholds for specific KPIs that indicates faults associated with selected alarms should be managed when these thresholds are passed/exceeded

  - A list of preparation actions required to manage the faults, such as changing the operational state of managed object

- Attributes for managed object ID that faults are associated with and attribute for observationTime.

Also, a report can be provided to represent the result of fault management targets associated with specific alarms, with identified root causes. The report will include the information regarding status of the fault management with following attributes:

-	Identified root causes for each fault management target

-	The status of fault management targets, for observed and predicted, which can indicate the successful mitigation/resolving of root causes for target fault

-	Any other relevant information related to the fault management which can be vendor specific.

#### 5.5.3.1	Closed Control Loop Model supporting Fault Management

A General ClosedControlLoop IOC is introduced to represent the information for control fragments for a CCL. The ClosedControlLoop is name-contained by SubNetwork or ManagedElement and is associated with a CCLreport that contans reportedinformation about the CCL. Accordingly, the report about a CCL can existi even when the CCL is deleted.

The capabilities of the CCL are contained in one or more CCLProfiles that describe what the CCL is capable of doing or can be configured to do - including information the network resources for which the CCL can execute decisions and actions. So, the ClosedControlLoop is associated with one or more CCLProfile(s).  The network resoruce may include a NetworkSlice or NetworkSliceSubnet, a RAN or CN subnet. For fault management, the CCL is associated with a fault management CCLProfile that contains the information on clearUserId,and FaultManagementTargetList

The operational information about the CCL is constined in the CCLScope(s) and CCLGoal(s), so the ClosedControlLoop is associated with one or more CCLScope(s) and one or more CCLGoal(s). The CCLScope defines what the CCL has been configured to read, evaluate, control, etc; while the CCLGoal defines what the CCL is required to achieve for the defined scope.

Figure 5.3.3.1-1: Closed Control Loop NRM fragment

### 5.5.4	Evaluation of solutions

The solution in clause 5.5.3 provided the required information for a CCL on for fault management as well as the information objects for a general CCL that supports the specific ccapability of for fault management. No conflicting solution for fault management is provided. it is recommended to apply the solution in in clause 5.5.3 as the solution for modelling a general CCL and its specific use for fault management.

Next modifications

Next modifications

Annex A:
PlantUML Code for figures

# A.1	Relationship UML diagram for CCL Management

# A1.1	Closed Control Loop NRM fragment (Figure 5.5.3.1-1)

@startuml

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

class ManagedEntity <<ProxyClass>>

class ClosedControlLoop <<InformationObjectClass>>

class CCLProfile <<dataType>>

class CCLGoal <<dataType>>

class CCLScope <<dataType>>

class CCLReport <<InformationObjectClass>>

class CCLGoalStatus <<dataType>>

ManagedEntity "1" *-- "*" ClosedControlLoop: <<names>>

ManagedEntity "1" *-- "*" CCLReport: <<names>>

ClosedControlLoop "1" *-- "*" CCLProfile: <<names>>

ClosedControlLoop "1" *-- "*" CCLGoal: <<names>>

ClosedControlLoop "1" *-- "*" CCLScope: <<names>>

ClosedControlLoop "1" -right- "*" CCLReport

CCLReport "1" --> "*" CCLGoalStatus

note left of ManagedEntity

Represents the following IOCs:

Subnetwork or

ManagedElement

end note

note top of CCLProfile

Represents these CCL purposes:

CoomunicationServiceAssurance

NetworkProblemResolution

FaultManagement

...

end note

@enduml

End of modifications