3GPP TSG-SA5 Meeting #158	S5-246519

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

# Source:	Ericsson España S.A.

# Title:	pCR TR 28.879 Authorization UC - updates

# Document for:	Approval

Agenda Item:	6.19.21

# 1	Decision/action requested

The group is asked to discuss and approve the proposal.

# 2	References

[1] 3GPP TR 28.879: " Study on OAM for service management and exposure to external consumers".

# 3	Rationale

This pCR aims to update the authorization UC, by providing further details on:

- Use case description, clarifying that the authorization is a two-stage process that requires interaction over CAPIF-1e (Authorization request) and CAPIF-2e interface (Service API invocation)

- Solutions and evaluations, specifying the touchpoints of CAPIF authorization framework with MSAC.

# 4	Detailed proposal

It is proposed to make the following changes in the latest version of TR 28.879 [1].

Begin Change

### 5.1.4	Use case #4: Authorization of the external MnS consumer to access the management service API

#### 5.1.4.1	Description

Upon completion of discovery, the API invoker is now ready to consume service APIs. To gain access to one or more service APIs, the API invoker needs to get authorized using CAPIF built-in OAuth2.0 framework. As noted in clause 6.5.2.3 of TS 33.122 [14], the authorization use case is a two-stage process, as follows:

- Stage #1: Authorization request (over CAPIF-1e interface). In this stage, the API invoker requests the CCF for authorization to access one or more service APIs. The CCF issues this authorization to the API invoker in the form of an JWT access token. This access token contains the API invoker’s permissions for requested service APIs.

- Stage #2: Service API invocation (over CAPIF-2e interface). In this stage, the API invoker issues a Service API invocation request to the AEF, sending the URI of the service API along with the access token received from the stage #1. The request is subjected to authorization, checking the API invoker’s service API invocation against the permissions in the access token. If authorization is successful, the requested service API can be invoked and the appropriate response is returned to the API invoker.

The 3GPP management system allows configuring authorization information (permissions) on a per MnS consumer basis, for authorization, leveraging role-based access control (RBAC). TS 28.319 [29] defines a framework for such a capability set, which is referred to as management service access control (MSAC). MSAC information is used by the MnS producer to authorize incoming MnS consumer requests.

When using CAPIF as the framework to expose MnS, the external MnS consumer plays the role of the API Invoker. Putting the above into the context, the following can be noted:

- The authorization information of an external MnS consumer is defined using MSAC.

- An access token represents the external MnS consumer’s authorization information scoping one or more service APIs, i.e. the list of service APIs that the external MnS consumer requests authorization for.

- An access token is generated by the CCF (issues the access token to the external MnS consumer) and interpreted by the MnS producer (reads the access token’s scope to authorize external MnS consumer’s invocation request).

The issue here is how to ensure CCF is able to issue access tokens that can be understood/interpreted by the MnS producer. This requires that CCF have access to external MnS consumer authorization information.

#### 5.1.4.2	Potential requirements

PREQ-FS_MExpo-Auth-0x: The 3GPP management system shall provide the capability to define authorization information for an external MnS consumer using MSAC.

PREQ-FS_MExpo-Auth-0y: The 3GPP management system shall provide the capability to make external MnS consumer’s authorization information available to the CCF, so that CCF can grant authorization for an external MnS consumer.

#### 5.1.4.3	Potential solutions

#### 5.1.4.3.1	Potential solution #1: Using AccessRule to configure authorization information.

5.1.4.3.1.1		Introduction

This potential solution describes the usage of MSAC to configure external MnS consumer’s authorization information. The information model for MSAC is described in clause 7 of TS 28.319 [29]. The MSAC information model (see clause 7.3 of TS 28.319 [29]) specifies three classes: Identity, which represents an identity of a MnS consumer, and the associated roles; Role, which represents the role name and a list of permissions in a network management system; and AccessRule, which represents a permission in a network management system.

5.1.4.3.1.2	Description

AccessRule class (see clause 7.3.3 of TS 28.319 [29]) allows protecting access to one or more data nodes, specifying which actions (CRUD operations and/or notifications) and optional management data (e.g., performance measurements, KPIs) associated to these nodes are allowed/disallowed. AccessRule instances are created by the 3GPP management system at design-time.

The AccessRule class consists of the following parameters:

- ruleName: identifies the access rule.

- dataNodeSelector: specifies the MnS resources scoped by this access rule. This attribute is a JEX expression (see 3GPP TS 32.161). The resources resulting from JEX expression resolution can be one or more MOIs or MOI attributes.

- operations: used to specify the operations (as specified in clause 11.1.1 in 3GPP TS 28.532 [17]) that can be applied over the scoped resources.

actions:

- for each resource and operation combination, this attribute sets whether this operation is allowed/disallowed over that resource.

- componentCData: specifies management data (performance measurements and alarms) associated to scoped resources.

It is worth noting that 3GPP system administrator can associate one or more Role class to the same AccessRule; conversely, two or more AccessRule can be associated to one Roles.

The authorization information for a MnS consumer can be captured with the Identity class, by binding it to one or more Roles.

#### 5.1.4.3.x	Potential solution #x: Identity class made available to the CCF

5.1.4.3.x.1	Introduction

From the 3GPP management system perspective, the Identity class represents the only MSAC information that can be made available to any access control system. On the other hand, AccessRule class attributes contains 3GPP management specific information (e.g., JEX expressions, DNs of MOIs, etc) that is not understood/interpretable by the CCF.  On the other hand, the Role class includes a list of access rules based on AccessRule class, so not useful neither for CCF.

In this regard, the MSAC information that is eligible for CCF access is limited to the  Identity class.

5.1.4.3.x.2	Description

The access token represents authorization information for an external MnS consumer (i.e. the “WHO”) to access a requested list of service APIs (i.e., the “WHAT”). This solution proposes making the Identity class available to the CCF, so that the CCF can use this information to grant authorization to external MnS consumers.

In MSAC, an Identity instance allows representing the authorization information of a MnS consumer. To make this instance represents the authorization information of an external MnS consumer (API invoker), it is needed to associate this instance with the external MnS consumer’s identifier (represented with “apiInvokerId” attribute, which is created and managed by the CCF). This association is done during the API invoker onboarding procedure (see description in clause 5.1.3.1 of TS 23.222 [5]), and can be captured in the API invoker’s profile (see definition in TS 23.222 [5]).

The CCF uses the above association to issue the access token to the external MnS consumer, thus granting authorization..

To obtain authorization to access one or more service APIs, the external MnS consumer forwards a request to the CCF over the CAPIF-1e interface. In this request, the external MnS consumer uses the “Obtain_Authorization” operation in the CAPIF_Security_API (see clause 5.6.2f.3 in  TS 29.222 [13]), sending the information represented with AccessTokenReq data type (see clause 8.5.4.2.6 of TS 29.222 [13]).

Table 5.1.4.3.x.2-1 lists the AccessTokenReq data type attributes , clarifying  their usage in the present use case. See table 8.5.4.2.6-1 of TS 29.222 [13] for the data type, presence indicator, cardinality, description and applicability information for the attributes of type AccessTokenReq.

Table 5.1.4.3.x.2-1: Definition of AccessTokenReq data type.

| Attribute name | Attribute additional information | Comments |
| --- | --- | --- |
| grant_type | The data type of this attribute is defined as "string" and presence qualifier is defined as "M" (see table 8.5.4.2.6-1 of TS 29.222 [13]). | Set to “client_credentials” |
| client_id | The data type of this attribute is defined as "string" and presence qualifier is defined as "M" (see table 8.5.4.2.6-1 of TS 29.222 [13]). | Set to “apiInvokerId”, which uniquely identifies the external MnS consumer (onboarded API invoker). |
| resOwnerId | The data type of this attribute is defined as "ResOwnerId" and presence qualifier is defined as "O" (see table 8.5.4.2.6-1 of TS 29.222 [13]). |  |
| client_secret | The data type of this attribute is defined as "string" and presence qualifier is defined as "O" (see table 8.5.4.2.6-1 of TS 29.222 [13]). | If the external MnS consumer has a password, it can be included here. |
| scope | The data type of this attribute is defined as "string" and presence qualifier is defined as "O" (see table 8.5.4.2.6-1 of TS 29.222 [13]). | It represents the requested scope, i.e. the list of service APIs per AEF that the external MnS consumer requests authorization for. This attribute contains a space-delimitated string as follows: 3gpp#aefId1: apiName, apiName2, … apiNameX; aefId2: apiName1, apiName apiNameY;…aefIdN:apiName1,apiName2,…apiNameZ |
| authCode | The data type of this attribute is defined as "string" and presence qualifier is defined as "C" (see table 8.5.4.2.6-1 of TS 29.222 [13]). | RNAA |
| redirect_uri | The data type of this attribute is defined as "string" and presence qualifier is defined as "O" (see table 8.5.4.2.6-1 of TS 29.222 [13]). | RNAA |

On the receipt of the “service API authorization request”, the CCF performs the following:

- Authenticates the external MnS consumer, validating the credentials supplied in the request (see “client_id” and “client_secret” in Table 5.1.4.3.x.2-1).

- If authentication is successful, the CCF authorizes the external MnS consumer, comparing the requested scope (see “scope” parameter in Table  5.1.4.3.x.2-1) and the allowed scope (defined during onboarding procedure).  If the requested scope is absent, then it is assumed that it matches the allowed scope.

- If authorization is successful, the CCF generates an JWT access token. The scope returned in this scope represents the authorized scope.

Upon generating the JWT access token, the CCF forwards  a response to the external MnS consumer, sending information represented with AccessTokenRsp data type (see clause 8.5.4.2.6 of TS 29.222 [13]).

Table 5.1.4.3.x.2-2 lists the attributes of AccessTokenRsp data type, clarifying their usage in the present use case. See table 8.5.4.2.7-1 of TS 29.222 [13] for the data type, presence indicator, cardinality, description and applicability information for the attributes of type AccessTokenRsp.

Table 5.1.4.3.x.2-2: Definition of AccessTokenRsp data type

| Attribute name | Attribute additional information | Comments |
| --- | --- | --- |
| access_token | The data type of this attribute is defined as "string" and presence qualifier is defined as "M" (see table 8.5.4.2.7-1 of TS 29.222 [13]). | It represents the JWT access token issued by the CCF. This access token is signed with the private key of the CCF. |
| token_type | The data type of this attribute is defined as "string" and presence qualifier is defined as "M" (see table 8.5.4.2.7-1 of TS 29.222 [13]). |  |
| expires_in | The data type of this attribute is defined as "DurationSec" and presence qualifier is defined as "M" (see table 8.5.4.2.7-1 of TS 29.222 [13]). |  |
| scope | The data type of this attribute is defined as "string" and presence qualifier is defined as "O" (see table 8.5.4.2.7-1 of TS 29.222 [13]). | It represents the authorized scope. This attribute contains information that will be used by MnS producers to authorize using MSAC. |

After CCF sends the access token to the external MnS consumer, then the consumer is ready to invoke Service APIs to AEF over CAPIF-2e interface. In this invocation, the external MnS consumer provides the access token, which will be used by targeted MnS producer for authorization.

#### 5.1.4.4	Evaluation of potential solutions

#### 5.1.4.4.x	Evaluation of potential solution #1

This solution demonstrates how MSAC information model is sufficient to configure the authorization information for an external MnS consumer. This means that MSAC works for any MnS consumer, regardless it accesses through CAPIF (external MnS consumer) or not (internal MnS consumer). The solution #1 fulfils the requirement PREQ-FS_MExpo-Auth-0x of the use case.

#### 5.1.4.4.a	Evaluation of potential solution #x

The solution demonstrates that Identity classs is the only MSAC information that can be useful for CCF. However, the solution is not complete, as there is an open question that require further clarification:

- What  read/write access does the CCF have to Identity class defined in MSAC?

The solution also demonstrates how the association of an Identity instance with “apiInvokerId” allows CCF to represent authorization information of an external MnS consumer. However, the solution does not specify:

- Which is responsible for doing this association, e.g. CCF.

- The format of the authorized scope (see “scope” attribute Table 5.1.4.3.x.2-2), so it can be used by targeted MnS producer to proceed with authorization using MSAC.

Therefore, it can be concluded that the solution #x partially fulfils the requirement PREQ-FS_MExpo-Auth-0x of the use case, at least while these open questions are not clarified.

End Change