3GPP TSG-SA5 Meeting #158	S5-246621

Orlando, USA, 18 - 22 November 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.538 | CR | 0099 | rev |  | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.538 correct type and multiplicity of availableEDNList |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eECM |  |  |  |  |  | Date: |  |  | 2024-11-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - There is no corresponding dataType called “AvailableEDNList”, and this dataType should support the multiple values. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the multiplicity of avaibleEDNList from “1” into “1…*”. - Correct the typo of the type name. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The definition will be incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1461 at commit 96d36ec505671394fb51c7d8e47f40f55a434732 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

6.4.1	Attribute Properties

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| eASIdentifier | It refers to EASID that identifies a particular application (e.g. SA6Video, SA6Game, … etc.) (see clause 7.2.4 in TS 23.558 [2]). | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| eASAddress | One or more URLs and/or IP Address(es) of EAS(s) (See TS 23.558 [2]). allowedValues: N/A | type: String multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| eASREquirementsRef | This is the DN of EASRequirements. allowedValues: Not applicable | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| eESFunctionRef | This is the DN of EESFunction. allowedValues: DN of the EESFunction MOI. | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| registrationInfo | This refers to the registration information (see clause see clause 8.4.3 and 8.4.4 in TS 23.558[2]). allowedValues: N/A | type: RegistrationInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| registrationExpiry | This specifies the expiration time of the EAS and EES Registration (see clause 8.4.3 and 8.4.4 in TS 23.558 [2]). | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| registrationID | This identifies particular EAS and EES registration. (see clause 8.4.3 and 8.4.4 in TS 23.558 [2]). | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| secCredential | This specifies the security credentials of the EAS and EES Registration (see clause 8.4.3 and 8.4.4 in TS 23.558 [2]). | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| edgeDataNetworkRef | This holds a list of DN of EdgeDataNetwork. | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| requiredEASservingLocation | It defines the location where the EAS service should be available (see clause 7.3.3.6 in TS 23.558 [2]). | type: ServingLocation multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| geographicalLocation | This refers to the Geographical Service Area, (see clause 7.3.3.3 in TS 23.558 [2] that is defined as a datatype (see clause 6.3.4). allowedValues: N/A | type: GeoLoc multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| latitude | This defines the single latitude coordinate. | type: Float multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| longitude | This defines the single longitude coordinate. | type: Float multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| civicLocation | This defines the civic locations, such as: a well-known buildings, parks, arenas, civic addresses, or ZIP code etc (see clause 7.3.3.3 in TS 23.558 [2]). | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| topologicalLocation | This refers to the Topological Service Area, (see clause 7.3.3.2 in TS 23.558 [2]) that is defined as a datatype (see clause 6.3.7). allowedValues: N/A | type: TopologicalServiceArea multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| geographicalCoordinates | This refers to the Topological Service Area, (see clause 7.3.3.2 in TS 23.558 [2]) that is defined as a datatype (see clause 6.3.8). allowedValues: N/A | type: GeographicalCoordinates multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| softwareImageInfo | This refers to the software image information (e.g. software image location, minimum RAM, disk requirements) (see clause 7.1.6.5 in ETSI NFV IFA-011 [7]). It is defined as a datatype (see clause 6.3.9). allowedValues: N/A | type: SoftwareImageInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| swImageRef | It indicates the reference to the actual software image that is represented by URL (see clause 7.1.6.5 in ETSI NFV IFA-011 [7]). | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| minimumDisk | It indicates the minimum disk size requirement for the EAS software (see clause 7.1.6.5 in ETSI NFV IFA-011 [7]). The unit is Megabyte. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| minimumRAM | It indicates the minimum RAM size requirement for the EAS software (see clause 7.1.6.5 in ETSI NFV IFA-011 [7]). The unit is Megabyte. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| diskFormat | It indicates the disk format requirement for the EAS software (see clause 7.1.6.5 in ETSI NFV IFA-011 [7]). | type: String multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| operatingSystem | It indicates the operating system requirement for the EAS software (see clause 7.1.6.5 in ETSI NFV IFA-011 [7]). | type: String multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| cellIDList | It represents the list of NR cells. The cell ID, together with the gNB Identifier (using gNBId of the parent GNBCUCPFunction or GNBDUFunction or ExternalCUCPFunction), identifies a NR cell within a PLMN. This is the NR Cell Identity (NCI). See subclause 8.2 of TS 38.300 [13]. AllowedValues: Not applicable | type: Integer multiplicity: * isOrdered: False isUnique: Yes defaultValue: None isNullable: False |
| trackingAreaIdList | It represents the list of tracking areas within a PLMN. | type: TAI multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| servingPLMN | It specifies the PLMN to be served. | type: PLMNId multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| ecsAddress | One or more URLs and/or IP Address(es) of ECS(s) (See TS 23.558 [2]). allowedValues: N/A | type: String multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| providerIdentifier | The identifier of the ECSP that provides the ECS (See TS 23.558 [2]). allowedValues: N/A | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| eDNConnectionInfo | It defines the set of information needed to connect to an EDN. | type: EDNConnectionInfo multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| eDNServiceArea | This parameter defines the service location for the EDN (see clause 7.3.3.4 in TS 23.558 [2]). | type: ServingLocation multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ednIdentifier | The identifier of the edge data network (See TS 23.558 [2]). allowedValues: N/A | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| affinityAntiAffinity | This parameter defines the affinity and anti-requirements of the EAS with other EAS on the same EDN. | type: AffinityAntiAffinity multiplicity: 1 isOrdered: N/A isUnique: defaultValue: None isNullable: False |
| affinityEAS | This parameter defines the EAS identifier with which the affinity is required. | type: String multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| antiAffinityEAS | This parameter defines the EAS identifier with which the anti-affinity is required. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| serviceContinuity | This parameter defines if the service continuity is required by the EAS. If the value is TRUE, the EAS will be deployed with an EES supporting service continuity. | type: Boolean multiplicity: 1..* isOrdered: False isUnique: True defaultValue: False isNullable: False |
| virtualResource | This parameter defines the virtual resource requirements of an EAS. | type: VirtualResource multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| virtualMemory | It indicates the minimum virtual memory size requirements for EAS in megabytes. (see clause 7.1.9.3.2.2 in ETSI NFV IFA-011 [7]). | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| virtualDisk | It indicates the minimum virtual disk storage requirement for the EAS (see clause 7.1.9.4.3.2 in ETSI NFV IFA-011 [7]). | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| virtualCPU | It indicates the virtual CPU requirement for the EAS (see clause 7.1.9.2.3.2 in ETSI NFV IFA-011 [7]). | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| eESAddress | One or more URLs and/or IP Address(es) of EES(s) (See TS 23.558 [2]). allowedValues: N/A | type: String multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| eESIdentifier | It identifies the EES, see 3GPP TS 23.558. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| eASFunctionRef | This is the DN of EASFunction. allowedValues: DN of the EASFunction MOI. | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| serviceContinuitySupport | This parameter defines whether the EES supports service continuity, see 3GPP TS 23.558 | type: Boolen multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| eESservingLocation | It defines the serving location for an EES. | type: ServingLocation multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| eESFunctionRef | This is the DN of EESFunction. allowedValues: DN of the EESFunction MOI. | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| aCID | Identifies the AC(s) that can be served by the EAS (See TS 23.558 [2]). allowedValues: N/A | type: String multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| eASProvider | The identifier of the ASP that provides the EAS (See TS 23.558 [2]). allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| eASdescription | Human-readable description of the EAS (See TS 23.558 [2]). allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| eASSchedule | The availability schedule of the EAS (e.g. time windows) (See TS 23.558 [2]). allowedValues: N/A | type: Duration multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| startTime | It defines the start time of the duration for which the EAS is available. allowedValues: N/A | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| endTime | It defines the send time of the duration for which the EAS is available. allowedValues: N/A | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| eASGeographicalServiceArea | The geographical service area that the EAS serves. ACs in UEs that are located outside that area shall not be served (See TS 23.558 [2]). allowedValues: N/A | type: GeoLoc multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| eASTopologicalServiceArea | The EAS serves UEs that are connected to the Core Network from one of the cells included in this service area. ACs in UEs that are located outside this area shall not be served. (See TS 23.558 [2]). allowedValues: N/A | type: TopologicalServiceArea multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| eASServicePermissionLevel | Level of service permissions e.g. trial, gold-class supported by the EAS (See TS 23.558 [2]). Allowed Values: TRIAL, SILVER, GOLD | type: StringENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| eASFeature | Service features e.g. single vs. multi-player gaming service supported by the EAS (See TS 23.558 [2]). Allowed Value: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| eASServiceContinuitySupport | Indicates if the EAS supports service continuity or not. This IE also indicates which ACR scenarios are supported by the EAS (See TS 23.558 [2]). Default value: FALSE allowedValues: FALSE, TRUE | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| eASDNAI | DNAI(s) associated with the EAS. This IE is used as Potential Locations of Applications. It is a subset of the DNAI(s) associated with the EDN where the EAS resides. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| eASAvailabilityReportingPeriod | The availability reporting period (i.e. heartbeat period) that indicates to the EES how often it needs to check the EAS's availability after a successful registration (See TS 23.558 [2]). allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| eASStatus | The status of the EAS (e.g. enabled, disabled, etc.) (See TS 23.558 [2]). Allowed values: ENABLED, DISABLED | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| reservationLocation | This parameter defines the location where the resource needs to be reserved | type: ServingLocation multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| resourceReservationRequirement | This parameter defines the resource requirements that needs to be reserved. | type: ResourceReservationRequirement multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| computeRequirement | This parameter defines the compute requirement for reservation (see VirtualComputeDesc in clause 7.1.9.2.2 in ETSI NFV IFA-011 [7]). | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| storageRequirement | This parameter defines the storaget requirement for reservation (see VirtualStorageDesc in clause 7.1.9.2.2 in ETSI NFV IFA-011 [7]). | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| networtkingRequirement | This parameter defines the networking requirement for reservation. It is described as the connection bandwidth in Kbit/s reserved for EAS to use. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| requestedReservationExpiration | This parameter defines the MnS consumer's requirememts for the validity period of the resource reservation. allowedValues: N/A | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| resourceReservationStatus | This parameter defines the status for the reserved resources. | type: ResourceReservationStatus multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| resourceId | It identifies a reserved resource. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| reservationStatus | This parameter defines the status for a reserved resource. This attribute is configured by MnS producer and can be read by MnS consumer. Allowed Value: RESERVED: which means the specified resources is reserved and available to be used by the ASP. USED: which means the reserved resource is used by ASP. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| relocationTriggerInfo | This attributes dictates the relocation trigger for the EAS. It is a complex type which include the following attributes. allowedValues: N/A | type: RelocationTriggerInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: LOCKED isNullable: False |
| relocationType | This attribute defines if the EAS is to be relocated immediately or at a future point of time. AllowedValue: “IMMEDIATE”, “FUTURE”, “NO-RELOCATION” | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: Not isNullable: False |
| futuristicTriggerTime | This attribute defines a time stamp in future at which the EAS relocation will be initiated. allowedValues: N/A | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| relocationRejectByASP | A Boolean attribute which can be updated by the ASP to indicate its disagreement with the relocation. The value TRUE indicate that the ASP do not agree with the relocation. Allowed Values: NA | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| relocationPolicy | This attribute described the EAS relocation policies from the ASP. YES: This dictates that an EAS can be relocated as and when required NO: This dictates an EAS cannot be relocated at all YESwNOTIFY: This indicates that an EAS can be relocated with a prior notification allowedValues: "YES", "NO", “YESwNOTIFY” Editors Note: The notification mechanism in FFS. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| availableEdgeVirtualResources | This parameter defines the available edge virtual resources managed by an EDN (see NfviCapacityInfo in clause 10.5.2.3 of ETS NFV SOL-005 [17]). | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| vnfdId | It indicates the identifier of the VNFD which contains the virtual resource requirements of an EAS. (see clause 7.1 in ETSI NFV IFA-011 [7]). | type: String multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| participatingOPiD | This identifies the PO. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| leadingOPiD | This identifies the LO. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| federationID | This identifies the particular federation created. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| FederationExpiry | This defines the time post which the federation relationship shall expire. allowedValues: N/A | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| initiationTime | Date and time of the federation initiated by the Leading operator. allowedValues: N/A | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| acceptedEDNList | It provides the list of EDN that are accepted by the LO. allowedValues: N/A | type: DN multiplicity: 1…* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| resourceQuota | This defines the virtual resource quota assigned to the LO by the PO as per the federation relationship. This may be the subset of available virtual resource (indicate with attribute availableVirtualResource) in the EDN. The LO will only be authorized to reserve and use this amount of resources. allowedValues: N/A | type: VirtualResource multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| availableEASResource | This defines the available EAS in the shared EDN. This will be the DN of EASProfile. allowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| availableEDNList | This defines information related with offered EDN available with PO. allowedValues: N/A | type: AvailableEDN multiplicity: 1…* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| federationID | This defines the federation ID provided by the PO to LO at the time of federation establishment. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| reservationID | This identifies the reserved block of resources. This will be the DN of EASResourceReservationJob. allowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| federateECSIdentifier | This defines the ECS that is to be shared as part of edge federation. This will be a DN of the ECS deployed in the participating operator domain for edge services. | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| federatedPOPIdentifier | The identifier of the participating operator | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| federatedECSProfile | The information related with ECS Profile. See clause 8.2.12 of [2]. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| servedEASList | This defines the list of EAS(s) available with the partner ECS. This specifies the will a DN of EASFunction instance. | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| servedEESList | This defines the list of EES(s) available with the partner ECS. This specifies the will a DN of EESFunction instance. | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| sharedECSInfo | This defines the ECS(s) belonging to P-OP that can be used in case of roaming and federation | type: FederatedECSInfo multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| federatedECSInfo | This defines the information related with shared ECS | type: FederatedECSInfo multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| availableEDN | This defines the available EDN. | type: DN multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| eASDeploymentMonitor | Provides monitoring for the process of deployment of EAS(s). The data type of this attribute is the "ProcessMonitor" as defined in TS 28.622[4]. allowedValues: N/A | type: ProcessMonitor multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

End of Changes

Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1461 at commit 96d36ec505671394fb51c7d8e47f40f55a434732

*** START OF CHANGE 1 ***

*** OpenAPI/TS28538_EdgeNrm.yaml ***

<CODE BEGINS>

openapi: 3.0.1

info:

title: 3GPP Edge NRM

version: 18.8.0

description: >-

OAS 3.0.1 specification of the Edge NRM

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.538; Edge NRM

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.538/

paths: {}

components:

schemas:

#-------- Definition of types-----------------------------------------------------

ServingLocation:

type: object

properties:

geographicalLocation:

$ref: '#/components/schemas/GeoLoc'

topologicalLocation:

$ref: '#/components/schemas/TopologicalServiceArea'

TopologicalServiceArea:

type: object

properties:

cellIdList:

type: array

items:

type: integer

trackingAreaIdList:

$ref: 'TS28541_NrNrm.yaml#/components/schemas/TaiList'

servingPLMN:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/PlmnId'

GeoLoc:

type: object

properties:

geographicalCoordinates:

$ref: '#/components/schemas/GeographicalCoordinates'

civicLocation:

type: string

GeographicalCoordinates:

type: object

properties:

latitude:

type: integer

longitude:

type: integer

EDNConnectionInfo:

type: object

properties:

dNN:

type: string

eDNServiceArea:

$ref: '#/components/schemas/ServingLocation'

AffinityAntiAffinity:

type: object

properties:

affinityEAS:

type: array

items:

type: string

antiAffinityEAS:

type: array

items:

type: string

VirtualResource:

type: object

properties:

virtualMemory:

type: integer

virtualDisk:

type: integer

virtualCPU:

type: string

vnfdId:

type: string

SoftwareImageInfo:

type: object

properties:

minimumDisk:

type: integer

minimumRAM:

type: integer

discFormat:

type: string

operatingSystem:

type: string

swImageRef:

type: string

RegistrationInfo:

type: object

properties:

registrationExpiry:

type: string

readOnly: true

registrationID:

type: string

readOnly: true

secCredential:

type: string

Duration:

type: object

properties:

startTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

endTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

EASServicePermission:

type: string

description: any of enumerated value

enum:

- TRIAL

- SILVER

- GOLD

EASFeature:

type: string

EASStatus:

type: string

description: any of enumerated value

enum:

- ENABLED

- DISABLED

- OVERLOAD_WARNING

ResourceReservationRequirement:

type: object

properties:

computeRequirement:

type: string

storageRequirement:

type: string

networkingRequirement:

type: integer

ResourceReservationStatus:

type: object

properties:

resourceId:

type: string

reservationStatus:

type: string

description: any of enumrated value

enum:

- RESERVED

- USEd

RelocationTriggerInfo:

type: object

properties:

triggerType:

type: string

description: any of enumrated value

enum:

- IMMEDIATE

- FUTURE

futuristicTriggerTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

AvailableEDN:

type: object

properties:

availableEDN:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

resourceQuota:

$ref: '#/components/schemas/VirtualResource'

FederatedECSInfo:

type: object

properties:

federateECSIdentifier:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

federatedECSProfile:

type: integer

servedEASList:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

servedEESList:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

#-------- Definition of types for name-containments ------

SubNetwork-ncO-EdgeNrm:

type: object

properties:

ECSFunction:

$ref: '#/components/schemas/ECSFunction-Multiple'

EdgeDataNetwork:

$ref: '#/components/schemas/EdgeDataNetwork-Multiple'

#-------- Definition of concrete IOCs --------------------------------------------

EdgeDataNetwork-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

ednIdentifier:

type: string

eDNConnectionInfo:

$ref: '#/components/schemas/EDNConnectionInfo'

- type: object

properties:

EASFunction:

$ref: '#/components/schemas/EASFunction-Multiple'

EESFunction:

$ref: '#/components/schemas/EESFunction-Multiple'

availableEdgeVirtualResources:

type: string

EASFunction-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

eASIdentifier:

type: string

eESAddress:

type: array

items:

type: string

registrationInfo:

$ref: '#/components/schemas/RegistrationInfo'

eASRequirementsRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

eASAddress:

type: array

items:

type: string

relocationTriggerInfo:

$ref: '#/components/schemas/RelocationTriggerInfo'

relocationRejectByASP:

type: boolean

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-ncO'

EASProfile-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

aCID:

type: string

eASProvider:

type: string

eASdescription:

type: string

eASSchedule:

$ref: '#/components/schemas/Duration'

eASGeographicalServiceArea:

$ref: '#/components/schemas/GeoLoc'

eASTopologicalServiceArea:

$ref: '#/components/schemas/TopologicalServiceArea'

eASServicePermissionLevel:

$ref: '#/components/schemas/EASServicePermission'

eASFeature:

$ref: '#/components/schemas/EASFeature'

eASServiceContinuitySupport:

type: boolean

eASDNAI:

type: string

eASAvailabilityReportingPeriod:

type: integer

eASStatus:

$ref: '#/components/schemas/EASStatus'

EESFunction-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

eESIdentifier:

type: string

eESServingLocation:

type: array

items:

$ref: '#/components/schemas/ServingLocation'

eESAddress:

type: array

items:

type: string

softwareImageInfo:

$ref: '#/components/schemas/SoftwareImageInfo'

serviceContinuitySupport:

type: boolean

eASFunctionRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

registrationInfo:

$ref: '#/components/schemas/RegistrationInfo'

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-ncO'

ECSFunction-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

eCSAddress:

type: string

providerIdentifier:

type: string

edgeDataNetworkRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

eESFuncitonRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

softwareImageInfo:

$ref: '#/components/schemas/SoftwareImageInfo'

sharedECSInfo:

$ref: '#/components/schemas/FederatedECSInfo'

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-ncO'

EASRequirements-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

requiredEASservingLocation:

$ref: '#/components/schemas/ServingLocation'

affinityAntiAffinity:

$ref: '#/components/schemas/AffinityAntiAffinity'

serviceContinuity:

type: boolean

virtualResource:

$ref: '#/components/schemas/VirtualResource'

softwareImageInfo:

$ref: '#/components/schemas/SoftwareImageInfo'

eASSchedule:

$ref: '#/components/schemas/Duration'

eASFeature:

$ref: '#/components/schemas/EASFeature'

relocationPolicy:

type: string

description: any of enumrated value

enum:

- YES

- NO

- YESwNOTIFY

federationID:

type: string

eASDeploymentMonitor:

$ref: 'TS28623_GenericNrm.yaml#/components/schemas/ProcessMonitor'

reservationJobRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

EASResourceReservationJob-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

reservationLocation:

$ref: '#/components/schemas/ServingLocation'

resourceReservationRequirement:

type: array

items:

$ref: '#/components/schemas/ResourceReservationRequirement'

requestedReservationExpiration:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

resourceReservationStatus:

type: array

items:

$ref: '#/components/schemas/ResourceReservationStatus'

EdgeFederation-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

participatingOPiD:

type: string

leadingOPiD:

type: string

federatedECSInfo:

$ref: '#/components/schemas/FederatedECSInfo'

OperatorEdgeFederation-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

federationID:

type: string

FederationExpiry:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

leadingOPiD:

type: string

availableEDNList:

type: array

items:

$ref: '#/components/schemas/AvailableEDN'

acceptedEDN:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

OperatorEdgeDataNetwork-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

availableEASResource:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

edgeDataNetworkRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

#-------- Definition of JSON arrays for name-contained IOCs ----------------------

EASFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/EASFunction-Single'

ECSFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/ECSFunction-Single'

EESFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/EESFunction-Single'

EdgeDataNetwork-Multiple:

type: array

items:

$ref: '#/components/schemas/EdgeDataNetwork-Single'

EASProfile-Multiple:

type: array

items:

$ref: '#/components/schemas/EASProfile-Single'

EdgeFederation-Multiple:

type: array

items:

$ref: '#/components/schemas/EASProfile-Single'

OperatorEdgeFederation-Multiple:

type: array

items:

$ref: '#/components/schemas/OperatorEdgeFederation-Single'

OperatorEdgeDataNetwork-Multiple:

type: array

items:

$ref: '#/components/schemas/OperatorEdgeDataNetwork-Single'

#--------------------------------- Definition ------------------------------------

resources-edgeNrm:

oneOf:

- $ref: '#/components/schemas/EASFunction-Single'

- $ref: '#/components/schemas/ECSFunction-Single'

- $ref: '#/components/schemas/EESFunction-Single'

- $ref: '#/components/schemas/EdgeDataNetwork-Single'

- $ref: '#/components/schemas/EASRequirements-Single'

- $ref: '#/components/schemas/EASProfile-Single'

- $ref: '#/components/schemas/EdgeFederation-Single'

- $ref: '#/components/schemas/OperatorEdgeFederation-Single'

- $ref: '#/components/schemas/OperatorEdgeDataNetwork-Single'

- $ref: '#/components/schemas/EASResourceReservationJob-Single'

<CODE ENDS>

*** END OF CHANGE 1 ***