3GPP TSG-SA5 Meeting #158	S5-246732

Orlando, Florida, USA 18 - 22 November 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.104 | CR | 0155 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add MDA capability for failure recovery analysis |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | DUMMY |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TS 28.104 clause 7.2.3.2 describes a use case and requirements for failure recovery, but the solution is not described. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new MDA capability for failure recovery analysis. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Solution is missing for approved requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.3.1.3, 8.4.3.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

### 8.4.3	MDA assisted fault management

#### 8.4.3.1	MDA assisted failure prediction

##### 8.4.3.1.1	MDA type

The MDA type for failure prediction analysis is: MDAAssistedFaultManagement.FailurePrediction.

##### 8.4.3.1.2	Enabling data

The enabling data for MDAAssistedFaultManagement.FailurePrediction MDA type are provided in table 8.4.3.1.2-1.

For general information about enabling data, see clause 8.2.1.

Table 8.4.3.1.2-1: Enabling data for failure prediction analysis

| Data category | Description | References |
| --- | --- | --- |
| Performance measurements | The deteriorated performance or the abnormal performance measurements based on certain performance monitoring threshold. 3GPP management system may monitor a set of performance measurements and their thresholds, so as to support the analytics of prediction of a network service failure. | The performance measurements as defined in TS 28.552 [4] |
| Alarm notifications | Alarm information, e.g. the alarm notification of network functions. | Alarm information and notifications as per TS 28.532 [11] |
| Configuration data | MOIs of the cells and 5GC NFs. | TS 28.541 [15] |
| Network analytics data | The control plane analysis result from the NWDAF, e.g. observed service experience related network data analytics. | TS 23.288 [10] |

##### 8.4.3.1.3	Analytics output

The specific information elements of the analytics output for failure prediction analysis, in addition to the common information elements of the analytics outputs (see clause 8.3), are provided in table 8.4.3.1.3-1.

Table 8.4.3.1.3-1: Analytics output for failure prediction analysis

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| failurePredictionObject | Indication of NR cells or NFs where the failure related issues occurred or potentially occur. | M | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| potentialFailureType | Indication of type of issues that can cause the failures. NOTE 1: The values can be defined as a list of example values: "Operational Violation", "Physical Violation" and "Time Domain Violation". See alarmType described in TS 28.532 [11]. | M | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| eventTime | This field holds the time of potential failure predicted. Examples: "20:15:00", "20:15:00-08:00" (for 8 hours behind UTC). | M | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| issueID | This filed holds the ID of this failure prediction which is reported. When reports, this identifier can be used to provide the information to management system to maintain. | M | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| perceivedSeverity | This field holds the value to indicate relative level of urgency for operator attention. NOTE 2: The value can be CRITICAL, MAJOR, MINOR, WARNING, INTERMEDIATE, CLEARED, see Recommendation ITU-T X.733 [27]. | M | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| recommendedActions | This field holds the recommended actions to failure prevention. The recommended action may be (but not limited to): Update 5GC NF (e.g., AMF and SMF) profile | O | type: RecommendedAction multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |

2nd change

#### 8.4.3.x	MDA assisted failure recovery

##### 8.4.3.x.1	MDA type

The MDA type for failure recovery analysis is: MDAAssistedFaultManagement.FailureRecovery.

##### 8.4.3.x.2	Enabling data

The enabling data for MDAAssistedFaultManagement.FailureRecovery MDA type are provided in table 8.4.3.x.2-1.

For general information about enabling data, see clause 8.2.1.

Table 8.4.3.x.2-1: Enabling data for failure recovery analysis

| Data category | Description | References |
| --- | --- | --- |
| Performance measurements | The deteriorated performance or the abnormal performance measurements based on certain performance monitoring threshold. 3GPP management system may monitor a set of performance measurements and their thresholds, so as to support the analytics of prediction of a network service failure. | The performance measurements as defined in TS 28.552 [4] |
| Alarm notifications | Alarm information, e.g. the alarm notification of network functions. | Alarm information and notifications as per TS 28.532 [11] |
| Configuration data | MOIs of the cells and 5GC NFs. | TS 28.541 [15] |
| Network analytics data | The control plane analysis result from the NWDAF, e.g. observed service experience related network data analytics. | TS 23.288 [10] |

##### 8.4.3.x.3	Analytics output

The specific information elements of the analytics output for failure recovery analysis, in addition to the common information elements of the analytics outputs (see clause 8.3), are provided in table 8.4.3.x.3-1.

Table 8.4.3.x.3-1: Analytics output for failure recovery analysis

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| issueDomain | Indication the domain of the service issue. Allowed values: - RAN_ISSUE; - CN_ISSUE; - BOTH | M | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| issueID | This field holds the ID of this failure recovery which is reported. When reports, this identifier can be used to provide the information to management system to maintain. | M | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| perceivedSeverity | This field holds the value to indicate relative level of urgency for operator attention. NOTE: The value can be Critical, Major, Minor, Warning, Indeterminate, Cleared, see Recommendation ITU-T X.733 [27]. | M | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| issueRootCause | The probable root cause of the issue, such as network failure, signalling overload. | O | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| recommendedActions | This field holds the recommended actions to failure recovery. The recommended action may be (but not limited to): Update 5GC NF (e.g., AMF and SMF) profile | O | type: RecommendedAction multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |

End of changes