3GPP TSG-SA5 Meeting #158	S5-246823

Orlando, USA, 18 - 22 November 2024

# Source:	Ericsson

# Title:	Event based charging issues

# Document for:	Discussion

Agenda Item:	7.3

# 1	Decision/action requested

The group is asked to discuss and agree a way forward.

# 2	References

[1]	3GPP TS 32.290 5G system; Services, operations and procedures of charging using Service Based Interface (SBI)

[3]	3GPP TS 32.291 5G system, charging service; Stage 3

[4]	3GPP TS 32.298 Charging Data Record (CDR) parameter description3

[5] 	3GPP TS 32.299 Diameter charging applications

# 3	Rationale

## 3.1	Background

There are currently three cases of charging specified in TS 32.290 [1], that can be used for service that are of event type e.g., SMS, Exposure API invocation: Immediate Event Charging (IEC), Post Event Charging (PEC) and session based (ECUR).

For IEC the NF (CTF) may invoke converged charging service towards the CHF, before service delivery and may deduct from the user’s account balance as well as CDR generation.

For PEC the NF (CTF) interacts with the CHF after the service delivery, mainly for CDR generation.

For ECUR the NF (CTF) interact with the CHF both before service delivery where it may make a reservation on the user’s account balance, and after service delivery where it may make a deduct from the user’s account. Both before and after a CRD may be created or it may be opened before and closed after.

## 3.1	Evaluation

There are advantages and disadvantages with all the charging scenarios.

The IEC gives the CHF a possibility of checking and deducting from the account balance before the service is delivered, but it cannot be certain that the service will be delivered even if the CHF have authorized the service. In some cases, this is what is wanted, e.g. the cost may be per submission and not per successfully delivery.

The PEC give the CHF knowledge of service delivery success or failures, but it cannot to check if the account balance can be deducted or not.

The ECUR gives the CHF both the possibility of checking the account balance before the delivery as well as knowledge of service delivery success or failure before deducting from the account balance. The main disadvantage of ECUR is, if there is a long time between the request of service delivery and the actual service delivery, the possible resource allocation for the session that is being held in the CHF over a long period.

# 4	Detailed proposal

## 4.1	General

There are at least two ways of solving the issues: reuse the refund mechanism specified in TS 32.299 [5] for Diameter, allow both IEC and PEC for the same event. The main difference between these is that the refund assumes service delivery success if no IEC with “Refund Information” is received, while the IEC and PEC assumes service delivery failed if no PEC is received.

## 4.2	Solution based on refund

In TS 32.299 [5] there is an additional AVP sent from the OCS., the Refund-Information AVP and it conveys relevant information for the OCS application relative to refund mechanism. When refund mechanism is implemented in the OCS this AVP may be included in the CCA of the previous IEC. This AVP shall be used by the CTF in case of a refund scenario and thus shall be included in the CCR for refund if previously received in CCA for IEC.

This could be reused for also the IEC in converged charging, by introducing a new information element in TS 32.290 [1] clause 7 “Refund Information” and in TS 32.291 [2] clause 6.1.6.2.1.1 a new attribute name “refundInformation” of type string.

The main issue with this solution is that if an IEC with a “Refund Information” isn’t received by the CHF, then the CHF assumes that the service delivery is successful. This means that the CTF cannot lose the “Refund Information” received until it has received an acknowledgement of a successful service delivery, since otherwise it may result in charging for undelivered services.

There is also no way for the CHF to know the time period when an IEC with a “Refund Information” can be expected, i.e. how long before the CHF can consider the service delivered. This could be solved by allowing the CHF to set a validity of the “Refund Information”, but this requires the CHF to know how long a service delivery can take or setting it to the maximum of all services.

## 4.3	Solution based on IEC and PEC

In TS 32.290 [1] these is nothing that wouldn’t allow an CTF to send both an IEC and a PEC for the same event, i.e. the IEC before the service delivery and the PEC after the service delivery.

This would however require some type of correlation possibilities between the IEC and PEC, but introducing a correlation identifier will again require the CTF not to lose this as for the refund. A better alternative would be to reuse the information that is already present in the charging information in both the IEC and PEC for the event, but this would require the CHF to know which are common and can be used. So having the CTF determining which information elements that could be used for correlating would be preferred. This could be done as an addition to the common data structure of the Charging Data Request in TS 32.290 [1] table 7.1.

Table 4.3.1: Addition to common data structure of Charging Data Request

| Information Element | Converged Charging Category | Description |
| --- | --- | --- |
| Correlation information | OC | This field contains the information on how two requests can be correlated, between events, i.e. between IEC and PEC. |
| Information Element List | OC | This field holds a list of information element included in the request that can be used in correlation. |

The “Information Element List” would contain references to the Resource Attribute in the same Charging Data Request that can be used for correlation of the IEC and PEC it could for instance for an SMS charging contain: “/sMSChargingInformation/originatorInfo/originatorSUPI”, “/sMSChargingInformation/recipientInfo/recipientSUPI”, and “/sMSChargingInformation/submissionTime”. This would allow the CHF to later find a match between the IEC and PEC

It will still require the CHF to hold the information for an undetermined time which could be solved if the CTF could state the time frame in which the PEC can be expected. This could be done as a further addition to the common data structure of the Charging Data Request in TS 32.290 [1] table 7.1.

Table 4.3.2: Addition to common data structure of Charging Data Request

| Information Element | Converged Charging Category | Description |
| --- | --- | --- |
| Correlation information | OC | This field contains the information on how two requests can be correlated, between events, i.e. between IEC and PEC. |
| Information Element List | OC | This field holds a list of information element included in the request that can be used in correlation. |
| Time until next request | OC | This field holds the maximum amount of time between the requests that can be correlated. |

This “Time until next request” will tell the CHF that if it hasn’t received a PEC within this time, it shall consider the IEC as failed. If the PEC is received this will include the information of service delivery.