3GPP TSG-SA5 Meeting #158	S5-246885

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

# Source:	Ericsson España S.A.

# Title:	pCR TR 28.879 Conclusions and recommendations

# Document for:	Approval

Agenda Item:	6.19.21

# 1	Decision/action requested

The group is asked to discuss and approve the proposal.

# 2	References

[1] 3GPP TR 28.879, " Study on OAM for service management and exposure to external consumers".

# 3	Rationale

The pCR proposes to add conclusions and recommendations for the exposure of management services.

# 4	Detailed proposal

It is proposed that the following changes be made to clause 6 of TR 28.879 [1].

Begin Change

# 6	Conclusions and recommendations

## 6.1 	Conclusions

### 6.1.0	General

The study has focused on the exposure of management services. This exposure means making management services available for consumption to external MnS consumers. An external MnS consumer represents a MnS consumer that discovers and consumes management services using the CAPIF framework.

When management services are exposed using the CAPIF, the 3GPP management system represents an API provider domain, whereas an external MnS consumer represents an API invoker.

The study has identified the need for 3GPP management system to define a functional entity that provides the API provider domain functions: AEF, AMF and APF. In this document, this functional entity is referred as to management services exposure function (MSEF). The MSEF specification has been left outside of this study, and has been postponed for normative phase.

For the exposure of management services, the study has reported on five key use cases.

### 6.1.1	Use case #1: MSEF registration into CAPIF

For 3GPP management system to become a recognized API provider domain, there is a need to register the MSEF into CAPIF. The registration is initiated by an existing AMF (API Management Function) instance, which sends MSEF registration information to the CCF.

The MSEF registration information needs to be represented with appropriate CAPIF data type (i.e., APIProviderEnrolmentDeatils), so it can be sent over CAPIF-5 interface.

The solution is described in clause 5.1.1.3.1. The solution is feasible and no gaps have been identified.

### 6.1.2	Use case #2: Publishing of management services to the CCF

For a management service to be discoverable and consumable through CAPIF, there is a need to publish this management service as one (or more) service APIs onto the CCF. The registration is initiated by an APF (API Publishing Function) instance, which sends management service information to the CCF.

- The management service information need to be represented with appropriate CAPIF data type (i.e., ServiceAPIDescription), so it can be sent over CAPIF-4 interface. The solution is described in clause 5.1.2.3.1. The solution is feasible and no gaps have been identified.

- The URL components of a management service (what is offered to MnS consumer) need to match with the URL components of a service API (what is offered to external MnS consumer). The solution is described in clause 5.1.2.3.2. The solution is feasible and no gaps have been identified.

- The APF instance is provided by the registered MSEF. The solution is described in clause 5.1.2.3.x. The solution is feasible and no gaps have been identified.

### 6.1.3	Use case #3: Configuring discovery policy information for an external MnS consumer

An operator may want to limit the visibility that certain API invokers have over published API information, according to the business agreements settled with the stakeholder owning the API invoker. To that end, the CCF is configured with discovery policy information on a per API invoker basis.

The discovery policy information, configured on CCF, is not subject to standardization. After analysis, it is concluded that this use case has no impact on 3GPP management system, so no requirements and solutions are proposed.

### 6.1.4	Use case #4: Authorization of the external MnS consumer to access the management service API

To get access to one or more service APIs, the external MnS consumer needs to get authorized. The consumer requests the CCF (over CAPIF-1e interface) an access token, that it can later use to invoke one or more service APIs (over CAPIF-2e interface). This invocation arrives to targeted MnS producer(s), which use the received token to authorize the API invocation using the MSAC framework. For it to happen:

- The authorization information of the external MnS consumer needs to be defined using MSAC information, which includes AccessRules, Roles and Identity. The solution is described in clause 5.1.4.3.1. The solution is feasible and no gaps have been identified

- The authorization information of the external MnS consumer needs to be made available to the CCF, so that CCF can grant authorization issuing the access token. The solution is described in clause 5.1.4.3.x. For the solution, some gaps have been identified, which need further clarification and specification during normative phase.

### 6.1.5	Use case #5: Logging the management service API

AEF (API Exposing Function) sends invocation logs to the CCF over CAPIF-3 interface, so they can be used for auditing and charging purposes. It is needed to represent service API invocation logs with appropriate CAPIF data type (e.g., InvocationLog data type). The solution is described in clause 5.1.5.3.1. The solution is feasible and no gaps have been identified.

## 6.2 	Recommendations

It is recommended to start normative work covering the following:

- Adding the capability for the 3GPP management system to implement the following solutions:

  - solution described in clause 5.1.1.3.1, to support the following use case: “ MSEF registration into CAPIF use case”.

  - solutions described in clauses 5.1.2.3.1, 5.1.2.3.2 and 5.1.2.3.3, to support the following use case: “Publication of management services to the CCF”

  - solution described in clauses 5.1.4.3.1, to support the following use case: “Authorization of the external MnS consumer to access the management service API”

  - solution described in clauses 5.1.5.3.1, to support the following use case: “Logging the management service API”

- Specification of MSEF. The MSEF represents a new entity within 3GPP management system implementing API provider domain functions, to communicate with CCF (over CAPIF-3/4/5 interfaces) and external MnS consumers (over CAPIF-2 interface). The final name of MSEF (currently non-binding) and supported capabilities need to be specified.

- Specification of a solution that allows making external MnS consumer’s authorization information available to the CCF, so that CCF can grant authorization issuing the access token. This to-be-developed solution will use the principles and guidelines captured in the solution described in clause 5.1.4.3.x.

End Change