3GPP TSG-SA5 Meeting #158	S5-246889

Orlando, USA, 18 - 22 November 2024

# Source:	Nokia

# Title:	Rel-19 pCR TS 28.abc Add stage 3 for controlling signalling monitoring

# Document for:	Approval

Agenda Item:	6.19.22 MonStra

# 1	Decision/action requested

The group is requested to discuss and approve the pCR below.

# 2	References

[1]	3GPP TS 28.abc V0.1.0 Signalling traffic monitoring management

# 3	Rationale

The 3GPP management system controls the network by a service-based and model-driven architecture with an object-oriented model.

In order to integrate the control of sending copied signalling messages into the 3GPP management system, the network resource model needs to be enhaced by corresponding informatin object classes (IOC).

The present contribution translates the implementation-independent network resource model into an implementation-specific yaml definition.

.

# 4	Detailed proposal

The following changes are proposed to TS 28.abc [1].

Begin of modifications

# 8	Network resource model stage 3

openapi: 3.0.1

info:

title: 3GPP NRM

version: 19.0.0

description: >-

OAS 3.0.1 specification of the signalling monitoring NRM

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.XXX; signalling monitoring NRM

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.xxx/

paths: {}

components:

schemas:

#-------- Definition of types-----------------------------------------------------

#-------- Definition of concrete IOCs --------------------------------------------

SignallingMonitoringControl-Single:

allOf:

- $ref: '#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

administrativeState:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AdministrativeState'

operationalState:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/OperationalState'

interfaceRefList:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnListRo'

signallingMonitoringCollectionUri:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/UriRo'

#-------- Definition of JSON arrays for name-contained IOCs ----------------------

SignallingMonitoringControl-Multiple:

type: array

items:

$ref: '#/components/schemas/SignallingMonitoringControl-Single'

#--------------------------------- Definition ------------------------------------

resources-SignallingMonitoringCtrl:

oneOf:

- $ref: '#/components/schemas/SignallingMonitoringControl-Single'

## Todo change in other stage 3

## > adding resources-SignallingMonitoringCtrl to 532 ProMnS

## > adding the SignallingMonitoringControl-Multiple to 541 5GC NF IOCs

## > adding the pointer (DN) from 5GC NF MOI to SignallingMonitoringControl MOI

End of modifications