3GPP TSG-SA5 Meeting #158 	S5-247079

Orlando, Florida, USA, 18 - 22 November 2024                               revision of S5-246745

# Source:	Nokia, Nokia Shanghai Bell

# Title:	New use case on recommendation for management data collection for predicted failure

# Document for:	Approval

Agenda Item:	6.19.2

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

None

# 3	Rationale

This pCR provides use case description, potential solution and conclusion for the new use case on recommendation for management data collection for predicted failure.

# 4	Detailed proposal

First Change

### 5.7.x	Use case X: Management data collection recommendation for predicted failures

#### 5.7.x.1	Description

In the current use case of MDA-assisted failure prediction, the analytics output includes the following information: failure type, estimated time of the failure event, perceived severity, and recommended actions. However, if the producer of MDA MnS is unable to recommend an action for failure prevention it may be useful to recommend to collect additional management data.

The process of collecting data from deployed field entities to address issues is time-consuming and resource-intensive. Therefore, it is advisable to collect relevant management data pre-emptively, enabling effective debugging and root cause analysis to prevent future occurrences of failure. To support root cause analysis, it is essential to collect required management data that facilitates the identification and resolution of predicted failures.

#### 5.7.x.2	Potential Requirements

REQ-MGMT_DATA_COLLECTION_PRED_FAILURE_MDA-01: MDA capability for failure prediction should have a capability to indicate the management data to be collected for the predicted failure, when abnormal status is predicted.

#### 5.7.x.3	Potential solution

The enabling data specified as part of the failure prediction use case in clause 8.4.3.1.2 in [2] can be reused for this use case.

This potential solution proposes to enhance the output of the use case “MDA assisted failure prediction” with the following attributes. The output of the prediction of each failure may an alternative recommendation which contains the following information:

- an optional attribute would indicate the management data need to be collected. The management data can be of any of the following or combined.

- Performance Measurements /KPI

- Trace/MDT

- QoE

For each of the management data type, the following information may be provided.

- Management data itself (PMs or Trace/MDT or QoE)

- NFs in which the collection needs to be performed

- Time duration for which the collection needs to be done.

#### 5.7.x.4	Evaluation of solutions

The solution proposed in clause 5.7.x.3 satisfies the requirements, and this solution is feasible for normative work.

Next Change

## 6.6	Fault management related analytics and alarm prediction

The use case, requirements and solution for Use case: Providing threshold statistics information for fault management is described in clause 5.6.1. It is recommended to add new attribute for threshold information in the MDARequest in TS 28.104 [2] to support providing statistics information in the analytics output. The detailed solution is described in clause 5.6.1.3.

The use case, requirements and solution for the capability of enhancing failure prediction is described in clause 5.6.2. It is recommended to extend the output of failure prediction in TS 28.104 [2] as described in clause 5.6.2.3.

The use case, requirements and solution for the capability of service failure recovery is described in clause 5.6.3. It is recommended to add new capability in TS 28.104 [2] to support service failure recovery. The detailed solution is described in clause 5.6.3.3.

The use case, requirements and solution for Use case: NF failure evaluation is described in clause 5.6.4. It is recommended to add new attribute in the MDA analytics output for failure prediction analysis in TS 28.104 [2] to support providing predicted network impact information in the analytics output. The detailed solution is described in clause 5.6.4.3.

The use case, requirements and solution for the use case: Management data collection recommendation for predicted failures is described in clause 5.7.x. It is recommended to extend the output of failure prediction as described in clause 5.7.x.3 in TS 28.104 [2].

End of changes