3GPP TSG-SA5 Meeting #158	S5-247120

Orlando, USA, 18 - 22 November 2024			 			(revision of S5-246665, and merge of S5-246424)

# Source:	Nokia, NTT DOCOMO, Huawei

# Title:	Rel-19 pCR TR 28.880 Potential solution for use case Handling of power shortages

# Document for:	Approval

Agenda Item:	6.19.20

# 1	Decision/action requested

The group is requested to discuss and approve the pCR below.

# 2	References

[1]          3GPP TR 28.880: "Study on energy efficiency and energy saving aspects of 5G networks and services".

# 3	Rationale

This pCR proposes a potential solution for Use case #11: Handling of power shortages, of TR 28.880 [1].

# 4	Detailed proposal

The following changes are proposed for TR 28.880[1].

1st Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[3]	ETSI GR NFV-IFA 029 (V3.3.1): "Network Functions Virtualisation (NFV) Release 3; Architecture; Report on the Enhancements of the NFV architecture towards "Cloud-native" and "PaaS"".

[4]	ETSI GS NFV-IFA 040 (V4.3.1): "Network Functions Virtualisation (NFV) Release 4; Management and Orchestration; Requirements for service interfaces and object model for OS container management and orchestration specification".

[5]	ETSI GS NFV-IFA 027 (V4.3.1): "Network Functions Virtualisation (NFV) Release 4; Management and Orchestration; Performance Measurements Specification".

[6]	ETSI GS NFV-IFA 027 (V5.2.1): "Network Functions Virtualisation (NFV) Release 5; Management and Orchestration; Performance Measurements Specification".

[7]	3GPP TS 22.261: "Service requirements for the 5G system".

[8]	ETSI GS OEU 020: "Operational energy Efficiency for Users (OEU); Carbon equivalent Intensity measurement; Operational infrastructures; Global KPIs; Global KPIs for ICT Sites".

[9]	ETSI EN 303 472: "Environmental Engineering (EE); Energy Efficiency measurement methodology and metrics for RAN equipment".

[10]	ISO/IEC 30134-3:2016: "Information technology -- Data centres -- Key performance indicators -- Part 3: Renewable energy factor (REF)".

[11]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[12]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[13]	3GPP TS 28.313: "Management and orchestration; Self-Organizing Networks (SON) for 5G networks".

[14]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[15]	Energy Benchmark 2023: Telco Insights and Industry Health Check.

[16]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[17]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[18]	3GPP TS 22.104: "Service requirements for cyber-physical control applications in vertical domains".

[19]	ETSI EN 303 471 V1.1.1 (2019-01): "Environmental Engineering (EE); Energy Efficiency measurement methodology and metrics for Network Function Virtualisation (NFV)".

[20]	ITU-T Recommendation L.1333: "Carbon data intensity for network energy performance monitoring".

[21]	ETSI ES 202 336-1 V1.2.1: "Environmental Engineering (EE); Monitoring and Control Interface for Infrastructure Equipment (Power, Cooling and Building Environment Systems used in Telecommunication Networks) Part 1: Generic Interface"

[22]	ETSI ES 202 336-2 V1.1.1 "Environmental Engineering (EE); Monitoring and Control Interface for Infrastructure Equipment (Power, Cooling and Building Environment Systems used in Telecommunication Networks) Part 2: DC power system control and monitoring information model"

[23]	ETSI ES 202 336-3 V1.1.1 "Environmental Engineering (EE); Monitoring and Control Interface for Infrastructure Equipment (Power, Cooling and Building Environment Systems used in Telecommunication Networks) Part 3: AC UPS power system control and monitoring information model"

[24]	ETSI ES 202 336-4 V1.1.1 "Environmental Engineering (EE); Monitoring and Control Interface for Infrastructure Equipment (Power, Cooling and Building Environment Systems used in Telecommunication Networks) Part 4: AC distribution power system control and monitoring information model"

[25]	ETSI ES 202 336-5 V1.1.1 "Environmental Engineering (EE); Monitoring and Control Interface for Infrastructure Equipment (Power, Cooling and Building Environment Systems used in Telecommunication Networks) Part 5: AC diesel back-up generator system control and monitoring information model"

[26]	ETSI ES 202 336-6 V1.1.1 "Environmental Engineering (EE); Monitoring and Control Interface for Infrastructure Equipment (Power, Cooling and Building Environment Systems used in Telecommunication Networks) Part 6: Air Conditioning System control and monitoring information model"

[27]	ETSI ES 202 336-7 V1.1.1 "Environmental Engineering (EE); Monitoring and Control Interface for Infrastructure Equipment (Power, Cooling and Building Environment Systems used in Telecommunication Networks) Part 7: Other utilities system control and monitoring information model"

[28]	ETSI ES 202 336-8 V1.1.1 "Environmental Engineering (EE); Monitoring and Control Interface for Infrastructure Equipment (Power, Cooling and Building Environment Systems used in Telecommunication Networks) Part 8: Remote Power Feeding System control and monitoring information model"

[29]	ETSI ES 202 336-9 V1.1.1 "Environmental Engineering (EE); Monitoring and Control Interface for Infrastructure Equipment (Power, Cooling and Building Environment Systems used in Telecommunication Networks) Part 9: Alternative Power Systems"

[30]	ETSI ES 202 336-10 V1.1.1 "Environmental Engineering (EE); Monitoring and Control Interface for Infrastructure Equipment (Power, Cooling and Building Environment Systems used in Telecommunication Networks) Part 10: AC inverter power system control and monitoring information model"

[31]	ETSI ES 202 336-11 V1.1.1 "Environmental Engineering (EE); Monitoring and Control Interface for Infrastructure Equipment (Power, Cooling and Building Environment Systems used in Telecommunication Networks) Part 11: Battery system with integrated control and monitoring information model"

[32]	ETSI ES 202 336-12 V1.2.1 "Environmental Engineering (EE); Monitoring and Control Interface for Infrastructure Equipment (Power, Cooling and Building Environment Systems used in Telecommunication Networks) Part 12: ICT equipment power, energy and environmental parameters monitoring information model"

[Y] 	3GPP TS 28.104: "Management and orchestration; Management Data Analytics (MDA)"

[Z]	3GPP TS 28.531: " Management and orchestration; Provisioning"

Next Change

## 5.11	Use case #11: Handling of power shortages

### 5.11.1	Description

In this use case, the 3GPP network management system becomes aware of current or upcoming power shortages. The network operator in turn takes actions (e.g. at service, network and/or resource level) to ensure the delivery of communication services such as emergency calls, first responder communication, voice calls etc. while reducing energy use.

### 5.11.2	Potential requirements

REQ-Energy_Shortage_Handling-01: The 3GPP management system, in case it is made aware of current energy shortages or upcoming energy shortages, should be able to take appropriate measures to conserve energy use in the network while ensuring delivery of important communication services.

NOTE:	Examples for communication services that need to be preserved include emergency calls, first responder communication, voice calls etc.

### 5.11.3	Potential solutions

#### 5.11.3.y	Potential solution #Y: Service adjustment for energy saving, based on available energy information

##### 5.11.3.y.1	Introduction

In this potential solution, it is proposed to enhance the MDA assisted Energy Saving use case (see clause 7.2.4 of TS 28.104 [Y]) to consider the power shortage information, available energy from the backup energy, and re-use the existing inputs such as traffic trends and provides service adjustment recommendations for different periods of time, and thus conserving available energy. This solution is applicable for the network connected to power source(s) (grid electricity, Backup energy and locally generated energy (see clause 6 of ITU-T Recommendation L.1333 [20])) and for the situations when the power outages and energy shortage information from the power source is available in advance.

NOTE: This solution does not consider LTE network elements involved in providing the communication services, i.e., it is applicable for 5G SA scenarios.

##### 5.11.3.y.2	Description

MDA assisted Energy Saving use case is defined in clause 7.2.4 of TS 28.104 [Y], and the corresponding data definitions are defined in clause 8.4.4 of TS 28.104 [Y]. This potential solution proposes enhancement to this MDA use case.

It is proposed to enhance the enabling data for this MDA use case with information on power shortage and outage information and enhance the analytics output from this MDA use case with service adjustment recommendations for different periods of time thus conserving available energy to extend network service lifetime.

Power shortage, outage information and available energy information (in case of backup battery-based power supply) of the power supply associated with the managed functions is needed as input from the operator. The operator should be able to associate this information for the managed functions. The available energy information for the power supply can be associated using the information that is available outside of 3GPP (e.g., Estimated remaining battery autonomy (time) during discharge as per Annex B of ETSI ES 202 336-11 [31] and alarms State of Charge (SoC) low, State of Health (SoH) abnormal, Battery discharge as per Annex A of ETSI ES 202 336-11 [31]).

NOTE 1:	This potential solution assumes that the power supply information is associated with the network elements and network functions (see clause 5.3.3.3).

NOTE 2:	This potential solution assumes that energy information for the power supply, such as e.g. the information defined in ETSI ES 202 336-11 [31] can be made available to the 3GPP management system. Which interface(s) may be used is out of scope of the present document.

NetworkSlice (see clause 6.3.1 of TS 28.541 [16]) represents the 5G services deployed as network slice, while ServiceProfile (see clause 6.3.3 of TS 28.541 [16]) data type includes attributes that can represent service SLA requirements. Hence, the service adjustment recommendations can be represented using the ServiceProfile.

The enhancements to the enabling data and analytics output for the MDA assisted Energy Saving use case (see clause 8.4.4 of TS 28.104 [Y]) are described below.

Extensions to the enabling data for this MDA use case are described in table 5.11.3.y.2-1.

Table 5.11.3.y.2-1: MDA enabling data extensions

| Data category | Description | References |
| --- | --- | --- |
| Configuration data | Power shortage, outage information of the power supply associated with the managed functions, i.e., associated with MOIs of the network functions (gNBs, cells, UPFs). |  |
|  | Available energy information (in case of back-up battery-based power supply) of the power supply associated with the managed functions | Estimated remaining battery autonomy (time) during discharge as per Annex B of ETSI ES 202 336-11 [31] |
| Performance measurements | Energy consumption KPIs as listed below: - NF Energy Consumption (EC) - Network Slice Energy Consumption (EC) - NG-RAN Energy Consumption (EC) | TS 28.554 [2] |
| Alarm notifications | alarms State of Charge (SoC) low, State of Health (SoH) abnormal, Battery discharge of the backup battery. | alarms State of Charge (SoC) low, State of Health (SoH) abnormal, Battery discharge as per Annex A of ETSI ES 202 336-11 [31] |

Extensions to the analytics output from this MDA use case are described in table 5.11.3.y.2-2.

Table 5.11.3.y.2-2: MDA analytics output extensions

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| networkServiceOperabilityRecommendations | This field holds the recommended actions of network service levels based on energy availability to conserving available energy and to extend network service lifetime. The recommended action may be (but not limited to): - Re-configuration of SLAs for the services and network slices, i.e., NetworkSlice IOC (see TS 28.541 [16]). The updated SLA can be expressed as ServiceProfile (see TS 28.541 [16]). The recommended values in ServiceProfile could indicate different levels of SLAs (i.e., the recommended QoS) for different time periods (expressed using attribute sliceAvailability) and applicable for entire network or parts of the network (expressed using coverageArea) - Modification of network slices, network slice subnets and/or individual 3GPP NFs, such as those deriving into scaling down VNF instance as virtualized parts of a 3GPP managed function (see TS 28.531 [Z]). | M | type: Recommended3GPPAction multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |

### 5.11.4	Evaluation of potential solutions

The potential solution #Y proposes to extend the MDA assisted Energy Saving use case to utilize power shortage and outage information, and the available energy from the backup energy that is associated with the network functions, along with network analytics and to provide recommendations to ensure the delivery of communication services for different time periods thus conserving available energy to extend network service lifetime.

This solution is feasible by identifying that power shortage information needs to be associated with the managed functions in the 3GPP management system and by using MDA for providing the recommendations.

NOTE 1:	The definition of the information elements (e.g., an IOC or attribute(s) representing the information) related to power shortage and outage information and how this is associated with the network functions (represented by ManagedFunction) will be part of the normative phase.

NOTE 2:	This potential solution assumes that energy information for the power supply can be acquired using interfaces defined outside of 3GPP, i.e., ETSI ES 202 336-11 [31]. If external interfaces can be consumed by the 3GPP management system, or if this information needs to be collected by the MNO and configured into the 3GPP management system will be part of normative phase.

NOTE 3:	This potential solution applicable for the scenario where network elements have backup battery system. The general solution for this use case will be for future study.

Next Change

## 6.11	Handling of power shortages

The use case, requirement and solution for handling of power shortages. According the corresponding requirement defined in clause 5.11, the solution proposes an approach to extend the MDA assisted Energy Saving for network elements with backup battery.

It is recommended to extend MDA assisted Energy Saving use case in TS 28.104 [Y] for normative work. It is also recommended to update NRM with information related to power shortage, outage information and available energy information, and how this is associated with the network functions (represented by ManagedFunction) will be part of the normative phase.

End of change