3GPP TSG-SA5 Meeting #158	S5-247327

Orlando, USA, 18 - 22 November 2024 									  revision of S5-246667

# Source:		Nokia

# Title:	Rel-19 pCR TR 28.880 Add use case on cell proximity-based energy saving

# Document for:	Approval

Agenda Item:	6.19.20

# 1	Decision/action requested

The group is requested to discuss and approve the pCR below.

# 2	References

[1]          3GPP TR 28.880: " Study on energy efficiency and energy saving aspects of 5G networks and services"

# 3	Rationale

This contribution proposes to add a new use case on cell proximity based energy saving.

The inference function has been introduced in TS28.105 and it is agreed that Energy Saving is one use case for which for AI/ML inference functions may be instantiated. This pCR adds the configurable features and requirements for AI/ML-based Energy Saving Functions.

This is key issue related to the following objective that is described in FS_Energy_OAM_Ph3.

- WT-1.3 Study new use cases, requirements and solutions for energy efficiency and energy saving, applying to NG-RAN and/or 5GC and/or network slicing, (including but not limited to intent based, analytics based and AI/ML assisted energy saving)

# 4	Detailed proposal

The following changes are proposed for TR 28.880[1].

1st Change

## 5.A	Use case #<A>: Cell proximity-based energy saving

Cell-proximity-based Energy saving compensation

### 5.A.1	Description

#### 5.A.1.1	Overview

TS 28.310 defines a capacity booster cell which can be deactivated when the load is low. For a capacity booster cell to be deactivated, a  candidate cell should have adequate overlap with the capacity booster cell to make sure that deactivation of the booster cell does not lead to a coverage hole. A candidate cell may have several capacity booster cells that overlap with its coverage and can be deactivated. This applies when several capacity booster cells are added to improve the capacity of an area covered by a larger coverage cell.

Note: "Overlap" here is the generalization of "overlaid" as used in TS28.310, overlaid means that the area covered by the small cell is also fully covered by the larger cell while overlap means that the area covered by the small cell can be partly or fully covered by the larger cell.

Example: A coverage cell has 2 or more capacity booster cells that are associated with it, say on the different sides of the coverage cell's border.  For cell deactivation in this scenario, there is need to decide which of those 2 or more capacity booster cells should be deactivated and in which order.

An energy saving function, e.g., the "domain-centralized ES function", needs to select the best among the capacity booster cells to be deactivated at a given time. The energy saving function should be enabled to support the selection for and sequencing of deactivation of the capacity booster cells.

The proposed cell-level activation and deactivation is for centralized energy saving.

#### 5.A.1.2	Degree of neighborliness

To activate or deactivate cells, an energy saving function (be it the Domain-centralized ES function, see TS 28.310 [12]) computes the degree to which the coverage of cells overlap among each other to ensure that the coverage of a deactivated cell (the capacity booster cell) is compensated by a candidate cell. The degree of neighborliness measures and characterizes the degree to which a capacity booster cell overlaps with the candidate cell. The energy saving function determines the degree of neighborliness among the cells. Alternatively, the degree of neighborliness can be configured onto the energy saving function, e.g., by the operator or another function. As input, the entity computing the degree of neighborliness takes 1) the cell configuration data including transmit powers, applied antennas characteristics and tilts; 2) the propagation characteristics like building information; and 3) performance information like cell handover statistics.

Note : the degree of neighborliness is information contained by the energy saving function, it does not need to be defined in the RAN.

#### 5.A.1.3	Cell grouping and categorization

To optimize the selection of cells to be deactivated or reactivated, the energy saving function needs to group the cells that need to be treated togther for energy saving into a group, say called the Energy Saving Cell Group. Within the Energy Saving Cell Group, the cells are categorized into two sets - the primary cells which are not deactivated and provide primary coverage and capacity booster cells which only enhance capacity and can thus be deactivated when the load is low. The entity computing the cell grouping can inform  the operator or an energy saving function about the group of cells that need to be treated together, i.e., the Energy Saving Cell Group and the categorization of cells either as a coverage or a capacity booster cell., Where the energy saving function does not have the capability to compute the Energy Saving Cell Group, it should be possible  to configure the Energy Saving Cell Group and cell categorisations to the energy saving function.

Note : the Energy Saving Cell Group is quasi-static information on the energy saving function, when configured it remains unchanged until reconfigured.

#### 5.A.1.4	Cell deactivation sequence

When selecting cells to deactivate, the energy saving function considers the load in a set of cells to then decides the most optimal cell(s) to be deactivated and the order in which they can be deactivated. Based on the degree of neighborliness among cells as well as the cell grouping and categorization as coverage or capacity booster cells, the the energy saving function can compute the order in which the cells can be deactivated. The energy saving function can inform the e.g., the operator of the cell deactivation order, e.g., by indicating each capacity booster cell’s rank in the deactivation process. The an Operator can be enabled to request for such a deactivation order. Where the energy saving function does not have the capability to determine the cell deactivation order, another function(e.g. an AIML inference function) can be enabled to configure the cell deactivation order onto the energy saving function.

Given the cell deactivation order, the energy saving function routinely compares the load in the set of cells (the Cell Group load) to the low and high load thresholds to decide if deactivation can be executed. The Cell Group load needs to be computed for the set of active cells. Moreover, an automaton function can be enabled to configure the activation or deactivation load thresholds.

Note 1: the term deactivation is used to represent a total switching off of the cell or any disabling of a part of the cell, e.g., a disabling of the cell's power amplifier.

Note 2: the activation sequence is expected to be the opposite order to the deactivation sequence, but whether extra details are required for the activation sequence will be addressed in the normatve work.

Note 3: For all the input information, the energy saving function may be configured by or based on the output of other automation functions including MDA functions and AIML inferenc efunctions. For example, MDA may derive the cell grouping which is then configured onto the Energy saving function

### 5.A.2	Potential requirements

REQ-NES-01: The 3GPP management system should have a capability allowing an authorized MnS consumer to configure an energy saving function with degree of neighborliness, describing the degree to which the coverage areas of any candidate neighbour cells overlap with each other.

REQ-NES-02:	The 3GPP management system should have a capability allowing an authorized MnS consumer to configure an energy saving function with information on the group of cells that need to be treated together (called the Energy Saving Cell Group) when selecting cell to be activated or deactivated.

NOTE REQ-1: Configuration is needed only if the energy saving function cannot compute the Energy Saving Cell Groups.

REQ-NES-03:	The 3GPP management system should have a capability allowing an authorized MnS consumer to configure an energy saving function with information on the cell categories indicating coverage cells and capacity booster cells.

NOTE REQ-2: Coverage cells are those that provide primary coverage and are not deactivated while capacity booster cells are those which only enhance capacity and can thus be deactivated when the load is low.

REQ-NES-04:	The 3GPP management system should have a capability allowing an authorized MnS consumer to configure an energy saving function with information on the cell deactivation order which is the capacity booster cell’s rank in the deactivation and reactivation process indicating the order in which the cells in a given area should be deactivated.

NOTE REQ-3: Configuration is needed only if the energy saving function does not determine the cell deactivation order.

REQ-NES-05:	The 3GPP management system should have a capability allowing an authorized MnS consumer to configure an energy saving function with the deactivation and reactivation load thresholds.

### 5.A.3	Potential solutions

#### 5.A.3.1	Potential solution #A: AI/ML-assisted Network Energy Saving

##### 5.A.3.1.1	Introduction

The centralized energy saving function can compute the sequence of deactivation of cells, e.g. with support from AIML inference functions or MDA Mns services. Based on the sequence, the ES function determines the cells to be deactivated at a given time. The deactivation of cells can be executed by the centralized energy saving function.

Note: This solution needs to be improved in the normative work with details about the usage of the information elements

### 5.A.4	Evaluation of potential solutions

Only one solution in clause 5.A.3.1 is provided for this use case. The solution needs further elaboration in the normative work.

Note: the use case, requirements and solution are a baseline which can be enhanced or extended in the normative work, e.g. to address support for an activation sequence if different from the deactivation sequence.

Next change

## 6.A	Cell proximity-based energy saving

The use case, requirement and solution for Cell proximity-based energy saving is described in clause 5.A. In this use case, four input information for energ saving is described including how the energ saving may be configured with that information. it is also described how an energy saving function may rely on an AIML inference function to obtain the four input information

It is recommended to add this use case and in TS 28.310 [12] and take the solution as baseline for normative work..

End of change