3GPP TSG-SA5 Meeting #158 	S5-246341

Orlando, USA, 18 – 22 November 2024

# Source:	Ericsson

# Title:	Corrections of Subcounter and Filter Format

# Document for:	Discussion and Endorsement

Agenda Item:	6.4.9

# 1	Decision/action requested

The group is asked to discuss and endorse to this proposal.

# 2	References

[1]	3GPP TS 32.404: "Performance Management (PM); Performance measurements - Definitions and template".

[2]	3GPP TS 28.552: "5G performance measurements"

[3]	3GPP TS 28.558: "UE level measurements for 5G system"

[4]	3GPP TS 28.622:	 "Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)"

[5]	3GPP TS 28.623: "Generic Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions"

# 3	Rationale

## 3.1	Problem statement

In both TS 28.552[2] and TS 28.558[3], the subcounter format definition is unclear.

- No Subcounter format definition. The only reference is TS32.404[1] which the subcounter is specified as numeric string.

  - In TS 32.404[1], only the form of ‘Cause group’.Cause, ‘Cause group’.sum, are specified

  - In TS 32.404[1], Cause is specified as numeric values. “For the standardised causes with numeric values explicitly specified in the reference specification, the subcounter name will be the number assigned to this cause in this reference specification”

  - In TS28.552[2], Subcounter is extended with more value type, e.g. bin, modulation schema, message type, etc. without a clear format definition.

- Mixed Subcounter and Filter definition. Examples are:

  - DRB.PdcpSduDelayDl_Filter, 
Where filter is a combination of PLMN ID and QoS level and S-NSSAI. 
Where PLMN ID represents the PLMN ID, QoS represents the mapped 5QI or QCI level, and SNSSAI represents S-NSSAI.

  - DRB.PacketLossRateUu.Filter, 
Where Filter is a combination of PLMN ID and QoS level and S-NSSAI. 
The QoS level represents the mapped 5QI or QCI.

  - DRB.RlcSduLatencyDl, The measurement is optionally split into subcounters per QoS level and subcounters per S-NSSAI
optionally DRB.RlcSduLatencyDl.QOS where QOS identifies the target quality of service class, and
optionally DRB.RlcSduLatencyDl.SNSSAI, where SNSSAI identifies the S-NSSAI.

  - conflicted performance measurement definitions which has same measurement name but different order of the subcounters or filters

    - TS28.552[2], Subclause 5.4.7 UPF: GTP.DelayDlPsaUpfNgranDist.5QI.Bin

    - TS28.552[2], Subclause 5.1.1.1.8 gNB: GTP.DelayDlPsaUpfNgranDist_5QI_Bin

- When more than one Subcounters are allowed in the same Performance measurement definition, the order is not unclear. This may become an unnecessary complexity implementation issue. Examples are:

  - L1M.SS-RSRP.Bin or L1M.SS-RSRPNrNbr.SSBIndex.Bin

- The " .sum" suffix is useless. A Performance measurement without subcounter is providing the possible sum value. Example is: RLM.FailRLSetupIub.1.sum, 1.sum identifies the aggregate of all the causes within the cause group "Radio Network Layer Cause". However, the RLM.FailRLSetupIub without any cause value is providing the number failed RL setup with any possible cause value.

- The " .ALL" suffix is added for 5GC UE level measurement in TS28.622[4] only. It is not specified in TS32.404[1] or TS28.552[2] or TS28.558[3]. This is a misalignment between SA5 specifications.

## 3.2	Potential subcounter format

A performance measurement may be split into more than one subcounter and/or filter based on multiple criteria as defined at the performance measurement definition.

The separator '.' or vendor-specific separator shall be used to append the Subcounter to a given performance measurement name. The resulting performance measurement name with single Subcounter shall be in the following form:

<familyName>.<measurementName>.<Subcounter>

If more than one Subcounters are specified for a given performance measurement, it shall be specified in a reasonable order. The separator '.' or vendor-specific separator shall be used to append an additional Subcounter to a given Subcounter. The resulting performance measurement name with multiple Subcounters shall be in the following form:

<familyName>.<measurementName>.<Subcounter1>.<…>.<SubcounterN>

The Subcounter shall be an alphanumeric string with at least one alphabet character as specified by following regular expression:

"^(?=.*[A-Za-z])[A-Za-z0-9]+$"

When a performance measurement is identified with Subcounter, one measurement result, which fulfilled the given Subcounter criteria, shall be provided by the producer. It is possible to specify a Subcounter with multiple applicable values, e.g. Cause. In this case, a list of measurement results, which fulfilled each applicable Subcounter values, shall be provided by the producer.

The following are examples of possible resulting performance measurement names with:

- no Subcounter: DRB.PacketLossRateUl

- one Subcounter with multiple applicable values: RRC.ConnEstabAtt.Cause;

- one Subcounter without multiple values: TMM.HoPrepInterReqMLB.ResOpt;

- two Subcounters with and without multiple applicable values: MM.HoExeInterFailMLB.UeCtxtRelCmd.Cause;

## 3.3	Potential performance measurement updates

The following are the potential performance measurement updates in TS28.552 and TS28.558

- There are many Subcounters which may be realized by using Filter, i.e., PLMN ID and QoS level (5QI, or QCI) and S-NSSAI. However, the format needs to be clarified.

- Clarifications on multiple Subcounters order and format.

- Clarifications on the Subcounters Cause format.

- Updates any conflicted performance measurement definitions.

- The suffix shall be reconsidered by use cases. So far there is no use case to support it.

## 3.4	Potential performanceMetrics updates

In TS28.622[4]/TS28.623[5], performanceMetrics are specified in multiple places. The Measurement metric definition is missing in TS28.552[2]. Then TS28.558[3] and TS28.622[4]/TS28.623[5], shall refer to this specification for Measurement metric definition.

In TS28.622[4]/TS28.623[5], the measurement metric is specified as followings, which filter based use cases are missing:

-	"family.measurementName.subcounter" for measurement types with subcounters

-	"family.measurementName" for measurement types without subcounters

-	"family" for measurement families

The definition of measurement metric needs to be added with Subcounter and Filter definition as well. The followings are the potential updates:

- "<familyName>.<measurementName>.<Subcounter1>.<…>.<SubcounterM>_<Filter1>_…_<FilterN>" for measurement result(s) of a combination of Subcounters and Filters

- "<familyName>.<measurementName>.<Subcounter1>.<…>.<SubcounterM>" for a measurement result of Subcounters only

- "<familyName>.<measurementName>_<Filter1>_…_<FilterN>" for measurement result(s) of Filters only

- "<familyName>.<measurementName>" for a measurement result without any Subcounter or Filter

- "<familyName>" for a list of measurement results of all applicable measurementNames without any Subcounter or Filter

# 4	Detailed proposal

We propose that:

- Update TS28.552[2] with the subcounter format clarification as proposed in clause 3.2.

- Update each performance measurement defined in TS28.552[2] and TS 28.558[3] as proposed in clause 3.3.

- Update measurement metric definition as proposed in clause 3.4 with following steps:

  - add measurement metric definition in TS28.552[2];

  - add reference in TS 28.558[3] to the measurement metric definition defined in TS28.552[2]

  - remove duplicated measurement metric defined for performanceMetrics, mgtDataName, and ueCoreMeasurements in TS28.622[4] and TS28.623[5].